3GPP TSG-SA WG6 Meeting #41-e	S6-210220

e-meeting, 18th – 26th January 2021	(revision of S6-210125)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0039 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Group management support for 5G-VN groups |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2021-01-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Enhancement of SEAL Group Managament is required to support management of 5G VN groups specified in TS 23.501 and TS 23.502. The proposed enhancements are described in TR 23.745 clause 7.9. In summary: - SEAL group management server shall enable a VAL server to create, update, delete, subscribe to changes of a 5G VN group. - SEAL group management client and the VAL server shall be able to identify that a group managed by the SEAL server is used for 5G LAN-Type service. For group management procedures pertaining to a 5G VN group, SEAL Group Management server shall use dynamic 5G VN group management procedures exposed by NEF via the N33 reference point, as specified in TS 23.501 clause 5.29.2 and in TS 23.502 clause 4.15.6. 5G VN group data is specified in TS 23.502 clauses 4.15.6.3b and 4.15.6.3c. SEAL Group Management server shall use the VAL server and VAL service identities to derive the 5G VN group data such as DNN, S-NSSAI, Application descriptors, etc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhancement of SEAL Group Managament to support management of 5G VN groups |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SEAL does not provide capabilities for 5G VN group management. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 10.2.2, (new) 10.2.5.x, 10.3.2.26, 10.3.2.28, 10.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GS	5G System

5GVN	5G Virtual Network

CAPIF	Common API Framework for northbound APIs

CRUDN	Create, Retrieve, Update, Delete and Notify

EPC	Evolved Packet Core

NEF	Network Exposure Function

NR	New Radio

PCC	Policy and Charging Control

SCEF	Service Capability Exposure Function

SEAL	Service Enabler Architecture Layer for Verticals

VAL	Vertical Application Layer

* * * Next Change * * * *

### 10.2.2	On-network functional model description

Figure 10.2.2-1 illustrates the generic on-network functional model for group management.

Figure 10.2.2-1: On-network functional model for group management

The group management client communicates with the group management server over the GM-UU reference point. The group management client provides the support for group management functions to the VAL client(s) over GM-C reference point. The VAL server(s) communicate with the group management server over the GM-S reference point.

The group management server interacts with the NEF of the underlying 3GPP network system via N33 reference point to perform group management procedures for 5G Virtual Network (5GVN) groups.

* * * Next Change * * * *

#### 10.2.5.X	N33

For group management procedures pertaining to a 5GVN group the group management server interacts with the NEF of the underlying 3GPP network using the dynamic 5G Virtual Network group management procedures exposed by the NEF via the N33 reference point, as specified in TS 23.501 [10] and in TS 23.502 [11].

* * * Next Change * * * *

#### 10.3.2.26	Configure VAL group request

Table 10.3.2.26-1 describes the information flow for configure VAL group request from a VAL server to a group management server.

Table 10.3.2.26-1: Configure VAL group request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL group ID | M | The group ID used for the VAL group. |
| VAL group description (see NOTE x) | M | Information related to the VAL group e.g. group definition including communication type, policy, group size, group leader. |
| VAL service ID list (see NOTE 1) | O | List of VAL services whose service communications are to be enabled on the group. |
| Geo ID list (see NOTE 1) | O | List of geographical areas to be addressed by the group. |
| Identity list (see NOTE 1) | O | List of VAL UE IDs who are invited to be member of the group. |
| Identity list subscription | O | Indicates interest to receive notifications of newly registered or de-registered VAL UE IDs. |
| NOTE 1: At least one of these IEs shall be present. NOTE x: Group communication type may indicate 5G LAN-Type communication using either Ethernet or IP (IPv4 and/or IPv6) transport. |  |  |

* * * Next Change * * * *

#### 10.3.2.28	Group announcement

Table 10.3.2.28-1 describes the information flow for a group management server to announce a VAL group to the group management clients.

Table 10.3.2.28-1: Group announcement

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | The group ID used for the VAL group. |
| VAL group description (see NOTE x) | M | Information related to the VAL group e.g. group definition including communication type and connection parameters, policy, group size, group leader. |
| VAL service ID list (see NOTE 1) | O | List of VAL services whose service communications are to be enabled on the group. |
| Geo ID list (see NOTE 1) | O | List of geographical areas to be addressed by the group. |
| Identity list (see NOTE 1, NOTE 2) | O | List of VAL UE IDs who are invited to be member of the group. |
| NOTE 1: At least one of these IEs shall be present. NOTE 2: This element is not present if it results in privacy concerns. NOTE x: Group communication type may indicate 5G LAN-Type communication using either Ethernet or IP (IPv4 and/or IPv6) transport. Group connection parameters may include DNN and S-NSSAI. |  |  |

* * * Next Change * * * *

### 10.3.8	Group announcement and join

#### 10.3.8.1	General

This subclause describes the procedures for establishing group communication from the group management server to the group management clients.

#### 10.3.8.2	Procedure

Pre-conditions:

1.	The group management client, group management server, VAL server and the VAL clients belong to the same VAL system.

2.	The VAL server is aware of the users' identities and is authorized to form a VAL group.

Figure 10.3.8.2-1: Procedure for establishing VAL group communication between the group management server and group management client1.

1.	The VAL server determines group information and the identity list to which the group announcement shall be sent. The decision can be based on the list of authorized UEs and other criteria (e.g. user consent, service, or vehicle driving profile).

2.	The VAL server configures VAL group for Uu communication defined by VAL Group ID for one or more VAL services with list of VAL Service ID with the group management server.

3.	The group management server creates an empty group based on the information provided in the Configure VAL group request. The group management server stores the mapping between the VAL group ID and the external Group Id in the VAL group document, along with a list of GPSIs corresponding to the identity list provided by the VAL server. The group management server also determines whether the group is for 5G LAN-Type communication and whether Ethernet or IP (IPv4 and/or IPv6) transport shall be used for the 5G LAN-Type communication.

4.	If 5G LAN-Type communication is to be used, the group management server creates a 5GVN group in the 5GS via N33 using the create group procedure specified in 3GPP TS 23.501 [10] clause 5.29.2 and 3GPP TS 23.502 [11] clause 4.15.6. The group management server creates the 5GVN group data and the 5GVN group membership data defined in 3GPP TS 23.502 [11] clause 4.15.6.3b to be configured in the 5GS. To create the 5GVN group data the group management server uses the 5G LAN-Type communication type information provided by the VAL server to set the PDU session type (Ethernet or IP) and maps the VAL server requester identity to DNN/S-NSSAI combination and maps the VAL service IDs to Application descriptors. To create the 5GVN group membership data the group management server maps the VAL group ID to the External Group ID and makes a list of GPSIs corresponding to the identity list provided by the VAL server.

NOTE 1:	This step is skipped for the case that a 5G LAN-Type communication is not being used.

NOTE 2:	The PDU session type, DNN, S-NSSAI provided within 5GVN group data cannot be modified in the 5GS after the create procedure.

NOTE 3:	The 5GS supports only a 1:1 mapping between DNN/S-NSSAI combination and 5GVN group.

5.	The group management server announces the VAL group to the group management clients. For a 5GVN group the announcement includes the communication type (IP or Ethernet), DNN, and S-NSSAI corresponding to the 5GVN group.

6		The group management client registers to VAL group communication using the VAL Group ID.

7.	The group management server records the users who have registered to be the members of the group.

8.	The group management server sends a VAL group registration response to the group management client.

9.	The group management server sends a configure VAL group response to the VAL server.

NOTE 4:	Step 9 may occur anytime after step 5.

10.	The group management server sends identity list notification about the newly registered users to the other members of the group and VAL server, whose subscription to receive notifications of newly registered VAL UE IDs is successful in step 8 and step 9 respectively.

11.	The group management client may inform VAL client about the updated identity list.