3GPP TSG-SA WG6 Meeting #41-e	S6-21xxxx

e-meeting, 18th – 26th January 2021	(revision of S6-210089)

Source:	Ericsson, Convida Wireless LLC

Title:	EEC registration clarification

Spec:	3GPP TS 23.558 v1.2.0

Agenda item:	7.6

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution introduces some clarifications for the EEC registration.

2. Reason for Change

The access of EDGE-1 functionalities (e.g. EAS discovery) should not depend on the registration but the generic authentication/authorization mechanism as described in cl.8.10:

After the successful EEC authentication/authorization, the EEC acquires a valid security credential for EEC related procedures including provisioning procedure, registration procedure, EAS discovery procedure.

Therefore, to perform an EAS discovery, EEC supplies discovery filters in the EAS discovery request, the discovery process doesn’t need the EEC context created in EES. In addition, there is no other EDGE-1 service depending on EEC registration.

3. Proposal

It is proposed to agree the following changes in TS 23.558.

* * * First Change * * * *

#### 8.4.2.1	General

An Edge Enabler Client performs registration with an Edge Enabler Server in order to enable initialization, update or removal of the Edge Enabler Client context information at the Edge Enabler Server. The context may be further used for edge-specific operations or processing, e.g. analytics.

NOTE:	The ECSP policy can require EEC registration to use Edge Computing services.

Editor's Note:	It is FFS whether and which information is provided by the Edge Enabler Client to enable the Edge Enabler Server to select the way of interaction with 3GPP network (e.g. selecting NEF, SCEF, or NEF+SCEF) for a UE.

* * * End of Changes * * * *