3GPP TSG-SA WG6 Meeting #42-bis-e	S6-210803

e-meeting, 12th – 20th April 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0287 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configuration of MC service UE label |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT |  |  |  |  |  | Date: |  |  | 2021-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | An authorized user, e.g. a dispatcher, needs to be able to configure the MC service UE label of an MC service UE to allow ad-hoc changes based on operational requirements. The solution offers changing of a MC service UE label without having to modify the MC service UEs initial MC service UE label. The same applies in a migration scenario, where a MC service UE from a partner MC system has migrated into the primary MC system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A temporary MC service UE label can be configured in the Location Management Server to replace the initial MC service UE label. The temporary MC service UE label is transmitted instead of the initial MC service UE label with location information reports. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MC service UE label can not be configured by authorized users based on operational requirements and identification of individual MC service UEs sending information location reports under the changed operational requirements will not be possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.2.17 (new), 10.9.2.18 (new), 10.9.2.19 (new), 10.9.2.20 (new), 10.9.3.10 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 10.9.2.17	MC service UE label configuration

Table 10.9.2.17-1 describes the information flow from the location management client to the location management server for configuration of the MC service UE label.

Table 10.9.2.17-1: MC service UE label configuration

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service user who initiates the configuration of MC service UE label |
| Set of MC service IDs | O (see NOTE) | Set of identities of the reporting MC service user on the MC service UE (i.e. MCPTT ID, MCVideo ID, MCData ID) for whom the temporary MC service label should be configured |
| MC service UE label | O (see NOTE) | The MC service UE label, as initially configured in the MC service UE |
| MC service UE label | O (see NOTE) | The temporary MC service UE label to be used in location information reports, instead of the initial MC service UE label |
| NOTE: When a temporary MC service UE label is provided with an initial MC service UE label or/and an optional set of MC service IDs, the LMS replaces the initial MC service UE label in the location information report with the temporary MC service UE label. If a set of MC service IDs or/and the initial MC service UE label is provided and no temporary MC service UE label is provided, the LMS uses the initial MC service label again. |  |  |

* * * Next Change * * *

#### 10.9.2.18	MC service UE label configuration response

Table 10.9.2.18-1 describes the information flow from the location management server to the location management client for MC service UE label configuration response.

Table 10.9.2.18-1: MC service UE label configuration response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the requesting MC service user |
| Configuration status | M | Indicates the configuration result |

* * * Next Change * * *

#### 10.9.2.19	MC service UE label information request

Table 10.9.2.19-1 describes the information flow from the location management client to the location management server to retrieve the MC service UE label configuration information.

Table 10.9.2.19-1: MC service UE label information request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service user who requests the MC service UE label information |
| Set of MC service ID(s) | O (see NOTE) | Set of identities of the reporting MC service user on the MC service UE (i.e. MCPTT ID, MCVideo ID, MCData ID) for whom existing MC service UE label information is requested |
| Set of MC service UE label(s) | O (see NOTE) | Set of MC service UE label(s) for whom existing MC service UE label information is requested. The information element can either be the initial or temporary MC Service UE label |
| NOTE: Either one needs to be present. |  |  |

* * * Next Change * * *

#### 10.9.2.20	MC service UE label information response

Table 10.9.2.20-1 describes the information flow from the location management server to the location management client to provide a combined set of MC service IDs and MC service UE label(s).

Table 10.9.2.20-1: MC service UE label information response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the requesting MC service user |
| Combined set of MC service ID(s) and MC service UE label(s) | M (see NOTE) | Set of MC service ID(s) and their corresponding initial/temporary MC service UE label(s) configured on the location management server |
| NOTE: In case an empty set is returned, no MC service UE label has been configured. If the initial and temporary MC service UE label are returned, no MC service ID(s) have been configured. |  |  |

* * * Next Change * * *

#### 10.9.3.10	Usage of MC service UE label configuration

##### 10.9.3.10.1	General

An authorized user, e.g. dispatcher, may need to temporarily change an MC service UE label according to operational requirements. E.g. the updated MC service UE label may consist of a new mission identifier, the unchanged device description and the organisational name.

For this purpose, the authorized user configures a temporary MC service UE label in the location management server. The LMS handles the initial MC service UE label and the corresponding temporary MC service UE label together.

If an initial MC service UE label is present in the location information report and the corresponding temporary MC service UE label is configured, the LMS shall replace the initial MC service UE label with the temporary MC service UE label.

##### 10.9.3.10.2	MC service UE label configuration procedure

Figure 10.9.3.10.2-1 illustrates the procedure of the MC service UE label configuration.

Figure 10.9.3.10.2-1: MC service UE label configuration procedure

1.	The location management client sends MC service UE label configuration to the location management server.

2.	The location management server checks if the requesting MC service user is authorized to configure the temporary MC service UE label.

3.	The location management server handles the configuration based on the initial MC service UE label and the corresponding temporary MC service UE label.

4.	The location management server replies with a MC service UE label configuration response indicating the configuration status.

##### 10.9.3.10.3	MC service UE label information request procedure

Figure 10.9.3.10.3-1 illustrates the procedure of the MC service UE label information request.

Figure 10.9.3.10.3-1: MC service UE label information request procedure

1.	The location management client sends the MC service UE label information request to the location management server.

2.	The location management server checks if the requesting MC service user is authorized to request the MC service UE label configuration information.

3.	The location management server sends the MC service UE label information response to the location management client.

* * * End Change * * * *