3GPP TSG-SA WG6 Meeting #42-e	S6-210507

e-meeting, 1st – 9th March 2021	(revision of S6-21xxxx)

Source:	Samsung

Title:	EN on EAS selection

Spec:	3GPP TS 23.558 v1.3.0

Agenda item:	7.6

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

This contribution closes an EN on EAS selection during target EAS discovery.

2. Reason for Change

Following EN:

Editor's Note:	How the target EAS is selected from the list of EASs received in step 4 is for further study.

The contribution proposes that to downselect the EAS information, the T-EES uses the expected and minimum service KPIs if received from the S-EES or apply ECSP policy if such KPIs are not received.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v1.3.0.

* * * First Change * * * *

#### 8.8.3.2	Discover target EAS

Figure 8.8.3.2-1 illustrates the procedure for fetching target EAS information. This procedure may be utilized by a source EAS, which undertakes the transfer of application context information to a target EAS directly, or can be invoked by the source EES itself on deciding to execute application context relocation.

Pre-conditions:

1.	Information related to the EES is available with the source EAS.

Figure 8.8.3.2-1: Discover target EAS

1.	The source EAS sends the EAS discovery request to the source EES or the source EES decides to execute the application context relocation. The EAS discovery request from the source EAS includes the requestor identifier [EAS ID] along with the security credentials and includes EAS discovery filter matching its EAS profile. If target DNAI is available at the source EAS via UP path management event notification, the source EAS provides the source EES with the target DNAI.

NOTE:	The trigger condition to invoke the Discover target EAS API is up to application service logic, which is out of scope of this specification.

2.	If the request is received from the source EAS, the source EES checks whether the requesting EAS is authorized to perform the discovery operation. If the source EES decided to execute the application context relocation or when the requesting EAS is authorized, the source EES checks if there exists a target EAS information (registered or cached) that can satisfy the requesting EAS information, additional query filters and the Expected service KPIs and the Minimum required service KPIs if received from the EEC during the EAS discovery or from the EAS in step 1. If the source Edge Enable Server finds the target EAS(s) in the cached or registered information, the flow continues with step 5, else the source EES retrieves the target EES address from the ECS as specified in clause 8.8.3.3 and continues with step 3.

3.	The source EES invokes the EAS discovery request on the target EES. The source EES discovery request includes the requestor identifier [EES ID] along with the security credentials and includes EAS discovery filter. In the EAS discovery filter, the source EES may include the Expected service KPIs and the Minimum required service KPIs if received from the EEC during the EAS discovery or from the EAS in step 1.

Upon receiving the request, the target EES may trigger the EAS management system to instantiate the target EAS that matches with EAS discovery filter IEs (e.g. AC ID) as in clause 8.11.

4.	The target EES discovers the target EAS(s) and responds with the discovered target EAS information to the source EES. To discover the EAS information, the T-EES utilizes the Expected service KPIs and the Minimum required service KPIs, if received from the S-EES. If neither, the Expected service KPIs nor the Minimum required service KPIs are available, the T-EES discovers the EAS information based on the ESCP policy. The source EES may cache the target EAS information.

5.	If the request was received from the source EAS, the source EES responds to the EAS with the target EAS Information.