3GPP TSG-SA WG6 Meeting #42-e	S6-210xxx

e-meeting, 1st – 9th March 2021	(revision of S6-210445)

Source:	Huawei, Hisilicon

Title:	5GMARCH Definitions

Spec:	3GPP TS 23.554

Agenda item:	7.9

Document for:	Approval

Contact:	Hongxia Hao, haohongxia@huawei.com

1. Introduction

This pCR proposes the definition of TS 23.554.

2. Reason for Change

To add some necessary definitions for keeping us in the same page on understanding these items.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554 v0.1.0.

* * * First Change * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

MSGin5G Server: the server providing MSGin5G Service.

Legacy 3GPP Message Gateway: the entity in MSGin5G Service to support interworking with legacy 3GPP message service (e.g. SMS, NIDD).

Non-3GPP Message Gateway: the entity in MSGin5G Service to support interworking with non-3GPP message service.

Message Gateway: general terminology for Legacy 3GPP Message Gateway, Non-3GPP Message Gateway, or both.

MSGin5G Message: the message supported by MSGin5G service, the message may be included in the message request between MSGin5G Server and MSGin5G Client or between MSGin5G server and Message Gateway. The message is not included in the message request between Legacy 3GPP UE and Message Gateway, and is not included in the request between the Application Server and the MSGin5G Server.

MSGin5G Client: the client that enables MSGin5G Message sending and receiving.

MSGin5G UE: the UE that has MSGin5G client.

Legacy 3GPP UE: the UE that supports legacy 3GPP message sending and receiving (e.g. SMS, NIDD, etc) in MSGin5G Service.

Non-3GPP UE: the UE that supports non-3GPP message sending and receiving (e.g. RCS message as specified in GSMA PRD RCC.07 [x], OMA LWM2M message as specified in OMA OMA-ERELD-LightweightM2M [y]).

MSGin5G Group: the group of UEs which members may be MSGin5G UE, Legacy 3GPP UE and Non-3GPP UE.

Standalone Communication: the messages exchanged between each party are sent separately, without establishing a session between two parties for sending messages.

Editor's note: Definitions alignment to Architecture is FFS.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	GSMA PRD RCC.07: "RCC.07 Rich Communication Suite 9.0 Advanced Communications Services and Client Specification".

[y] 	OMA OMA-ERELD-LightweightM2M-V1_1-20180612-C: "Enabler Release Definition for LightweightM2M".

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>