3GPP TSG-SA WG6 Meeting #42-e	S6-210614-rev1

e-meeting, 1st – 9th March 2021	(revision of S6-210613)

Source:	one2many, Samsung

Title:	Pseudo-CR on Message Aggregation

Spec:	3GPP TS 23.554 v0.1.0

Agenda item:	7.9

Document for:	Approval

Contact:	Peter Sanders (peter.sanders@one2many.eu)

1. Introduction

This pCR includes Solution #17 from TR 23.700-24 into the TS as agreed in the Conclusion section of the TR.

2. Reason for Change

New text.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554 v0.1.0 as clause 7.1.4 in the current version of the TS.

* * * First and only change * * *

All below text is new

### 7.1.4	Message Aggregation

#### 7.1.4.1	General

Based on maximum segment size allowed to transmit over available transport, the MSGin5G service can optimize communications by aggregating one or more messages towards the target.

NOTE: It is possible to carry multiple messages inside the MSGin5G message payload that is implementation specific and out of the scope of the current specification.

The following pre-conditions apply for message aggregation:

- The recipient UE(s) support a MSGin5G Client or the (Legacy-3GPP and non-3GPP) Message Gateway support the MSGin5G Client capability.

- The MSGin5G Client 1 and MSGin5G Client 2 are registered with the MSGin5G Server.

#### 7.1.4.2	Message Aggregation at MSGin5G Client

Figure 7.1.4.2-1 shows the procedure for a MSGin5G Client aggregating point-to-point messages and group messages each carrying small amounts of data.

NOTE 1:	Aggregation of multiple messages can also be done with the Application Client; in this case it is implementation specific and out of the scope of the current specification.”

Figure 7.1.4.2-1: MSGin5G Client aggregates messages towards target end point

1)	Application Client(s) on UE 1 initiates a request to the MSGin5G Client 1 to send a message to another target or to send a group message.

2)	The MSGin5G Client 1 checks if aggregation is allowed for this message, checks the message data size, and the priority level to determine if the received message can be aggregated. For example, MSGin5G Client 1 finds that the messages have small payload size when compared to the maximum segment size that can be transmitted over available transport and are not high priority messages, which could be sent as per scheduling policy towards a selected target.

NOTE 2:	MSGin5G Client 1 decides to continue aggregating messages until optimal use of segment size before sending message towards MSGin5G Server.

NOTE 3:	How the MSGin5G Client 1 uses individual message priority, maximum time to wait, etc for aggregating and sending is out of scope of the present document.

3)	The MSGin5G Client 1 aggregates multiple MSGin5G message requests intended for a selected target and sends the aggregated message request as defined in Table 7.1.4.2-1 and Table 7.1.4.2-2 according to scheduling policy towards MSGin5G Server.

Table 7.1.4.2-1: Aggregated message request (MSGin5G Client to MSGin5G Server)

| Information element | Status | Description |
| --- | --- | --- |
| Originator MSGin5G Service ID | M | The service identity of the sending MSGin5G Client. |
| Recipient MSGSin5G Service ID | M | The service identity of the receiving MSGin5G Client. |
| Message ID | M | Unique identifier of the aggregated message |
| Number of individual messages | M | Indicates total number of messages which are aggregated into single message |
| List of individual messages | M | Each element in this list contains information as specified in Table 7.1.4.2-2. |

Table 7.1.4.2-2: Individual message data

| Information element | Status | Description |
| --- | --- | --- |
| Individual Message ID | M | Unique identifier of this individual message |
| Application ID | O | Identifies the application for which the payload is intended |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e. delivery report) |
| Payload | M | Payload of the message |
| Priority Type | O | Application priority level requested for this message. |

NOTE 4:	Total size of Aggregated message request is less than or equal to maximum segment size allowed to be transmitted over available transport.

4)	MSGin5G server checks whether the MSGin5G Client 1 is authenticated and authorized to send Aggregated message request. If authenticated and authorized, the MSGin5G server skips step 5.

5)	If MSGin5G Client 1 is not authorized to send Aggregated message request or the Aggregated message request is not valid, the MSGin5G server sends Aggregated message reject to the MSGin5G Client 1. The information elements defined in Table 7.1.4.2-3 are included in the request.

Table 7.1.4.2-3: Aggregated message reject (MSGin5G Server to MSGin5G Client)

| Information element | Status | Description |
| --- | --- | --- |
| Destination MSGin5G Client ID | M | The identity of the MSGin5G Client sending the original message. |
| Message ID | M | Unique identifier of this message |
| Reject Reason | O | This IE contains the failure reason, e.g. the originator is not authorized to send a message request or one of the multiple messages aggregated has an issue, may be included in this IE. |

Editor's Note: If the Appliction ID should be in table 7.1.4.2-3 instead of table 7.1.4.2-2 is FFS.

6)	If MSGin5G Client 1 is authorized to send Aggregated message request, the MSGin5G server sends Aggregated message request towards the selected target MSGin5G Client(s) 2.

7)	The MSGin5G Client 2 splits the received Aggregated message request into multiple individual MSGin5G message requests per application and sends them towards Application Client(s).

8)	The Application Client(s) may initiate sending MSGin5G message delivery report, if requested in the original message that is received as in Step 7. MSGin5G Client 2 forwards the MSGin5G message delivery report towards the Application Client(s) on UE 1 via MSGin5G Server and MSGin5G Client 1.

NOTE 5:	The delivery reports can also be aggregated into a single message.

* * * Next change * * * *

#### 7.1.4.3	Message Aggregation at MSGin5G Server

Figure 7.1.4.3-1 shows the procedure for MSGin5G Server aggregating application-to-point messages each carrying small data targeted towards the target UE.

NOTE 1:	Aggregation of multiple messages can also be done at the Application Server; in this case it is implementation specific and out of the scope of the current specification.

Figure 7.1.4.3-1: MSGin5G Client aggregates messages towards target end point

1)	The Application Server initiates to send application-to-point message or a group message towards target UE(s) 1 and sends the request to MSGin5G Server and includes an indication that message aggregation is allowed for this message.

2)	The MSGin5G Server checks the message data size and the priority level to determine if the received message can be aggregated. For example, MSGin5G server finds that the messages have small payload size when compared to the maximum segment size that can be transmitted over available transport and are not high priority messages, which could be sent as per scheduling policy towards a selected target.

NOTE 2:	MSGin5G Server decides to continue aggregating messages until optimal use of segment size before sending message towards MSGin5G Client 1.

3)	The MSGin5G Server aggregates multiple MSGin5G message requests intended for the target UE and sends the aggregated message request as defined in Table 7.1.4.3-1 and Table 7.1.4.2-2 according to scheduling policy towards the MSGin5G Client 1.

Table 7.1.4.3-1: Aggregated message request (MSGin5G Server to MSGin5G Client)

| Information element | Status | Description |
| --- | --- | --- |
| Recipient MSGSin5G Service ID | M | The service identity of the receiving MSGin5G Client. |
| Message ID | M | Unique identifier of this message |
| Number of individual messages | M | Indicates total number of messages which are aggregated into single message |
| List of Individual messages | M | Each element in this list contains information as specified in Table 7.1.4.2-2 |

Editor's Note: Table 7.1.4.3-1 is identical to table 7.1.4.2-1 but lacks the Originator MSGin5G Service ID. It is FFS if only table 7.1.4.2-1 is sufficient.

NOTE 3:	Total size of Aggregated message request is less than or equal to maximum segment size allowed to transmit over available transport.

4)	The MSGin5G Client 1 splits the received Aggregated message request into multiple individual MSGin5G message requests per application and sends towards Application Client(s) as notifications.

NOTE 4:	The MSGin5G Server may aggregate messages towards the target UE if it receives message from multiple MSGin5G UEs (instead of application server).

* * * End of changes * * * *