3GPP TSG-SA6 Meeting #42-bis-e	S6-211076

- Online, , 12th Apr 2021 - 20th Apr 2021									 (Revision of S6-210855)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0037 | rev | 3 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Slice Capability Management functional model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2021-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This paper provides the detailed functional model, additional procedures, as well as the SEAL API details for the introduced network slice capability management (NSCM) functionality at SEAL spec (in clause 16 of TS23.434). |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 16, we provided input to the 16.2 clause and sub-clauses which relate to the functional model for NSCM. Moreover, we provide procedures for the network-based mechanism and remove the UE-based mechanism as provided in 16.3.2.3 due to dependencies with SA2). Also, 16.4 was provided (which relates to the SEAL API for NSCM operation). Finally, this contribution provides necessary updates in clause 16.3, clause 15 (for mapping the NSCM to service-based representation) and changes the name of slice enabler layer from NSE to NSCM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The details on functional model and API description for NSCM feature will not be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.1, 16.2.2, 16.2.3, 16.2.3.1 (new), 16.2.3.2 (new), 16.2.3.3 (new), 16.2.4, 16.2.4.1 (new), 16.2.4.2 (new), 16.2.4.3 (new), 16.2.4.4 (new), 16.2.4.5 (new), 16.3, 16.3.2.2.1, 16.3.2.2.2, 16.3.2.2.3, 16.3.2.2.4, 16.3.2.3, 16.3.2.4, 16.4, 16.4.1, 16.4.2, 16.4.2.1, 16.4.2.2, 15.2, 15.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

# 16	Network slice capability management

## 16.1	General

The network slice capability management is a SEAL service that offers network slice capability management capabilities, such as support for vertical application to slice re-mapping (which can be defined as the mapping of the UEs running a vertical application to different slice), to one or more vertical applications. In particular, network slice capability management uses a network-based mechanism to apply the slice re-mapping based on the network slice capability management server configuration, where the network slice capability management server acting as AF influences the URSP rules for the application traffic per UE by providing a guidance on the route selection parameters (including the S-NSSAI and DNN mapping).

NOTE:	The network slice capability management server, acts as an AF within the trusted domain (MNO-owned,or trusted third party).

## 16.2	Functional model

### 16.2.1	General

The functional model for the network slice capability management is based on the generic functional model specified in clause 6.2. It is organized into functional entities to describe a functional architecture which addresses the support for network slice capability management aspects for vertical applications. Since the slicing is a feature which considers the Uu interfaces, only the on-network functional model is specified in this clause.

### 16.2.2	Functional model description

Figure 16.2.2-1 illustrates the generic functional model for network slice capability management.

Figure 16.2.2-1: Functional model for network slice capability management

The network slice capability management client communicates with the network slice capability management server over the NSCM-UU reference point. The network slice capability management client provides the support for network slice capability management functions to the VAL client(s) over NSCM-C reference point. The VAL server(s) communicates with the network slice capability management server over the NSCM-S reference point. It is assumed that the network slice capability management server is deployed at the 5G system domain. The network slice capability management server, acting as AF, may communicate with the 5G Core Network functions via NEF (N33) reference point (for interactions with PCF).

16.2.3	Functional entities description

#### 16.2.3.1 General

The functional entities for network slice capability management SEAL service are described in the following subclauses.

#### 16.2.3.2 Network slice capability management server

The network slice capability management server functional entity provides the enablement of the network slicing aspects to support the VAL applications. Such enablement supports the mapping or migration of one or more vertical applications to one or more network slices (from a set of network slices, as provided by the 3GPP network system) as described in procedure in clauses 16.3.2.3 and 16.3.2.4. Such adaptation assumes that the UE is subscribed to more than one slice and is done via providing a guidance to update the URSP rules at the 5GS (denoted in 16.3.3 as network-based mechanism).

#### 16.2.3.3 Network slice capability management client

The network slice capability management client functional entity acts as the application client for the slice enablement. The network slice capability management client interacts with the network slice capability management server to trigger a network slice adaptation due to an application requirement change. This trigger may be due to an application QoS requirement change, a service operation change. The NSCM client may receive a network slice / DNN re-mapping notification from the NSCM server. The NSCM client may optionally notify the VAL client on the network slice / DNN re-mapping.

16.2.4	Reference points description

#### 16.2.4.1	General

The reference points for the functional model for network slice capability management are described in the following subclauses.

#### 16.2.4.2	NSCM-UU

The interactions related to network slice capability management functions between the network slice capability management server and the network slice capability management client are supported by NSCM-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.501 [10].

#### 16.2.4.3	NSCM-C

The interactions related to network slice capability management functions between the VAL client(s) and the network slice capability management client within a VAL UE are supported by the NSCM-C reference point. The NSCM client may receive an application requirement change over NSCM-C. Further, the NSCM client may provide a notification on the network slice adaptation upon successful adaptation of the slice to application mapping.

#### 16.2.4.4	NSCM-S

The interactions related to network slice capability management functions between the VAL server(s) and the network slice capability management server are supported by the NSCM-S reference point. This reference point is an instance of CAPIF-2 reference point as specified in 3GPP TS 23.222 [8].

#### 16.2.4.5	N33

The reference point N33 supports the interactions between the network slice capability management server and the NEF and is specified in 3GPP TS 23.501 [10]. N33 is used for the network-based mechanism for slice re-mapping, where NSCM server acting as AF influences the URSP rules for the application traffic per UE by providing a guidance on the route selection parameters (including the S-NSSAI and DNN mapping), as specified in TS23.502 clause 4.15.6.10, TS 23.503 clause 6.6.2.2, TS 23.548 clause 6.2.4.

## 16.3	Procedures and information flows for network slice capability management

### 16.3.1	General

### 16.3.2	Network slice adaptation for VAL application

#### 16.3.2.1	General

This subclause describes the procedure for network slice adaptation at the Network Slice Capability Management (NSCM) server, based on a request from a VAL server to adapt the network slice for the VAL application. This request is handled between the NSCM server and the NSCM client per each VAL UE of the VAL application.

16.3.2.2	Information flows

##### 16.3.2.2.1	Network slice adaptation request

Table 16.3.2.2.1-1 describes the information flow network slice adaptation request from the VAL server to the NSCM server.

Table 16.3.2.2.1-1: Network slice adaptation request

| Information element | Status | Description |
| --- | --- | --- |
| VAL service ID | M | The VAL service ID of the VAL application for which the network slice adaptation may corresponds to. |
| List of VAL UE IDs | M | List of the VAL UE IDs within the VAL service for which the slice adaptation request corresponds |
| Network slice adaptation cause | O | The cause which necessitate the network slice change (e.g. VAL service profile change, VAL service operation change). |
| Requested S-NSSAI | O | Indication of the new S-NSSAI which is requested. |
| Requested DNN | O | Indication of the new DNN which is requested. |

Editor's note: The details of Network slice adaptation request is FFS

##### 16.3.2.2.2	Network slice adaptation response

Table 16.3.2.2.2-1 describes the information flow network slice adaptation response from the NSCM server to the VAL server.

Table 16.3.2.2.2-1: Network slice adaptation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network slice adaptation with the underlying network. |

| Information element |  |  |
| --- | --- | --- |

##### 16.3.2.2.3	Network slice adaptation trigger

Table 16.3.2.2.3-1 describes the information flow Network slice adaptation trigger from the NSCM client to the NSCM server.

Table 16.3.2.2.3-1: Network slice adaptation trigger

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID(s) | M | The VAL UE ID(s) within the VAL service, for which the network slice adaptation trigger applies |
| VAL service ID | M | The VAL service ID of the VAL application for which the network slice configuration may corresponds to. |
| Requested S-NSSAI | M | Indication of the new S-NSSAI which is requested. |
| Requested DNN | O | Indication of the new DNN which is requested. |

##### 16.3.2.2.4	Network slice adaptation notification

Table 16.3.2.2.4-1 describes the information flow network slice adaptation notification from the NSCM server to the NSCM client and optionally to the VAL client.

Table 16.3.2.2.4-1: Network slice adaptation notification

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network slice adaptation. |

#### 16.3.2.3	Procedure for VAL server-triggered and network-based network slice adaptation for VAL application

Figure 16.3.2.3-1 illustrates the VAL server-triggered and network-based procedure where the NSCM server supports the network slice adaptation with the underlying 3GPP system for the VAL UEs of the VAL application.

Figure 16.3.2.3-1: Network slice adaptation for VAL application

1.	The VAL server sends a network slice adaptation request to the NSCM server for the VAL application (and the VAL UEs within the VAL application). This request may be in the form of exact requested network slice (and optionally DNN) for all the VAL UEs of the VAL application; or indication that the VAL application needs to be remapped to a different network slice (and optionally DNN).

2.	The NSCM server processes the request and triggers the network slice configuration per VAL UE within the VAL Application.

3.	The NSCM server acting as AF provides the updated S-NSSAI and DNN per VAL UE. In particular, NSCM server sends this information to the PCF via NEF as part of the AF-driven guidance for URSP determination to 5G system (as specified in TS23.502 clause 4.15.6.10, TS 23.503 clause 6.6.2.2, TS 23.548 clause 6.2.4). This guidance may update the route selection parameters to indicate different sets of PDU Session information (DNN, S-NSSAI) that can be associated with applications matching the application traffic.

NOTE: 	NSCM server provides the updated S-NSSAI/DNN as a suggestion/guidance to PCF; however it is up to PCF to decide whether to perform the slice/DNN re-mapping

4.  Upon successful adaptation of the route selection parameters, the NSCM server provides a network slice adaptation response to the VAL server, providing information on the fulfilment of the network slice adaptation request per VAL application.

#### 16.3.2.4	Procedure for VAL UE-triggered and network-based network slice adaptation for VAL application

Figure 16.3.2.4-1 illustrates the VAL UE-triggered and network-based procedure where the NSCM server supports the network slice adaptation with the underlying 3GPP system for the VAL UEs of the VAL application.

Pre-condition:

-	The NSCM client has connected to the NSCM server;

Figure 16.3.2.4-1: Network slice adaptation for VAL application

1.	The VAL client provides a new application requirement to the NSCM client, indicating a new service profile for the VAL application. This may be in the form of a change at the application QoS requirements, service operation change, or other application-related parameters.

2.	The NSCM client sends a network slice adaptation trigger to the NSCM server for the VAL application. This trigger may be in the form of exact requested network slice (and optionally DNN) for all the VAL UEs of the VAL application; or indication that the VAL application needs to be remapped to a different network slice (and optionally DNN).

3.	The NSCM server processes the request and triggers the network slice configuration per VAL UE within the VAL Application.

4.	The NSCM server acting as AF provides the updated S-NSSAI and DNN per VAL UE. In particular, NSCM server sends this information to the PCF via NEF as part of the AF-driven guidance for URSP determination to 5G system (as specified in TS23.502 clause 4.15.6.10, TS 23.503 clause 6.6.2.2, TS 23.548 clause 6.2.4). This guidance may update the route selection parameters to indicate different sets of PDU Session information (DNN, S-NSSAI) that can be associated with applications matching the application traffic.

NOTE: 	NSCM server provides the updated S-NSSAI/DNN as a suggestion/guidance to PCF; however it is up to PCF to decide whether to perform the slice/DNN re-mapping

5.  The NSCM server may send a notification to the NSCM client indicating success or failure.

## 16.4	SEAL APIs for network slice capability management

### 16.4.1	General

Editor's note: The definition of SEAL APIs for NSCM-UU is FFS

Table 16.4.1-1 illustrates the SEAL APIs for network slice capability management.

Table 16.4.1-1: List of SEAL APIs for network slice capability management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_NetworkSliceAdaptation | Network_slice_adaptation | VAL server | Request /Response |

### 16.4.2	SS_NetworkSliceAdaptation API

#### 16.4.2.1	General

API description: This API enables the VAL server to communicate with the network slice capability management server for network slice adaptation over NSCM-S.

#### 16.4.2.2	Network_Slice_Adaptation

API operation name: Network_Slice_Adaptation

Description: Requesting for network slice adaptation.

Known Consumers: VAL server.

Inputs: See subclause 16.3.2.2.1

Outputs: See subclause 16.3.2.2.2

See subclause 16.3.2.3 for the details of usage of this API operation.

* * * * * * * SECOND CHANGE * * * * * * *

# 15	Service-based interface representation of the functional model for SEAL services

## 15.1	General

The functional models for SEAL services is represented using functional entities and reference points between the functional entities as specified in subclause 6. The vertical applications consume the SEAL services in the form of APIs. Each SEAL service offers these APIs on a service-based interface to all its consumer entities.

## 15.2	Functional model representation

Figure 15.2-1 illustrates the service-based interface representation of the functional model for SEAL services.

Figure 15.2-1: SEAL generic functional model representation using service-based interfaces

The SEAL function(s) exhibit the service-based interfaces which are used for providing and consuming SEAL services. The service APIs are specified for each SEAL function enabled over the service-based interface. The service-based interfaces of specific SEAL services are specified in this document. All the interactions with SEAL are governed based on the reference point interactions of the functional models specified in subclause 6. VAL function represents the functionalities of the VAL server.

NOTE:	The service-based interface Sval for the VAL function is out of scope of the present document.

The service APIs offered by the SEAL function(s) are published and discovered on the CAPIF core function as specified in 3GPP TS 23.222 [8].

Editor's note:	Refinement of the SEAL services in service-based interface representation is FFS.

## 15.3	Service-based interfaces

Table 15.3-1 specifies the service-based interfaces supported by SEAL.

Table 15.3-1: Service-based interfaces supported by SEAL

| Service-based interface | Application functionEntity | Mapping server entity | APIs offered |
| --- | --- | --- | --- |
| Slm | Location management function | Location Management Server | Specified in subclause 9.4 |
| Sgm | Group management function | Group management server | Specified in subclause 10.4 |
| Scm | Configuration management function | Configuration management server | Specified in subclause 11.4 |
| Sim | Identity management function | Identity management server | Specified in subclause 12.4 |
| Skm | Key management function | Key management server | Specified in subclause 13.4 |
| Snrm | Network resource management function | Network resource management server | Specified in subclause 14.4 |
| Snscm | Network slice capability management function | Network slice capability management server | Specified in subclause 16.4 |
| Cccf | CAPIF core function | Not applicable | Specified in subclause 10 of 3GPP TS 23.222 [8] |