3GPP TSG-SA WG6 Meeting #43	S6-211299

e-meeting, 24th May – 2nd June 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0067 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API and information flow description for Temporary groups |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL, eV2XAPP |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The information flows and API for temporary group is to be added corresponding to the procedure in clause 10.3.10. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced information flows and API for temporary group procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete information for the procedure may hinder the progress of stage 3 work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.10.1, 10.3.2.a (new), 10.3.2.b (new), 10.3.2.c (new), 10.3.2.d (new), 10.3.2.e (new), 10.4.1, 10.4.x (new), 10.4.x.1 (new), 10.4.x.2 (new), 10.4.x.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.3.10.1	Temporary group formation within a VAL system

Figure 10.3.10.1-1 below illustrates the temporary group formation within a VAL system.

Temporary groups are formed by combining two or more groups. The temporary group formation is applicable only for groups configured with at least one common VAL service. The temporary group formation shall be rejected if any of the requested VAL services are not common to all the constituent VAL service groups.

Pre-conditions:

1.	The group management client, group management server, VAL server and the VAL group members belong to the same VAL system.

2.	The group management client has retrieved the group configurations of the groups.

Figure 10.3.10.1-1: Temporary group formation within a VAL system

1.	The group management client of the VAL user requests temporary group formation operation to the group management server, which is the group management server of one of the groups to be regrouped. The identities of the groups being combined shall be included in this message. The group management client may indicate the security level required for the temporary group. The group management client may indicate the priority level required for the temporary group.

2.	The group management server checks whether temporary group formation operation is performed by an authorized VAL user, based on group policy. The group management server checks whether group1 or group2 is a temporary group. If group 1 or group2 is a temporary group, then the group regrouping will be rejected, otherwise the group regrouping can proceed.

3.	The group management server creates and stores the information of the temporary group, including the temporary VAL group ID, the VAL group ID of the groups being combined, the priority level of the temporary group and the security level of the temporary group. If the authorized VAL user does not specify the security level and the priority level, the group management server shall set the lowest security level and the highest priority of the constituent groups. If VAL service types of the groups being combined are not identical, group management server determines the overlapping part and stores the VAL service list for the temporary group.

4.	The group management server notifies the VAL server regarding the temporary group creation with the information of the constituent groups, i.e. temporary VAL group ID, group1's VAL group ID and group2's VAL group ID. If VAL service list is included, VAL server stores it and provides VAL service types accordingly.

5.	The group management server notifies the VAL group members of the constituent VAL groups by sending temporary group formation notification messages.

6.	The VAL group members of the constituent VAL groups send individual temporary group formation notification response messages.

7.	The group management server provides a temporary group formation response to the group management client of the authorized VAL user. If VAL service list is included, group management client stores it and initiates VAL service types accordingly.

8.	The VAL group members of the constituent VAL groups individually request group configuration data from the group management server for the temporary group. The group configuration data includes security, priority, and other parameters.

* * * Next Change * * * *

#### 10.3.2.a	Temporary group formation request

Table 10.3.2.a-1 describes the information flow for the temporary group formation request from the group management client to the group management server.

Table 10.3.2.a-1: Temporary group formation request

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID list | M | List of VAL group IDs to be combined |
| VAL service list (see NOTE) | O | A subset of the common VAL service(s) to be applied for the temporary group |
| NOTE: If this information element is not present, all the VAL service(s) that are common to the groups being combined will be applicable for the temporary group. |  |  |

#### 10.3.2.b	Temporary group formation response

Table 10.3.2.b-1 describes the information flow for the temporary group formation response from the group management server to the group management client.

Table 10.3.2.b-1: Temporary group formation response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | O (see NOTE) | VAL group ID of the temporary group |
| VAL service list | O | List of VAL services whose service communications are to be enabled on this temporary group |
| Result | M | Indicates the success or failure of temporary group formation. |
| NOTE: Shall be present if the Result information element indicates that the temporary group formation operation is successful. Otherwise VAL group ID shall not be present. |  |  |

#### 10.3.2.c	Temporary group formation notify

Table 10.3.2.c-1 describes the information flow temporary group formation notify from the group management server to the VAL server.

Table 10.3.2.c-1: Temporary group formation notify

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID list | M | List of constituent VAL group IDs |
| VAL group ID | M | VAL group ID of the temporary group |
| VAL service list | O | List of VAL services whose service communications are to be enabled on this temporary group. |

#### 10.3.2.d	Temporary group formation notification

Table 10.3.2.d-1 describes the information flow temporary group formation notification from the group management server to the group management client.

Table 10.3.2.d-1: Temporary group formation notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID list | M | List of constituent VAL group IDs |
| VAL group ID | M | VAL group ID of the temporary group |

#### 10.3.2.e	Temporary group formation notification response

Table 10.3.2.e-1 describes the information flow temporary group formation notification response from the group management client to the group management server.

Table 10.3.2.e-1: Temporary group formation notification response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID list | M | List of constituent VAL group IDs |
| VAL group ID | M | VAL group ID of the temporary group |

* * * Next Change * * * *

### 10.4.1	General

Table 10.4.1-1 illustrates the SEAL APIs for group management.

Table 10.4.1-1: List of SEAL APIs for group management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_GroupManagement | Query_Group_Info | VAL server | Request /Response |
|  | Update_Group_Info | VAL server | Request /Response |
| SS_GroupManagementEvent | Subscribe_ Group_Info_Modification | VAL server | Subscribe/Notify |
|  | Notify_Group_Info_Modification | VAL server |  |
|  | Notify_Group_Creation | VAL server |  |
| SS_TemporaryGroups | Subscribe_TempGroupFormation | VAL server | Subscribe/Notify |
|  | Notify_TempGroupFormation | VAL server |  |

* * * Next Change * * * *

### 10.4.x	SS_TemporaryGroups API

#### 10.4.x.1	General

API description: This API enables the VAL server to communicate with the group management server for receiving information about temporary groups from the group management server over GM-S.

#### 10.4.x.2	Subscribe_TempGroupFormation operation

API operation name: Subscribe_TempGroupFormation

Description: Subscription for the notification of new temporary group formed at the group management server.

Known Consumers: VAL server.

Inputs:

Outputs:

Editor's note:	The details of the Subscribe_TempGroupFormation operation is FFS.

#### 10.4.x.3	Notify_TempGroupFormation operation

API operation name: Notify_TempGroupFormation

Description: Notification of the new temporary group formed at the group management server.

Known Consumers: VAL server.

Inputs: See clause 10.3.2.c

Outputs: None

See subclause 10.3.10.1 for the details of usage of this API operation.