3GPP TSG-SA WG6 Meeting #43	S6-211427

e-meeting, 24th May – 2nd June 2021	(revision of S6-211241)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0065 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SEAL enable 5G CN capabilties for SEAL groups |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2021-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per conclusions in TR 23.745, for the KI#17 (Using 5G CN capabilities for SEAL Groups), it is concluded in clause 9, that the solution #21 (Enabling 5G CN capabilities for SEAL groups) as the way forward for the normative phase and SEAL should be enhanced with this new solution. This CR proposes udpates to SEAL group management procedures with support of external group identifier identified by the 3GPP core network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SEAL group management procedures updated with support for external group Id. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enabling 5G CN capabilities for SEAL groups as concluded in FFAP work, will be unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.2, 10.3.1, 10.3.3, 10.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

### 10.3.1	General

Group management procedures apply to on-network VAL service only.

Group creation provides a dedicated VAL group to individual VAL users to enable the required communication for one or multiple VAL services. This includes the normal group creation by administrators or by authorized user/UE. The group management server supports the external group identifier within the VAL group document to enable the SEAL servers to consume the NEF services for the member UEs of the VAL group.

NOTE:	If an authorized VAL user/UE wants to participate in a new group created by the authorized VAL user/UE, then the authorized VAL user/UE needs to have been included in the new group as a member.

* * * Next Change * * *

### 10.3.3	Group creation

Figure 10.3.3-1 below illustrates the group creation operations by authorized VAL user/UE/administrator to create a group. It applies to the scenario of normal group creation by a VAL administrator or by authorized user/UE.

Pre-conditions:

1.	The group management client, group management server, VAL server and the VAL group members belong to the same VAL system.

2.	The authorized VAL user/UE/administrator is aware of the users' identities which will be combined to form the VAL group.

Figure 10.3.3-1: Group creation

1.	The group management client of the authorized VAL user/UE/administrator requests group create operation to the group management server. The identities of the users or UEs being combined and the information of the VAL services that are enabled on the group shall be included in this message.

2.	During the group creation, the group management server creates and stores the information of the group. The group management server performs the check on the policies e.g. maximum limit of the total number of VAL group members for the VAL group(s). The external group identifier, identifying the member UEs of the VAL group at the 3GPP core network, is stored in the newly created VAL group’s configuration information.

NOTE:	The exact policies are out of scope of the present document.

3.	The group management server may conditionally notify the VAL server regarding the group creation with the information of the group members.

4.	The VAL group members of the VAL group are notified about the newly created VAL group configuration data.

5.	The group management server provides a group creation response to the group management client of the administrator/authorized VAL user/UE.

* * * Next Change * * *

### 10.3.7	Location-based group creation

Figure 10.3.7-1 below illustrates the location-based group creation.

Pre-conditions:

1.	The group management client, group management server, VAL server, location management server and the VAL group members belong to the same VAL system.

2.	The authorized VAL user/UE/administrator is not aware of the users' or UE identities which will be combined to form the VAL group.

Figure 10.3.7-1: Location-based group creation

1.	The group management client or the VAL server requests location-based group create operation to the group management server. The location criteria for determining the identities of the users or UEs to be combined shall be included in this message.

2.	The group management server requests the location management server for obtaining the users or UEs corresponding to the location information.

3.	The location management server composes the list of users or UEs within the requested location.

4.	The group management server receives the composed list of users or UEs from the location management server.

5.	During the group creation, the group management server creates and stores the information of the location-based group. The group management server performs the check on the policies e.g. maximum limit of the total number of VAL group members for the VAL group(s).

NOTE:	The exact policies are out of scope of the present document.

Editor's Note:	Group updates due to Users or UEs moving in or out of the location is FFS.

6.	If an external group identifier, identifying the member UEs of the VAL group at the 3GPP core network is available, then the external group ID is stored in the newly created VAL group’s configuration information.

7.	The group management server provides a location-based group creation response to the group management client or the VAL server.

* * * End of	 Changes * *