3GPP TSG-SA WG6 Meeting #43	S6-21xxxx

e-meeting, 24th May – 2nd June 2021	(revision of S6-211205)

Source:	Ericsson

Title:	E3 exposure context transfer

Spec:	3GPP TS 23.558 v2.1.0

Agenda item:	7.6

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution adds a E3 exposure context transfer btw. the EESes (triggered by T-EAS).

2. Reason for Change

As discussed in conf.call on April 1st, the EDGE-3 exposure context (created by consumer EAS interaction with EES and stored in the producer EES) may be transferred from the S-EES to the T-EES so that the T-EAS can utilize the existing exposure subscriptions in the T-EES.

As a valid option, if the T-EAS receives the corresponding correlation id from the S-EAS, it can request the T-EES by including the correlation id and then the T-EES can retrieve the EDGE-3 exposure context from the S-EES; otherwise, the T-EAS can re-subscribe the exposure subscriptions on the T-EES.

This is similar as “Inter NG-RAN node N2 based handover with I-SMF change” (see cl.4.23.7.3 of TS 23.502) where both AMF and I-SMF are changed and SM context is relocated to the target I-SMF based on target AMF provided “SM context ID”.

The benefit is signalling reduction by re-establishing E3 context in the target side in one shot, instead of creating them one by one. Also, if EAS layer is willing to continue the state in the existing E3 subscritptions (e.g. number of remaining reports, remaining report duration, analytics state), this EDGE API provides the help to achieve E3 exposure subscription continuitity.

Previous discussion shows T-EAS can also perform bundled subscription on T-EES.

The fact is that there is no such bulk subscription for different edge-3 APIs, the T-EAS has to do it one by one. It is not expected to design an aggregated API for EDGE-3.

Also, the ongoing analytics triggered by the existing E3 subscription (e.g. location estimate) is something better to pertain (see TS 23.288 cl.6.1B for analytics context transfer) when EAS is changed so that the accurate analytics information (expected UE location) can be given to the T-EAS to support Service Continuity planning feature.

Rev1: improve the procedure description in 8.8.2.x; removed EN since each subscription has its own subscription ID as described in 8.6, using a single EAS context ID for EES and EAS is not suitable.

Rev2: besides the current EAS driven E3 context reloation, captured Huawei’s desire to have EES driven E3 context relocation which adds E3 subscription id as correlation information into the SSC of EEC context.

Considering the fact that EEC might not be registered in the EES, hence the EEC context is not established. In order to have such EES driven option, it is captured the applied pre-conditions for such EES driven E3 context relocation in 8.9.1.5:

- EEC registration happened with the UE ID supplied from EEC.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes in TS 23.558.

* * * First Change * * * *

### 6.5.10	EDGE-9

EDGE-9 reference point enables interactions between two EESs. EDGE-9 reference point may be provided between EES within different EDN (Figure 6.5.10-1) and within the same EDN (Figure 6.5.10-2).

Figure 6.5.10-1: Inter-EDN EDGE-9

Figure 6.5.10-2: Intra-EDN EDGE-9

EDGE-9 supports:

a)	discovery of T-EAS information to support ACR;

b)	EEC context retrieval and relocation procedures; and

c)	EDGE-3 exposure context retrieval procedure.

* * * Next Change * * * *

### 6.7.2	APIs provided by the Edge Enabler Layer

Table 6.7.2-1 summarizes the APIs exposed by the ECS.

Table 6.7.2-1: APIs provided by the ECS

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eecs_ServiceProvisioning | EEC | 8.3 |
| Eecs_EESRegistration | EES | 8.4.4 |
| Eecs_TargetEESDiscovery | EES | 8.8.3.3 |

Table 6.7.2-2 summarizes the APIs exposed the EES.

Table 6.7.2-2: APIs provided by the EES

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eees_EECRegistration | EEC | 8.4.2 |
| Eees_EASRegistration | EAS | 8.4.3 |
| Eees_EASDiscovery | EEC | 8.5 |
| Eees_UELocation | EAS | 8.6.2 |
| Eees_ACRManagementEvent | EAS | 8.6.3 |
| Eees_AppClientInformation | EAS | 8.6.4 |
| Eees_UEIdentifier | EAS | 8.6.5 |
| Eees_SessionWithQoS | EAS | 8.6.6 |
| Eees_TargetEASDiscovery | EAS, EES | 8.8.3.2 |
| Eees_AppContextRelocation | EEC | 8.8.3.4 |
| Eees_ACREvents | EEC | 8.8.3.5 |
| Eees_AutomatedACR | EAS | 8.8.3.6 |
| Eees_ContextRetrieval | EAS | 8.8.3.x |

NOTE:	The event exposure related APIs (e.g. Eees_EASDiscovery and Eees_ACREvents) can be realized as single event subscription API.

* * * Next Change * * * *

### 8.2.8	EEC Context

The EEC Context includes information about an EEC for receiving edge enabler services.

Table 8.2.8-1: EEC Context

| Information element | Status | Description |
| --- | --- | --- |
| EEC ID | M | Unique identifier of the EEC. |
| EEC Context ID | M | Identifier assigned to the EEC Context |
| Source EES Endpoint | M | The endpoint address (e.g., URI, IP address) of the EES that provided EEC context ID. |
| UE Identifier | O | The identifier of the hosting UE (i.e., GPSI or identity token) |
| List of EDGE-1 subscriptions | O | List of subscriptions IDs for capability exposure to the EEC ID |
| UE location | O | Latest UE location of the UE hosting the EEC which was available at the EES. |
| List of AC Profiles | O | Information about the ACs as described in Table 8.2.2-1. |
| List of Service Session Contexts | O | List of associated Service Session Context IEs. Each Service Session Context includes information maintained by the EES for the services (involving UE related resources) received from an EAS registered to the EES. |
| > Service Session Context | M | Service Session Context is described in Table 8.2.8-2 |

Editor's Note:	It is FFS whether the EEC Context ID can be optional.

Table 8.2.8-2: Service Session Context

| Information element | Status | Description |
| --- | --- | --- |
| EAS ID | M | Identifier of the EAS providing the application services |
| EAS Endpoint | M | Endpoint information of the EAS. |
| AC ID | O | Identifier of the AC ID for which the service session is provided, if determined. |
| List of EDGE-3 subscriptions | O | List of subscriptions IDs for capability exposure to the EAS. |

Editor's Note:	Additional IE(s) for the subscriptions to CN the list of IEs is FFS, pending SA2 discussions (see TR 23.748 clause 9.2.6).

Editor's Note:	It is FFS whether the EEC Context information captured in table 8.2.A-2 requires further updates and alignment to include Edge-3 subscription information.

* * * Next Change * * * *

#### 8.8.2.4	S-EAS decided ACR scenario

The procedure in this clause illustrates the scenario for S-EAS decided ACR.

In this procedure, the S-EAS may detect the need of ACR locally or is notified by the S-EES. The S-EAS make the decision about whether to perform the ACR, and starts the ACR at a proper time.

Pre-conditions:

1.	The S-EAS may depend on the receipt of certain User plane path management events from the S-EES, e.g. "user plane path change" events or "ACR monitoring" events, to detect the need for an ACR. For the following procedure it is assumed that the S-EAS has subscribed to continuously receive the respective events from the S-EES; and

2.	The EEC has subscribed to receive ACR information notifications for target information notification events and ACR complete events from the S-EES, as described in clause 8.8.3.5.2.

Figure 8.8.2.4-1: S-EAS decided ACR scenario

The S-EAS decided ACR scenario is outlined with four main phases: detection, decision, execution and clean up.

Phase I: ACR Detection

1.	The S-EAS either receives notifications from source Edge Enabler Sever indicating that ACR may be required ("ACR monitoring" event), or self detects the need for ACR (e.g. upon receipt of a "user plane path change" event). If the notification indicates "ACR monitoring" event, then the notification will also contain the T-EAS information (see clause 8.6.3.2.3). The S-EAS may detect that ACR may be required for an expected or predicted UE location in the future as described in clause 8.8.1.

NOTE 1:	How the S-EAS self detects the local need for ACR is outside the scope of this specification.

Phase II: ACR Decision

2.	The S-EAS makes the decision to perform the ACR

NOTE 2:	How the S-EAS determines when to start the ACR is outside the scope of this specification.

Phase III:	ACR Execution

3.	The S-EAS discovers the T-EAS as described in clause 8.8.3.2. When in step 1 the ACR has been triggered for service continuity planning, then UE Location and Target DNAI values in the Retrieve T-EES procedure contain the expected UE Location and expected Target DNAI. After S-EAS determines the T-EAS to use, the S-EAS may apply the AF traffic influence with the N6 routing information of the T-EAS in the 3GPP Core Network (if applicable).

4.	The S-EES sends the target information notification to the EEC as described in clause 8.8.3.5.3.

5.	The S-EAS transfers the application context to the T-EAS selected in step 3. This process is out of scope of the present specification. After successful ACT, if T-EAS received EDGE-3 exposure context ID(s) from the S-EAS, the T-EAS may initiate the procedure for EDGE-3 exposure context retrieval as described in clause 8.8.3.x.

NOTE x:	The EDGE-3 exposure context retrieval is applicable if T-EAS and S-EAS belong to the same ASP.

Editor's Note:	It is FFS whether the EDGE-3 exposure context retrieval procedure is applicable in other scenarios.

Phase IV:	Post-ACR clean up

When in step 1 the ACR has been triggered for service continuity planning, if the UE does not move to the predicted location, the EEC does not connect to T-EES, the AC does not connect to the T-EAS. Step 6 is skipped.

NOTE 3:	When in step 1 the ACR has been triggered for service continuity planning, step 6 would only be performed after the UE moves to the expected location.

6.	All required entities perform clean-up. The S-EAS informs the S-EES of the complete of ACT and the S-EES informs the EEC of the completion of ACR as described in clause 8.8.3.5.3.

* * * Next Change * * * *

#### 8.8.3.x	Retrieve EDGE-3 exposure context procedure

Figure 8.8.3.x-1 illustrates the procedure for EDGE-3 exposure context retrieval (e.g. location subscription as described in clause 8.8.2.2.3). T-EES retrieves the EDGE-3 exposure context data from the S-EES based on request sent by the T-EAS.

NOTE 1:	This procedure is suitable for the single UE exposure subscription over EDGE-3 which is UE related.

Pre-condition:

1.	The application context transfer is finished in the EAS.

Figure 8.8.3.x-1: EDGE-3 exposure context retrieval procedure

1.	The T-EAS sends Retrieve EDGE-3 exposure context request message to the T-EES.

2.	If the request is authorized by the T-EES, the T-EES sends the Retrieve EDGE-3 exposure context request to the S-EES, the request includes a list of EDGE-3 exposure context ID(s) received from step 1.

3.	If the request is authorized by the S-EES, the S-EES sends the Retrieve EDGE-3 exposure context response message to the T-EES. The context data is included in the response message.

4.	The T-EES responds with Retrieve EDGE-3 exposure context response message to the T-EAS. If EDGE-3 exposure context data is received from the S-EES, the T-EES stores the context data and may update the list of EDGE-3 exposure context ID(s). The T-EES also updates the Notifiation Target Address (if received) provided by the T-EAS for each context ID.

NOTE 2:	This context ID can be represented by subscription ID in the EDGE-3 interaction.

8.8.4.x	Retrieve EDGE-3 exposure context request (from T-EAS to T-EES)

Table 8.8.4.x-1 describes the information elements for the Retrieve EDGE-3 exposure context request from T-EAS to T-EES.

Table 8.8.4.x-1: Retrieve EDGE-3 exposure context request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | O | Identifier of the UE |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| List of EDGE-3 exposure context ID(s) | M | A list of the EDGE-3 exposure context identifiers. |
| Notification Target Address | O | Notification Target Address of the EAS where the notification is to be sent by the EES. |
| Source EES Endpoint | M | Source EES Endpoint information |

#### 8.8.4.y	Retrieve EDGE-3 exposure context response (from T-EES to T-EAS)

Table 8.8.4.y-1 describes the information elements for the Retrieve EDGE-3 exposure context response from T-EES to T-EAS.

Table 8.8.4.y-1: Retrieve EDGE-3 exposure context response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful |
| > List of updated EDGE-3 exposure context ID(s) | O | If the result is successful, the EES may provide the updated context identifier(s). The absence of an identifier implies no change for the context identifier. |
| Failure response | O | Indicates that the request failed |
| > Cause | O | Indicates the cause of request failure |

#### 8.8.4.a	Retrieve EDGE-3 exposure context request (from T-EES to S-EES)

Table 8.8.4.a-1 describes the information elements for the Retrieve EDGE-3 exposure context request from T-EES to S-EES.

Table 8.8.4.a-1: Retrieve EDGE-3 exposure context request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | O | Identifier of the UE |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| List of EDGE-3 exposure context ID(s) | M | A list of the EDGE-3 exposure context identifiers. |

#### 8.8.4.b	Retrieve EDGE-3 exposure context response (from S-EES to T-EES)

Table 8.8.4.b-1 describes the information elements for the Retrieve EDGE-3 exposure context response from S-EES to T-EES.

Table 8.8.4.b-1: Retrieve EDGE-3 exposure context response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful |
| > EDGE-3 exposure context information | M | The context information of EDGE-3 exposure data. |
| Failure response | O | Indicates that the request failed |
| > Cause | O | Indicates the cause of request failure |

* * * Next Change * * * *

#### 8.8.5.1	General

Table 8.8.5.1-1 illustrates the APIs for ACR.

Table 8.8.5.1-1: ACR APIs

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_TargetEASDiscovery | Request | Request/Response | EAS, EES |
| Eecs_TargetEESDiscovery | Request | Request/Response | EES |
| Eees_AppContextRelocation | Request | Request/Response | EEC |
| Eees_ACREvents | Subscribe | Subscribe/Notify | EEC |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |
| Eees_ContextRetrieval | Request | Request/Response | EAS, EES |
| Eees_AutomatedACR | Request | Request/Response | EAS |

* * * Next Change * * * *

#### 8.8.5.x	Eees_ContextRetrieval_Request operation

API operation name: Eees_ContextRetrieval_Request

Description: The consumer requests for the context retrieval.

Inputs: See clause 8.8.4.x, 8.8.4.a.

Outputs: See clause 8.8.4.y, 8.8.4.b.

See clause 8.8.3.x for details of usage of this operation.

* * * Next Change * * * *

#### 8.9.1.5	Other EEC Context handling

Elements of the list of Service Session Context(s) information shall be created by the EES when it determines that a registered EAS is providing services to an AC on the served EEC.

Editor's Note:	How the EES determines that a registered EAS is providing services to an AC is FFS.

Elements of the list of Service Session Context(s) information shall be determined to be stale when the EES determines that a registered EAS is no longer providing services to an AC on the served EEC.

NOTE 1:	In this version of specification, mechanisms used by EES to determine that a registered EAS is no longer providing services to an AC on a served EEC are implementation-dependent.

An EEC Context shall be updated as follows:

-	When EEC Context(s) are created, either after a registration request or based on EEC Context relocation procedure, the EES shall check whether the UE Identifier corresponds to an existing EEC Context and update the EEC Context accordingly.

-	When EEC subscription requests corresponding to the EEC ID are processed, the "List of EDGE-1 subscriptions" shall be updated accordingly

-	When EAS subscription requests corresponding to the UE ID in the EEC context are processed, the "List of EDGE-3 subscriptions" shall be updated accordingly

NOTE xx:	If EEC is registered in the EES with UE ID supplied and the "List of EDGE-3 subscriptions" is present in the Service Session Context, the S-EES can relocate EDGE-3 exposure subscription context (e.g. location subscription as described in clause 8.8.2.2.3) related to a UE to the T-EES.

* * * End of Changes * * * *