3GPP TSG-SA WG6 Meeting #43	S6-21xxxx

e-meeting, 24th May – 2nd June 2021	(revision of S6-211168)

Source:	Huawei, Hisilicon

Title:	APIs provided by Message Gateway

Spec:	3GPP TS 23.554

Agenda item:	0.3.0

Document for:	Approval

Contact:	Hongxia Hao, haohongxia@huawei.com

1. Introduction

This pCR proposes APIs provided by Message Gateway.

2. Reason for Change

In 5GMARCH TR, there is description that more APIs are expected to be identified during the normative phase.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554 v0.3.0.

* * * First Change * * * *

# 8	Procedures, APIs and information flows

Editor's note:	The clause number of the following procedure will be changed in the future version.

* * * Next Change * * * *

## 8.Z	APIs provided by Message Gateway

### 8.Z.1	L3G_Message_Delivery API

#### 8.Z.1.1	General

API description: This API enables the MSGin5G Server to delivery MSGin5G Message to the Legacy 3GPP Message Gateway.

#### 8.Z.1.2	Send_Message operation

API operation name: Send_MSGin5G_Message

Description: Send a MSGin5G Message to Legacy 3GPP Message Gateway.

Known Consumers: M5S.

Inputs: Refer subclause 8.2.3

Outputs: Refer subclause 8.2.3

See subclause 8.2.2 for the details of usage of this API operation.

### 8.Z.2	N3G_Message_Delivery API

#### 8.Z.2.1	General

API description: This API enables the MSGin5G Server to delivery MSGin5G message to the Non-3GPP Message Gateway.

#### 8.Z.2.2	Send_Message operation

API operation name: Create_MSGin5G_Message

Description: Send a MSGin5G message to Non-3GPP Message Gateway.

Known Consumers: M5S.

Inputs: Refer subclause 8.2.3

Outputs: Refer subclause 8.2.3

See subclause 8.2.2 for the details of usage of this API operation.

* * * Next Change * * * *

### 8.Z.3	L3G_Delivery_Status_Report API

#### 8.Z.3.1	General

API description: This API enables the MSGin5G Server to delivery MSGin5G Message to the Legacy 3GPP Message Gateway.

#### 8.Z.3.2	Report_Message_Delivery_Status operation

API operation name: Report_Message_Delivery_Status

Description: Send a MSGin5G Message delivery status report to Legacy 3GPP Message Gateway.

Known Consumers: M5S.

Inputs: Refer subclause 8.2.3

Outputs: Refer subclause 8.2.3

See subclause 8.2.2 for the details of usage of this API operation.

### 8.Z.4	N3G_Delivery_Status_Report API

#### 8.Z.4.1	General

API description: This API enables the MSGin5G Server to delivery MSGin5G message to the Non-3GPP Message Gateway.

#### 8.Z.4.2	Report_Message_Delivery_Status operation

API operation name: Report_Message_Delivery_Status

Description: Send a MSGin5G message delivery status report to Non-3GPP Message Gateway.

Known Consumers: M5S.

Inputs: Refer subclause 8.2.3

Outputs: Refer subclause 8.2.3

See subclause 8.2.2 for the details of usage of this API operation.