3GPP TSG-SA WG6 Meeting #43	S6-211372

e-meeting, 24th May – 2nd June 2021	(revision of S6-211272)

3GPP TSG|WG-??? Meeting #nn 	XX-yyxxxx

Location, Country, Date 	(revision of xx-yyxxxx)

- Source:	CATT

- Title:	New SID on study on 5G-enabled fused location service capability exposure

- Document for:	Approval

Agenda Item:	9

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Study on 5G-enabled fused location service capability exposure

## Acronym: FS_5GFLS

## Unique identifier:

Potential target Release: Rel-18

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No |  |  |  |  |  |
| Don't know | X |  | X |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| 5G_HYPOS | SA1 | 800013 | 5G positioning services |
| EAV | SA1 | 840039 | 5G Enhancement for UAVs |
| LPHAP | SA1 | 910036 | Low Power High Accuracy Positioning for industrial IoT scenarios |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 820016 | Enhancement to the 5GC LoCation Services | Normative work for 5GC LoCation Services in Rel-16 |
| 820025 | Study on application layer support for Factories of the Future in 5G network | Study related to application layer support for future factories in Rel-17 |
| 830077 | NR Positioning Support | Normative work for NR positioning in Rel-16 |
| 870001 | Enhancement to the 5GC LoCation Services-Phase 2 | Normative work for 5GC LoCation Services in Rel-17 |
| 890025 | Enhanced application layer support for V2X services | Normative work for application layer enhanced support for V2X in Rel-17 |
| 900025 | Application layer support for Unmanned Aerial System (UAS) | Normative work for application layer support for UAS in Rel-17 |
| 900160 | NR Positioning Enhancements | Normative work for NR positioning in Rel-17 |

Dependency on non-3GPP (draft) specification: none

## 3	Justification

SA1 has specified in 3GPP TS 22.261, 3GPP TS 22.104, 3GPP TS 22.125 the requirements for positioning service and performance requirements (including high-accuracy positioning) for various vertical applications. It is addressed in 3GPP TS 22.261 that the combination of 3GPP positioning technologies with non-3GPP positioning technologies (such as GNSS, TBS, sensors, WLAN/Bluetooth-based positioning) can support the improvement of accuracy, positioning service availability, reliability and/or confidence level, the reduction of positioning service latency, the increase of the update rate of the position-related data, increase the coverage (service area). Adaptability and flexibility are among the key features of the 5G system to serve a wide diversity of verticals and services, in different environments (e.g. rural, urban, indoor). This applies to high accuracy positioning and translates into the ability to satisfy different levels of services and requirements, for instance on performance (e.g. accuracy, positioning service availability, positioning service latency) and on functionality (e.g. security).

In SA6, the discussions in e.g. eV2XAPP, UASAPP, FS_FFAPP have involved different sources of location services (e.g. 3GPP technologies and non-3GPP technologies) available at the application layer, the application enabler needs to consider the combination or fusion of different location technologies as to improve the location performance, adapt to different environments, make sure UE provided location is not spoofed and meet the diversified vertical service’s needs.

## 4	Objective

The objectives of the study include the following:

1)	Based on SA1 service requirements, SA2 positioning service exposure and existing SA6 work, specify architectural requirements, architectural enhancements or new architecture in application layer through the study of following aspects:

-	Enabling location performance (accuracy, availability and latency) enhancements through combined use and fusion of 3GPP and non-3GPP location technologies at the application layer;

-	Enabling continuity of location services in different environments at the application layer;

-	Identification and configuration of location related requirements (including location QoS) for vertical application services with the use of SA2-defined mechanisms;

-	Architecture enhancement leveraging 5G positioning and location services;

-	Location data handling capabilities (e.g. logging, management);

-	Enhanced efficiency on location service processing (e.g. by leveraging edge computing capabilities);

-	Enabling value-added location service capabilities exposure to vertical applications;

-	Enhancements on SEAL location management addressing the aspects above.

2)	Identify key issues and develop solutions based on aspects above.

NOTE:	This study will not duplicate solutions already available in SA2 and RAN.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | 23.XXX | Study on 5G-enabled fused location service capability exposure | TSG#94 | TSG#95 | Zhang, Ling, CATT, zhangling@catt.cn |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

## 6	Work item Rapporteur(s)

Zhang, Ling, CATT, zhangling@catt.cn

## 7	Work item leadership

SA6

## 8	Aspects that involve other WGs

SA2 for core network architecture aspects, SA3 for security aspects, SA5 for charging aspects.

## 9	Supporting Individual Members

Supporting IM name

CATT

China Mobile

China Unicom

China Telecom

CALTTA

Convida Wireless LLC

Tencent

Lenovo, Motorola Mobility

Huawei