3GPP TSG-SA WG6 Meeting #43	S6-211420

e-meeting, 24th May – 2nd June 2021	(revision of S6-211260)

Source:	Samsung, Convida

Title:	Pseudo-CR on EEC context continuity feedback

Spec:	3GPP TS 23.558 v2.1.0

Agenda item:	7.6

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

This contribution resolves the following EN:

Editor's Note: whether the EEC registration response provides feedback with regard to the outcome of the EEC context relocation is FFS.

2. Reason for Change

EEC context can contain information about EEC's subscriptions over EDGE-1 interface. During EEC context continuity, if these subscriptions are successfully transferred from the S-EES to the T-EES, then the EEC is not required to subscribe again. However, if the context continuity fails, the EEC needs to create new subscriptions with the T-EES.

To make an informed decision on creating new subscriptions, the EEC needs to know the status of context retrieval at the T-EES; which is currently not available.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v2.1.0.

* * * First Change * * * *

##### 8.4.2.2.2	EEC registration

Figure 8.4.2.2.2-1 illustrates EEC registration procedure.

Pre-conditions:

1.	The EEC is authorized to access the EES for the purpose of performing registration and has received relevant security credentials as specified in clause 8.11; and

2.	The EEC has received service provisioning information from the ECS, including information for accessing the EES.

Figure 8.4.2.2.2-1: EEC registration procedure

1.	The EEC sends EEC registration request to the EES. The request from the client includes the security credentials received after successful authorization for edge computing services and may include a proposed expiration time. The request also optionally includes information indicating to the EES how the EEC expects to use the services of the EES.

If the EEC is moving to this EES from the purview of another EES, called source EES, the request from the EEC may include the identity and endpoint of the source EES and an EEC context ID that was provided by the source EES to maintain continuity of the EEC context and to authorize EEC context relocation.

2.	Upon receiving the request from the EEC, the EES validates the registration request and verifies the security credentials. The EES further determines whether the requirements that were indicated in the AC Profile(s) can be fulfilled and reserves corresponding resources.

3.	Upon successful validation of the request, if the received EEC registration request contains an EEC context ID and a source EES Endpoint, the EES performs a EEC Context Pull relocation (clause 8.9.2.2) from the source EES. The source and target EES perform EEC Context handling as detailed in 8.9.1.

NOTE 1:	Only a single EEC Context ID may be provided in the EEC registration request.

NOTE 2:	In this version of specification, each registration procedure relocates a single EEC context.

If the EEC registration request fails after the EEC Context Pull relocation, e.g., the EES cannot reserve the necessary resources while meeting the capability requirements of the existing registered EECs, the EES shall determine the EEC Context information stale and send a failure response with a corresponding cause.

4.	The EES sends a successful EEC registration response, which includes the registration ID and may include a newly assigned EEC context ID. If step 3 was executed, the EEC registration response also includes EEC context retrieval result. The EEC stores the new EEC context ID and uses it if and when it registers with another EES. The EES may also provide an expiration time to indicate to the EEC when the registration will automatically expire. To maintain the registration, the EEC shall send a registration update request prior to the expiration. If a registration update request is not received prior to the expiration time, the EES shall treat the EEC as implicitly de-registered.

If the EEC context retrieval result indicates that the EEC context pull relocation was not successful, then the EEC performs the required EDGE-1 subscriptions at the T-EES.

* * * Next Change * * * *

##### 8.4.2.3.3	EEC registration response

Table 8.4.2.3.3-1 describes information elements in the EEC registration response from the EES to the EEC.

Table 8.4.2.3.3-1: EEC registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the registration request was successful. |
| > Registration ID | M | Identifier of the EEC registration. |
| > Expiration time | M | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. |
| > EEC context ID | O | Identifier of the EEC Context information available at the EES that performed the registration. |
| > EEC context retrieval result (NOTE) | O | Indicates whether the EEC context retrieval from the S-EES was successful or not. |
| Failure response | O | Indicates that the registration request failed. |
| > Cause | M | Provides the cause for registration request failure. |
| NOTE: The EEC context retrieval IE shall be included if the EEC registration request included EEC Context ID and S-EES details. |  |  |