3GPP TSG-SA WG6 Meeting #44	S6-211573_rev1

e-meeting, 12th – 20th July 2021	(revision of S6-21xxxx)

Source:	Huawei, Hisilicon

Title:	Remove ENs with no actions

Spec:	3GPP TS 23.554

Agenda item:	8.2

Document for:	Approval

Contact:	Hongxia Hao, haohongxia@huawei.com

1. Introduction

This pCR is to remove some ENs with no actions.

2. Reason for Change

These abbreviations are necessary for understanding this specification.

The ENs in clause1 , clause 3.1 are not needed, nothing needs to do based on these two ENs.

The EN in 8.7.1.5 has been addressed in 8.2.3.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554 v1.0.0

* * * First Change * * * *

# 1	Scope

The present document specifies the functional architecture, procedures, information flows and APIs for MSGin5G Service. MSGin5G Service provides messaging communication capability in 5GS especially for Massive Internet of Things (MIoT).

MSGin5G Service includes the following message communication models:

-	Point-to-Point message;

-	Application-to-Point message/ Point-to-Application message;

-	Group message;

-	Broadcast message.

The corresponding service requirements are defined in 3GPP TS 22.262 [2].

* * * Next Change * * * *

- 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Application-to-Point messaging: message that is originated at an Application Server in the network and terminated at a UE.

Broadcast Messaging: A MSGin5G message that is delivered to UEs in a Broadcast Area.

MSGin5G Service: an MNO message service using the 5G System that enables Point- to-Point, Application-to-Point, Point-to-Application, Group and Broadcast message delivery for thing-to-thing communication and person-to-thing communication.

MSGin5G Server: A server in MSGin5G Service that receives and delivers MSGin5G messages among MSGin5G Service endpoints.

Legacy 3GPP Message Gateway: the entity in MSGin5G Service to support interworking with Legacy 3GPP UEs.

Non-3GPP Message Gateway: the entity in MSGin5G Service to support interworking with Non-3GPP UEs.

Messaging Topic: an identifier for a topic to which a UE or an Application Server can subscribe to in order to receive messages that are characterized by a Message Topic.

Message Gateway: general terminology for Legacy 3GPP Message Gateway or Non-3GPP Message Gateway.

MSGin5G Client: the client that enables MSGin5G message sending and receiving.

MSGin5G message: the message defined in the present specification that is exchanged between the MSGin5G Service endpoints under the MSGin5G Service.

MSGin5G UE:  the UE that uses MSGin5G Client in MSGin5G Service.

Legacy 3GPP UE: the UE that supports legacy 3GPP message sending and receiving (e.g. SMS, NIDD, etc) in MSGin5G Service.

Non-3GPP UE: the UE that supports non-3GPP message sending and receiving (e.g. RCS message as specified in GSMA PRD RCC.07 [3], OMA LWM2M message as specified in OMA OMA-ERELD-LightweightM2M [4]) in MSGin5G Service.

NOTE:	The MSGin5G UE utilizes MSGin5G Client in MSGin5G Service. The Legacy 3GPP UE and Non-3GPP UE does not utilize MSGin5G Client in MSGin5G Service.

Non-MSGin5G UE: general terminology for Legacy 3GPP UE or Non-3GPP UE.

MSGin5G Group: the group of UEs which members may be MSGin5G UE, Legacy 3GPP UE and Non-3GPP UE.

Point-to-Application messaging: message that is originated at a UE and terminated at an Application Server.

Point-to-Point messaging: message that originates at a UE and terminates at a UE, where at least one of the end points is a MSGin5G UE.

Broadcast Area: an area consisting of one or more cells where the broadcast message is delivered.

* * * Next Change * * * *

- 8.7.1.5	From Non-3GPP UE to MSGin5G UE

This procedure is used for message reply from Non-3GPP UE to MSGin5G UE.

Figure 8.7.1.5-1 shows the message delivery procedure from Non-3GPP UE to MSGin5G UE.

Pre-conditions:

1.	MSGin5G Client in MSGin5G UE and Non-3GPP UE has registered with the MSGin5G server.

2.	The Non-3GPP UE received a message from the MSGin5G UE.

3.	The Non-3GPP Message Gateway is aware of the Non-3GPP message client on the Non-3GPP UE and provides the mapping to MSGin5G Service ID.

4.	The Non-3GPP UE replies to the MSGin5G UE upon receiving the message from the MSGin5G UE.

Figure 8.7.1.5-1: Non-3GPP UE replies to MSGin5G UE

1.	The Non-3GPP UE sends a Non-3GPP message request to the Non-3GPP Message Gateway.

2.	The Non-3GPP Message Gateway translates the Non-3GPP message to MSGin5G message with delivery report requested and sends a MSGin5G message request to the MSGin5G Server as specified in clause 8.3.2 with the following clarifications:

a)	The MSGin5G message request includes Originating MSGin5G Service ID, Recipient MSGin5G Service ID and Message ID information elements in Table 8.3.2-1, and may include Delivery status required, Application ID, Payload, Priority type information elements from Table 8.3.2-1.

b)	Upon receiving the MSGin5G message request, the MSGin5G Server determines the recipient is a MSGin5G UE and the Non-3GPP UE is allowed to reply message to the MSGin5G UE.

3.	The MSGin5G Server forwards the MSGin5G message request to the target MSGin5G Client as specified in clause 8.3.3.

4.	If the delivery status report is request by the Non-3GPP UE, the MSGin5G Client in MSGin5G UE sends an MSGin5G message delivery report to the MSGin5G Server, the MSGin5G Server forwards the MSGin5G message delivery report to the Non-3GPP Message Gateway, the Non-3GPP Message Gateway translates the MSGin5G message delivery report to a Non-3GPP message delivery report and sends it to the Non-3GPP UE.