3GPP TSG-SA WG6 Meeting #44-e meeting	S6-211676

12th – 20th July 2021, Online	(revision of S6-21xxxx)

Source:	Huawei, Hisilicon

Title:	Service announcement alignment with TS 23.247

Spec:	3GPP TR 23.783 v1.4.0

Agenda item:	10.1

Document for:	Approval

Contact:	Cuili – Ge <gecuili@huawei.com>

1. Introduction

This pCR is proposed to update the MBS service announcement for alignment with TS 23.247.

2. Reason for Change

1) Due to the service announcement described in clause 6.11 of TS 23.247, the following information are also need for multicast MBS service announcement:

-	PLMN ID of the PLMN in which the service is delivered may be included when the MBS Session ID is Source Specific IP Multicast Address;

-	DNN and S-NSSAI of the PDU Session to indicate which PDU Session is associated with the MBS Session.

2) In order to well maintain the consistency of information flow table, the content in clause 6.13.1.2.1 is proposed to replaced by a reference to the clause 6.12.1.2.1.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.783 v1.4.0.

- * * * First Change * * * *

##### 6.12.1.2.1	Multicast MBS service announcement

Table 6.12.1.2.1-1 describes the information flow multicast MBS service announcement from the MC service server to the MC service client.

Table 6.12.1.2.1-1: Multicast MBS service announcement

| Information element | Status | Description |
| --- | --- | --- |
| MBS session ID | M | The identity of the multicast MBS session used for DL data delivery, it is either the TMGI or source specific IP multicast address |
| SDP information | M | SDP with media and application layer control signalling information applicable to groups that can use this multicast MBS session (e.g. codec, protocol id, FEC information) |
| Announcement acknowledgment | O | Indicate if an acknowledgement of the multicast MBS service announcement is required |
| Join status report indication | M | An indication that the when the status of UE joining the multicast MBS session is requested. |
| PLMN ID (NOTE) | O | PLMN ID of the PLMN in which the service is delivered. |
| DNN and S-NSSAI | O | DNN and S-NSSAI of the PDU Session to indicate which PDU Session is associated with the multicast MBS Session. |
| NOTE: It is present only when the MBS session ID is source specific IP multicast address. |  |  |

Editor’s note:	Whether the MC service group ID is needed for the multicast MBS service announcement.

- * * * Next Change * * * *

##### 6.13.1.2.1	Multicast MBS service announcement

Refer to the definition in clause 6.12.1.2.1	Multicast MBS service announcement.

service announcement