3GPP TSG-SA WG6 Meeting #45-bis-e	S6-212203

e-meeting, 11th – 19th October 2021	(revision of S6-21xxxx)

Source:	Huawei

Title:	Add solutions of performance and analytics exposure

Spec:	3GPP TR 23.700-99 v0.2.0

Agenda item:	10.4

Document for:	Approval

Contact:	jia xiaoqian<jiaxiaoqian@huawei.com>

1. Introduction

The key issue of network slice related performance and analytics exposure was introduced last meeting. To enable the third-party to adjust the network slice or to configure the information which associates a service/UE to a network slice, the third-party is allowed to monitor the performance data and analytics data of the network slice, service or application.  The network slice related performance data may be collected from multiple sources (e.g., OAM system or 5GC functions) to help the NSCE layer to perform data aggregation or analyses to generate the data required by the third-party. The solutions of performance data and analytics data exposed to the third-party needs to be studied.

2. Reason for Change

This contribution proposes a possible solution to address the key issue of peformance and analytics exposure.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-99 v0.2.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS)"

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[5]		3GPP TS 28.531: "Management and orchestration; Provisioning".

[6]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[7]	3GPP TS 22.261: "Service requirements for the 5G system".

[8]	3GPP TS 28.533: "Management and orchestration; Generic management services".

[9]	3GPP TR 22.835: " Study on enhanced access to and support of network slices".

[10]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS) ".

[11]	3GPP TS 28.535: "Management and orchestration; Management services for communication service assurance; Requirements".

[12]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[13]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[14]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[15]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[x]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[y]	3GPP TS 28.104: " Management and orchestration; Management Data Analytics".

[z]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[m]	3GPP TS 22.263: "Services and System Aspects; Service requirements for video, imaging and audio for professional applications (VIAPA)".

* * * Next Change * * * *

## 6.X	Solution Y: Network slice related performance and analytics exposure

### 6.X.1	Solution description

#### 6.X.1.1	General

This solution addresses the key issue 7 of performance and analytics exposure described in clause 5.7.

This solution provides a possible procedure to illustrate the network slice related performance and analytics exposure capability provided by network slice capability exposure server.

To enable the third-party to adjust the network slice or to configure the information which associates a service/UE to a network slice, the third-party is allowed to monitor the performance data and analytics data of the network slice, service or application. The network slice related performance data may be collected from multiple sources (e.g., OAM system or 5GC functions) to help the NSCE layer to perform data aggregation or analyses to generate the data required by the third-party. The solutions of performance data and analytics data exposed to the third-party needs to be studied.

#### 6.X.1.2	Network slice related performance and analytics exposure

Figure 6.X.1.2-1 illustrates the network slice related performance and analytics exposure process to address the key issue 7 of performance and analytics exposure described in clause 5.7.

Pre-conditions:

- The VAL client has subscribed to the capability of Network slice related performance and analytics exposure.

- The network slice enabler layer is capable to interact with NEF and OAM system.

Figure 6.X.1.2-1 Network slice related performance and analytics exposure process

1.	The VAL server sends a request to NSCE server to collect the desired service/application specific performance data and analytics data, the detailed content of the reported data depends on the type of the application and services. For example, the VAL may request the NSCE server to report the performance and analytics report of the service of the imaging for professional applications.

Note 1: This request may be triggered by a specific event or request from the VAL client to the VAL server. Besides, the VAL client can trigger the performance data and analytics data collection procedure if the performance issue are detected by the VAL client itself.

2.	On receiving the request from VAL server, NSCE server retrieves the performance data and analytics data of network slice from 5GS. For OAM system, the APIs defined in clause 11.3, TS 28.532[15] is utilized. For CN functions, the APIs of Nnwdaf_AnalyticsInfo service defined in clause 7.3, TS 23.288[x] can be utilized.

The data of QoS information defined in TS 23.501[3] and analytics data defined in TS 23.288[x], network slice instance related performance data defined in TS 28.552[12] and network analytics data in TS 28.104[y] exposed by OAM system may be acruqired. For example, when the performance and analytics report of the service of the imaging for a professional application is required, the imaging system latency as defined in TS 22.263[m] should be reported and analysed. In this case, the NSCE requests the OAM system to report the one-way/Round-trip packet delay of a specific S-NSSAI between PSA UPF and NG-RAN in clause 5.4, TS 28.552[12] and requests the NWDAF to report the User Data Congestion Analytics report defined in clause 6.8, TS 23.288[x].

3.	NSCE server sends a request to application layer to request the KQI/QoE data. For example, in the service of a vedio for a specific application, the QoE of Corruption duration metric or jitter duration defined in clause 16.2, TS 26.114[z] is request to report.

4.   VAL server sends the requested QoE data to NSCE server.

5.   NSCE server may send a request to NSCE client to collect the KQI data of services (e.g., the jitter duration of a vedio service), the network performance related data (e.g., the downlink/uplink RSRP of serving cell measured by 5G UE) and the end users information (e.g., 5G UE’s running time) if possible.

6.  NSCE client report the requested data collected by itself if possible, the information collected by the NSCE client depends on the third-parties’ implementations.

7.	NSCE server correlates and analyses the performance data of network slice instance, the analytics data of group of UEs and the KQI/QoE data to generate the performance data and analytics data report as required by VAL server.

8.  NSCE server sends the service/application related performance data and analytics response to VAL server. For example, when the performance and analytics report of the service of the imaging for a professional application is required, the report may acquire the calculeted imaging system latency of the application, and a rank of the latency performance.

Note 2: Both periodical or adhoc reporting are possible, the VAL layer can trigger a periodical reporting or a one-off reporting. The result of the reporting can be subscribed by the VAL layer, it can request both or any one of them.