3GPP TSG-SA WG6 Meeting #45-bis-e	S6-212204

e-meeting, 11th – 19th October 2021	(revision of S6-21xxxx)

Source:	Huawei

Title:	Add solutions of fault management

Spec:	3GPP TR 23.700-99 v0.2.0

Agenda item:	10.4

Document for:	Approval

Contact:	jia xiaoqian<jiaxiaoqian@huawei.com>

1. Introduction

The key issue of network slice fault management capability is introduced last meeting. The third-party is allowed to monitor the problems of a network. The NSCE layer should help the third-party to scanning, diagnosing and identifying problems within a network. The performance data and alarm data from multiple sources is helpful to characterize the quality of the network connection.

2. Reason for Change

This contribution proposes a possible procedures of network slice fault management capability to address the following issues:

-	Whether and how additional APIs dedicated to network slice fault management capability are required?

-	How to define the APIs to expose the fault report to verticals client/server

-	How to coordinate with fault supervision management services provided by O&M systems defined in SA5 to fill the gap in fault management.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-99 v0.2.0.

* * * First Change * * * *

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS)"

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[5]		3GPP TS 28.531: "Management and orchestration; Provisioning".

[6]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[7]	3GPP TS 22.261: "Service requirements for the 5G system".

[8]	3GPP TS 28.533: "Management and orchestration; Generic management services".

[9]	3GPP TR 22.835: " Study on enhanced access to and support of network slices".

[10]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS) ".

[11]	3GPP TS 28.535: "Management and orchestration; Management services for communication service assurance; Requirements".

[12]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[13]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[14]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[15]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[x]	3GPP TS 32.111-1: "Management and orchestration; Fault management, Part 1: 3G fault management requirements".

[y]	3GPP TR 22.867: "Technical Specification Group Services and System Aspects; Fault management, Study on 5G Smart Energy and Infrastructure".

* * * Next Change * * * *

## 6.X	Solution Y: Network slice fault management capability

### 6.X.1	Solution description

#### 6.X.1.1	General

This solution addresses the key issue 4 of network slice fault management described in clause 5.4.

This solution provides a possible procedure to illustrate the network slice fault management capability exposed by NSCE server.

To enable the VAL server to monitor the problems of a network. The NSCE layer should help the VAL server to scanning, diagnosing and identifying problems within a network. The performance data and alarm data from multiple sources is helpful to characterize the quality of the network connection.

#### 6.X.1.2	Network slice fault management capability exposure

Figure 6.X.1.2-1 illustrates the network slice fault management process to address the key issue 4 of network slice fault management described in clause 5.4.

Pre-conditions:

The VAL client has subscribed to the network slice fault management capability.

The network slice enabler layer is capable to interact with NEF and OAM system.

The VAL client has checked the status of application layer.

Figure 6.X.1.2-1 Network slice fault management process

1.	The VAL server sends a request to NSCE server to request for the fault diagnosis of the applications and networks when it detected a fault in the application layer, the services and the network slices. This request may be triggered by the errors of the applications detected by the VAL server itself, or the VAL server may periodically collect the fault diagnostic report subscribing to the network slice fault management capability. The fault information of applications may be included in this request. For example, the Service Availability Failure Events defined in clause 5.5.3, TR 22.867[y].

2.	On receiving the request from VAL server, NSCE server retrieves the performance data of network slice from 5GS, e.g., QoS information defined in TS 23.501[3] exposed by NEF, network slice instance related performance data defined in TS 28.552[12] exposed by OAM system.

3.	NSCE server retrieves the alarms of network slice instances from OAM system via the procedures defined in TS 28.545[10], and the alarms are defined in clause 4.1.1.1, TS 32.111-1[x], e.g., the fault of communication, environmental, equipment, processing error, QoS for device/resource/file/functionality/smallest.

4.	NSCE server may request the alarm information (e.g., the 5GS network is not work or the required performance is under the threshold which leads the service’s problem) collected by NSCE client if possible. The information collected by the NSCE client depends on the third-parties’ requirements and implementation.

5.	NSCE client report the requested fault information to NSCE server.

6.   NSCE server correlates the applications, services and the network resources of the specific network slice instances which supports the applications and services with the S-NSSAI, NSI ID, Application ID, and then diagnoses the causes of the fault of the applications or services by analysing the fault information from different sources. For example, the RAN function of the slice instance which is utilized to support the service of the smart grid application, for a certain time duration, the smart grid suffered the bad experience caused by Service Availability Failure Events, and in the the RAN function is detected continuously to report an alarm of environmental fault in the same time duration, then the environment fault may be the root cause of the Service Availability Failure and should be prioritized.

7.  NSCE server reports the fault diagnostic result to VAL server to indicate the causes of the applications or services. If there is no problems within the 5GS network, the VAL server may be requested to check the application layer itself again.

8.  If the NSCE server detects that the application/service error is caused by the 5GS, the VAL server may send the report to OAM system to indicate the server fault which causes the application/service failure by utilizing the NSCM-OAM interface.