3GPP TSG-SA WG6 Meeting #44	S6-211868

e-meeting, 12th – 20th July 2021	(revision of S6-21xxxx)

Source:	one2many

Title:	Pseudo-CR on Broadcast message delivery procedure

Spec:	3GPP TS 23.554 v1.1.0

Agenda item:	8.2

Document for:	Approval

Contact:	peter.sanders@everbridge.com

1. Introduction

The pCR introduces the procedure for delivery of the broadcast message.

2. Reason for Change

New text to resolve EN in clause 7.1

3. Conclusions

-

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554 v1.1.0

* * * First Change * * * *

## 7.1	General

Massive Internet of Things (MIoT) is one of key market segments of 5G. The typical IoT device communication is sending and receiving small data which can be delivered just in a message. The MSGin5G Service is designed and optimized for massive IoT device communication including thing-to-thing communication and person-to-thing communication.

The MSGin5G Service is a message enabler for applications. An Application Client in a UE utilizes MSGin5G Service to send a message to another UE, to multiple UEs or to the Application Server, or the Application Server utilizes the MSGin5G Service to send a message to a UE or to multiple UEs. All messages will be routed via the MSGin5G Server in the 5G system. The MSGin5G service flow is shown in figure X.1-1.

If the UE supports a legacy 3GPP message service (e.g. SMS, NIDD, or CB) and does not support the MSGin5G Service (i.e. UE has no MSGin5G Client), the message will be translated to the appropriate message delivery mechanism by the Legacy 3GPP Message Gateway. A UE that does not support any 3GPP message service can connect to the MSGin5G Service via Non-3GPP Message Gateway that facilitates the translation between the MSGin5G Service and non-3GPP message delivery mechanism. The connection between such UE and the gateway can be via 3GPP access or non 3GPP access (e.g. WLAN) and is out of scope of the present specification.

An Application Server resides outside the 3GPP domain and connects to the MSGin5G Server via a CAPIF-aware reference point.

The message communication models include:

-	Point-to-Point messaging: message that is originated at a UE (UE A) and terminated at another UE (UE B, a Legacy 3GPP UE or a Non-3GPP UE).

-	Application-to-Point messaging: message that is originated at an Application Sever and terminated at a UE.

-	Point-to-Application messaging: message that is originated at a UE and terminated at an Application Sever

-	Group Messaging: message that is originated at a UE or an Application Server and is terminated at a group of UEs (a group member can be of type UE A, Legacy 3GPP UE or Non-3GPP UE).

-	Broadcast Messaging: message that is originated at an Application Sever or a MSGin5G UE and broadcasted to all the UEs in a specific Broadcast Area within coverage of a cell or of multiple cells, existing broadcast function (e.g. CB) specified in 3GPP TS 23.041 [14] is reused in broadcast messaging of MSGin5G service.

-	Topic Messaging: message that is originated at an Application Sever or a UE and is delivered to all UEs and Application Server(s) that have subscribed to the topic.

Figure 7.1-1: The MSGin5G Service overview

* * * Next Change * * * *

### 8.7.X	Broadcast message

#### 8.7.X.1	General

A MSGin5G message may be delivered via broadcast to MSGin5G UEs or to non-MSGin5G UEs in a Broadcast Area.

Editor's Note: Delivery of the Broadcast message to MSGin5G UEs via the Message Gateway requires an architecture change.

#### 8.7.X.2	Broadcast message delivery procedure

Pre-condition:

Configuration has been applied in the Message Gateway that determines if the Payload is to be delivered to non-MSGin5G UEs or the MSGin5G message, including all its elements, is to be delivered to MSGin5G UEs.

Procedure:

Figure 8.7.X.2-1 shows the procedure for the broadcast message.

Figure 8.7.X.2-1: Message Delivery in Broadcast Area

- An MSGin5G message request is sent by an Application Server or a MSGin5G UE to the MSGin5G Server as specified in clause 8.3.2.

a)	The MSGin5G message includes the Originating MSGin5G Service ID, the Recipient Broadcast Area ID, the Message ID, the Payload and may include the Application ID and Delivery status required elements.

b)	The MSGin5G Server authenticates the message and verifies that the sender is authorized to intiate a broadcast message.

- The MSGin5G Server delivers the message to the CBCF in the 5G core network via the Message Gateway as specified in clause 8.3.3. The CBCF is the Recipient Broadcast Area end-point.

- The CBCF delivers the message to the MSGin5G UEs in the Broadcast Area or to the non-MSGin5G UEs in the Broadcast Area. This step is out of scope of the present specification.

- If the broadcast message is delivered to MSGin5G UEs the Message Gateway includes the MSGin5G message, including all elements, for broadcast.

- If the broadcast message is delivered to non-MSGin5G UEs the Message Gateway includes only the Payload and the Broadcast Area elements for broadcast.

If the Delivery status requested element is included in the MSGin5G broadcast message, a recipient MSGin5G UE sends the delivery report via a point-to-point message as specified in clause 8.7.1.

* * * End of Change * * * *