3GPP TSG-SA WG6 Meeting #45	S6-211883

e-meeting, 25th August – 3rd September 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0282 | rev |  | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Notification URL correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | at&t |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData3 |  |  |  |  |  | Date: |  |  | 2021-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A notification URL from the MCData notification server is not always needed. Thus it cannot be a mandatory IE in the Create notification channel response. When a UE requests the MCData notification server to provide notificaiton services it sends the Create notification channel request to the MCData notification server. In return, the MCData notification server provides two URLs (the Notification URL and the Callback URL) for the UE to use to obtain the notificaiton service, as described in table 7.13.3.1.33-1. The “Notification URL” is needed and mandatory for a PULL method notification service where the MCData client uses it to receive (i.e. PULL) the notification from the MCData notification server. In the PUSH method notification service, the “Notification URL” is optionally used by some implementations to start the PUSH notification (such as WebSockets) while is not needed for other implementation (such as OS based notificaiton) and thus cannot be mandatory. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the “Notification URL” IE in table 7.13.3.1.33-1 from Mandatory to Optional. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Will give wrong guidance to stage 3 development. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.13.3.1.33 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * First Change * * *

##### 7.13.3.1.33	Create notification channel response

Table 7.13.3.1.33-1 describes the information flow for the create notification channel response sent from the MCData notification server to the message notification client.

Table 7.13.3.1.33-1: Create notification channel response

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData client initiating the request |
| Validity duration | M | How long the notification channel will last (i.e. channel lifetime) as granted by the MCData notification server |
| Notification URL | O | The URL to receive the notification message if a Pull method is requested . For some PUSH method implementation (such as WebSockets) this URL is used to start the PUSH notification service from the MCData notification server |
| Callback URL | M | The URL used by the Message notification client to subscribe to MCData message store notifications |