3GPP TSG-SA WG6 Meeting #45-e meeting	S6-211897

25th Aug – 03rd Sep 2021, Online

Source:	Intel, Nokia, Nokia Shanghai Bell

Title:	FS_eEDGEAPP: New Key Issue on EES and MEC Platform alignment

Spec:	3GPP TR 23.700-98

Agenda item:	10.8

Document for:	Approval

Contact:	samar.shailendra@intel.com

# 1	Introduction

As specified in TS 23.558 (Rel-17), the relationship between EDGEAPP architecture and ETSI MEC architecture has been discussed. While the TS indicates that there are similarities between the two architectures, it does not provide the details how the two architectures can co-exist, and how an application can call the APIs specific to the two architectures. The requirements related to multivendor environment also have not been discussed in the TS document.

To address the alignment aspects between the two architectures, this pCR proposes to add new key issue to study how the two architectures can coexist and an application can call the APIs specific to the two platforms.

# 2	Discussion

As part of the alignment of the two architecture (EDGEAPP and ETSI MEC), it is expected that they can coexist and interoperate seamlessly in a single/multiple deployment environment. It is possible that in a deployment environment, there are more than one instances of EES and/or MEC Platform exist. Moreover, these instances can be a part of single EDN or different EDNs. Both EDGEAPP and ETSI MEC defines EDGE-9 and Mp3 interfaces to interconnect between different instances of EES and that of MEC Platforms respectively. But there is no interface exist to interconnect EES with MEC Platform. For the co-existence of EDGEAPP and ETSI MEC in a single/multiple deployment environment, it is essential to define an interface between EES and MEC Platform. Such an interface and the APIs are essential to address the scenarios such as (as documented in clause 6.5.10 of TS 23.558):

- Application Context Relocation between EES and MEC Platform

- Discovery of Target EAS or Target MEC Application

# 3		Proposal

It is proposed to modify the text of TR 23.700-98 as follows.

1st CHANGE

## 4.x	Key issue #x: Alignment of EES and MEC Platform

As described in Annex C of TS 23.558 (Rel-17), in a deployment the EES and MEC platform can be collocated and will also require the alignment between the EDGE-9 and Mp3 interfaces. This alignment of EES and MEC Platform is required to address the challenges such as

-	Application Context Relocation between EES and MEC Platform

-	Discovery of Target EAS or Target MEC Application

This KI addresses the open issues:

- Anlayze the potential overlap between existing EES and MEC platform APIs for functionalities identified in clause 6.5.10 of TS 23.558.

- Recommend how to aligns the APIs as identified in i).

- Identify and analyze additional functionalities/procedures to be supported between EES and MEC Platform.

- Recommend additional interface and/or APIs for any new functionalities identified in iii).

END OF CHANGES