3GPP TSG-SA WG6 Meeting #45	S6-211930

e-meeting, 25th August – 3rd September 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0285 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of deposit file indication from MCData FD request using HTTP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData3 |  |  |  |  |  | Date: |  |  | 2021-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As part of the MCData User profile configuration we have the below configurations which can be used to determine whether the MCData communication to be stored into the MCData message store account of the user : Store communication in Message Store, Store private communication in Message Store, Store group communication in Message Store Since these configuration paramters are already present we do not require per request deposit file indication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removes the Deposit file indication IE from the MCData FD request (using HTTP) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Will lead to confusion while developing stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

##### 7.5.2.1.5	MCData FD request (using HTTP)

Table 7.5.2.1.5-1 describes the information flow for the MCData FD request (in subclause 7.5.2.4.2) sent from the MCData client to the MCData server.

Table 7.5.2.1.5-1: MCData FD request (using HTTP) from MCData client to MCData server

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending file |
| Functional alias | O | The functional alias associated with MCData user sending the file. |
| MCData ID (see NOTE) | O | The identity of the MCData user receiving file |
| Functional alias (see NOTE) | O | The associated functional alias of the MCData user identity towards which the data is sent. |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition indication | O | Indicates whether file download completed reported is expected or not |
| Download indication | O | Indicates mandatory download |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |
| Content reference | M | URL reference to the content and file metadata information |
| Emergency indicator | O | Indicates that the data request is for MCData emergency communication |
| NOTE: Either the MCData ID or the functional alias must be present. |  |  |

* * * * * * * END	 CHANGE * * * * * * *