3GPP TSG-SA WG6 Meeting #45	S6- 211961

e-meeting, 25th August – 3rd September 2021	(revision of S6-211xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0079 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for TSC services procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2021-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TR 23.745 Solution #22 incorporates SEAL support for TSN and 5G native TSC services. It is applicable for other vertical applications as well. It is proposed to use that solution for enhancing TS 23.434 NRM function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allow NRM server to use 5G native TSC for resource management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NRM does not support 5G-native TSC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.2.3, 14.3.2.X (new), 14.3.2.Y (new), 14.3.2.Z (new), 14.3.2.A (new), 14.3.2.B (new), 14.3.2.C (new), 14.4.1, 14.3.X (new), 14.3.X.1 (new), 14.3.X.2 (new), 14.3.X.3 (new), 14.3.X.4 (new), 14.4.2.X (new), 14.4.2.Y (new), 14.4.2.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 14.2.2.3	On-network functional model for network resource management for 5G-native TSC

5G-native TSC refers to time sensitive communication service offered natively (i.e. without integration with a TSN system) by the 5GS for the UEs connected to the 5GS. The architecture for the 5G-native TSC is depicted in Figure 14.2.2.3-1. The SEAL NRM server acts as an AF towards the 5G Core Network and performs coordination of QoS flows to fulfill the end-to-end QoS requirements for the UEs involved in the TSC communication.

Upon request from a VAL server via the NRM-S reference point it configures the TSC end-to-end QoS flows in the 5GS. In line with other SEAL service enablers the SEAL NRM server provides a RESTful interface on the NRM-S reference point. The SEAL NRM server interacts with the 5GS PCF over the N5 reference point to configure the 5G QoS and TSCAI parameters in the 5GS.

Figure 14.2.2.3-1: On-network functional model for network resource management for 5G-native TSC

* * * Next Change * * * *

#### 14.3.2.X	TSC stream discovery request

Table 14.3.2.X-1 describes the information flow TSC stream discovery request from the VAL server to the NRM server.

Table 14.3.2.X-1: TSC stream discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| Stream specification | M | Stream specification includes MAC addresses of the source and destination DS-TT ports as defined in IEEE 802.1CB [IEEE8021CB]. |

#### 14.3.2.Y	TSC stream discovery response

Table 14.3.2.Y-1 describes the information flow TSC stream discovery response from the NRM server to the VAL server.

Table 14.3.2.Y-1: TSC stream discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the TSC stream discovery with the underlying network. |
| Stream specification | M | Stream specification includes MAC addresses of the source and destination DS-TT ports as defined in IEEE 802.1CB [IEEE8021CB]. |
| List of traffic specifications | M | The traffic classes supported by the DS-TTs and available end-to-end latency values and Priority Code Point (PCP) values |

#### 14.3.2.Z	TSC stream creation request

Table 14.3.2.Z-1 describes the information flow TSC stream creation request from the VAL server to the NRM server.

Table 14.3.2.Z-1: TSC stream creation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL Stream ID | M | It identifies the VAL stream. |
| Stream specification | M | Stream specification includes MAC addresses of the source and destination DS-TT ports as defined in IEEE 802.1CB [IEEE8021CB]. |
| Traffic specification | M | It includes Priority Code Point (PCP), MaxFrameInterval, MaxFrameSize, MaxIntervalFrames, MaxLatency, etc., as described in IEEE 802.1Qcc [IEEE8021Qcc] in clause 46.2. |

#### 14.3.2.A	TSC stream creation response

Table 14.3.2.A-1 describes the information flow TSC stream creation response from the NRM server to the VAL server.

Table 14.3.2.A-1: TSC stream creation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the TSC stream creation. |
| VAL Stream ID | M | It identifies the VAL stream. |

#### 14.3.2.B	TSC stream deletion request

Table 14.3.2.B-1 describes the information flow TSC stream deletion request from the VAL server to the NRM server.

Table 14.3.2.B-1: TSC stream deletion request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL Stream ID | M | It identifies the VAL stream. |

#### 14.3.2.C	TSC stream deletion response

Table 14.3.2.C-1 describes the information flow TSC stream deletion response from the NRM server to the VAL server.

Table 14.3.2.C-1: TSC stream deletion response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network TSC stream deletion. Even in case of failure, the stream is deemed unusable. |
| VAL Stream ID | M | It identifies the VAL stream. |

* * * Next Change * * * *

### 14.3.X 5G-native TSC resource management procedures

#### 14.3.X.1	General

The procedures related to the 5G-native TSC network resource management are described in the following subclauses.

#### 14.3.X.2	TSC stream discovery procedure

The TSC stream discovery procedure is used by the VAL server to discover the availability of resources for TSC communication for the given stream specification (i.e., between the target UEs) prior to creating the stream.

Pre-conditions:

1.	Each UE has an established Ethernet PDU session and DS-TTs are assigned to VLANs by the NRM server. The traffic classes are configured on each DS-TT.

2.	The NRM server has populated the management information base with the 5G bridge management and port management information. The latter is related to the Ethernet ports located in the DS-TTs including bridge delay per DS-TT Ethernet port pair per traffic class.

3.	NRM server has calculated the bridge delay for each port pair, i.e. composed of (ingress DS-TT Ethernet port, egress DS-TT Ethernet port) including the DS-TT residence time, packet delay budget (PDB) and propagation delay for both UL from sender UE and DL to receiver UE.

Figure 14.3.X.2-1: TSC stream discovery procedure

1.	The NRM server receives a request from a VAL server on NRM-S reference point to discover the connectivity and available QoS characteristics between DS-TTs identified by the stream specification.

2.	The NRM server validates the connectivity between the DS-TTs connected in the same VLAN based on the stored managed objects info base, identifies the traffic classes supported by the DS-TTs and calculates the end-to-end latency (including the residence time of the DS-TTs, PDBs, and propagation delay).

3.	NRM server responds to the VAL server with the stream specification and a list of traffic specifications with the available end-to-end latency and the traffic classes supported by the DS-TTs.

#### 14.3.X.3	TSC stream creation procedure

This procedure allows the VAL server to create a TSC stream. The TSC stream creation procedure enables the VAL server to establish TSC connectivity with the required QoS between the UEs connected to the 5GS after the stream discovery procedure.

Pre-conditions:

1.	Each UE has an established Ethernet PDU session for its DS-TT port MAC address.

2.	Connectivity between the DS-TTs has been validated by the TSC stream discovery procedure.

3.	NRM server maintains mapping from the traffic class to TSC QoS.

Figure 14.3.X.3-1: TSC stream creation procedure

- NRM server receives a TSC stream creation request from a VAL server to create a TSC stream identified by a VAL Stream ID, between DS-TT ports in the stream specification and for the traffic class in the traffic specification.

NOTE: It is left for stage 3 to decide whether to combine the QoS monitoring subscription request with the TSC stream creation request.

2.	NRM server calculates the schedule for the VAL Stream ID based on the information collected earlier from the 5GS via N5. It provides per-stream filtering and policing parameters according to IEEE 802.1Q [IEEE8021Q] used to derive the TSC QoS information and related flow information. NRM server also provides the forwarding rule according to IEEE 802.1Q [IEEE8021Q] used to identify the DS-TT MAC address of the corresponding PDU session. Based on the 5GS bridge delay information it determines the TSC QoS information and TSC Assistance information for the stream.

3.	NRM server triggers via N5 the AF request procedure as shown in 3GPP TS 23.502 [11] annex F.2 for the TSC stream for both UL QoS flow (sender UE to UPF/bridge) and DL QoS flow (UPF/bridge to receiver UE). The AF request includes the DS-TT port MAC address, TSC QoS information, TSC Assistance Information, flow bit rate, priority, Service Data Flow Filter containing flow description including Ethernet Packet Filters. The QoS flow will be assigned for the PDU session for the source MAC address for the UL direction and for the PDU session for the destination MAC address for the DL direction. The configuration sent over N5 also includes the gate control list (including AdminControlList, AdminBaseTime, AdminCycleTime and Tick Granularity) for the traffic specification. The gate control parameters are for the hold and forward buffering by the DS-TT for the respective TSC flow. This information is delivered to the DS-TT by the 5GS.

4.	NRM server sends TSC stream creation response to the VAL server with the result of TSC stream creation for the VAL Stream ID.

#### 14.3.X.4	TSC stream deletion procedure

This procedure allows the VAL server to delete a TSC stream.

Pre-conditions:

1.	The TSC stream is configured in the 5GS and the DS-TTs.

Figure 14.3.X.4-1: TSC stream deletion procedure

1.	NRM server receives a request from VAL server to delete a TSC stream for with a VAL Stream ID.

2.	NRM server identifies the MAC addresses of the DS-TTs involved in the stream based on the stored information for the VAL Stream ID.

3.	NRM server triggers via N5 the AF session modification procedure defined in 3GPP TS 23.502 [11] clause 4.15.6.6 for MAC addresses referred to by the VAL Stream ID. NRM server uses the procedure to delete both UL QoS flow (sender UE to UPF/bridge) and DL QoS flows (UPF/bridge to receiver UE) from the PDU sessions of the UEs referred to by the VAL Stream ID.

4.	NRM server sends TSC stream deletion response to the VAL server with the result of TSC stream deletion for the VAL Stream ID.

* * * Next Change * * * *

### 14.4.1	General

Table 14.4.1-1 illustrates the SEAL APIs for configuration management.

Table 14.4.1-1: List of SEAL APIs for network resource management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_NetworkResourceAdaptation | Reserve_Network_Resource | VAL server | Request /Response |
|  | Request_Unicast_Resource | VAL server | Request /Response |
|  | Update_Unicast_Resource | VAL server | Request /Response |
|  | Request_Multicast_Resource | VAL server | Request /Response |
|  | Notify_UP_Delivery_Mode | VAL server | Subscribe/Notify |
|  | TSC_Stream_Discovery | VAL server | Request /Response |
|  | TSC_Stream_Creation | VAL server | Request /Response |
|  | TSC_Stream_Deletion | VAL server | Request /Response |

* * * Next Change * * * *

#### 14.4.2.X	TSC_Stream_Discovery

API operation name: TSC_Stream_Discovery

Description: Requesting to discover the connectivity and available QoS characteristics between DS-TTs.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.X

Outputs: See subclause 14.3.2.Y

See subclause 14.3.X.2 for the details of usage of this API operation.

* * * Next Change * * * *

#### 14.4.2.Y	TSC_Stream_Creation

API operation name: TSC_Stream_Creation

Description: Requesting the NRM to create a TSC stream.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.Z

Outputs: See subclause 14.3.2.A

See subclause 14.3.X.3 for the details of usage of this API operation.

* * * Next Change * * * *

#### 14.4.2.Z	TSC_Stream_Deletion

API operation name: TSC_Stream_Deletion

Description: Requesting the NRM to delete the TSC stream.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.B

Outputs: See subclause 14.3.2.C

See subclause 14.3.X.4 for the details of usage of this API operation.

* * * End of Change * * * *