3GPP TSG-SA WG6 Meeting #45	S6-211976

e-meeting, 25th August – 3rd September 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0038 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Failure of EEC executed ACR via T-EES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2021-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Following EN in clause 8.8.2.6: Editor's note: When the procedure fails, the use of other mechanisms to alleviate this failure is FFS. In case the EEC executed ACR via T-EES fails, the EEC has multiple options; for e.g. it can: - reinitiate the procedure with the same EES or a different EES obtained during service provisioning; - choose to continue availing the services from the S-EAS, if possible; - it can avail services from the cloud application server. The exact action taken by the EEC as a result of this procedure failing should be implementation specific, and therefore the EN is removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The EN is replaced with the following NOTE: NOTE x: If this procedure fails, the EEC can reinitiate the procedure with the same EES or a different EES obtained in step 3. The EEC can also choose to continue availing the services from the S-EAS, if possible. The actions taken by the EEC as a result of this procedure failing is out of scope of current release. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary EN in the TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.8.2.6	EEC executed ACR via T-EES

Figure 8.8.2.6-1 illustrates the procedure for the EEC to execute the ACR via T-EES.

Pre-condition:

1.	The EEC has the S-EAS information that serves the AC.

Figure 8.8.2.6-1: EEC executed ACR via T-EES

Phase I: ACR Detection

1.	The EEC detects that ACR may be required as described in clause 8.8.1. The EEC may detect that ACR may be required for an expected or predicted UE location in the future as described in clause 8.8.1.

Phase II: ACR Decision

2.	The EEC decides to proceed with required procedures for ACR.

NOTE 1:	If supported, the AC can be involved in the decision. It is out of scope of the present document how the AC is involved.

Phase III:	ACR Execution

3.	The EEC determines the T-EES by using the provisioned information or performing service provisioning procedure per clause 8.3. When in step 1 the ACR for service continuity planning is triggered, then the Connectivity information and UE Location used in the service provisioning procedure contain the expected Connectivity information and expected UE Location. If the UE is within the service area of the T-EES, upon selecting the T-EES the UE may need to establish a new PDU connection to the target EDN. The EEC performs EAS Discovery with the T-EES per clause 8.5.2.

4.	The EEC performs ACR launching procedure (as described in clause 8.8.3.4) to the T-EES with the ACR action indicating ACR initiation and the corresponding ACR initiation data (with the need to notify the EAS). The T-EES may apply the AF traffic influence with the N6 routing information of the T-EAS in the 3GPP Core Network (if applicable). Then the T-EES sends the ACR Notify message to the T-EAS. The EEC also subscribes to receive ACR information notifications for ACR complete events from the T-EES, as described in clause 8.8.3.5.2.

5.	The T-EAS initiates ACT between the S-EAS and the T-EAS. This process is out of scope of the present specification.

When in step 1 the ACR has been triggered for service continuity planning, if the UE does not move to the predicted location the EEC does not connect to T-EES, the AC does not connect to the T-EAS. Steps 6 and 7 are skipped.

NOTE 2:	The S-EAS or T-EAS can further decide to terminate the ACR, and the T-EAS can discard the application context based on information received from EEL and/or other methods (e.g. monitoring the location of the UE). It is up to the implementation of the S-EAS and T-EAS whether and how to make such a decision.

NOTE 3:	When in step 1 the ACR has been triggered for service continuity planning, steps 6 and 7 would only be performed after the UE moves to the expected location.

Phase IV:	Post-ACR clean up

6.	The T-EAS sends the ACR Complete message to the T-EES to confirm that the ACR has completed.

7.	The T-EES sends the ACR information notification message to the EEC as described in clause 8.8.3.5.3.

If the procedure fails after step 4, it will be terminated with an appropriate cause in the ACR Response message to the EEC in step 7. The EEC may then proceed attempting to obtain services from the T-EAS discovered in step 3 without service continuity support. Alternatively, the EEC may resume the present procedure starting with step 3 and selecting a different T-EES.

NOTE 4:	The support of ACR between EDNs operated by different ECSPs is dependent on business agreement between the ECSPs.

NOTE x:	If this procedure fails, the EEC can reinitiate the procedure with the same EES or a different EES obtained in step 3. The EEC can also choose to continue availing the services from the S-EAS, if possible. The actions taken by the EEC as a result of this procedure failing is out of scope of current release.