3GPP TSG-SA WG6 Meeting #45	S6-212029

e-meeting, 25th August – 3rd September 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 0012 | rev | - | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to add the missing API operation for C2 operation mode configuration complete notification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP |  |  |  |  |  | Date: |  |  | 2021-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clause 7.4.2.1 step 4, a notification is sent from UAE server to UASS when the UAE server completes the C2 operation mode configurations with the UAS UEs. Currently, the API operation corresponding to the step 4 is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing API operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 implementation may be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1, 8.2.2.5 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.2.1	General

Table 8.2.1-1 illustrates the UAE server APIs.

Table 8.2.1-1: List of UAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| UAE_C2OperationModeManagement API | Configure_C2OperationMode |  | UAS application specific server | Request/ Response |
|  | Notify_SelectedC2Mode (NOTE) |  | UAS application specific server | Subscribe/notify |
|  | Notify_C2ModeSwitching (NOTE) |  | UAS application specific server | Subscribe/notify |
|  | Notify_C2OperationModeConfigComplete (NOTE) |  | UAS application specific server | Subscribe/notify |
| NOTE: The subscribe operation for Notify_C2OperationModeConfigComplete, Notify_SelectedC2Mode and Notify_C2ModeSwitching is part of Configure_C2OperationMode |  |  |  |  |

* * * Next Change * * * *

#### 8.2.2.5	Notify_C2OperationModeConfigComplete

API operation name: Notify_C2OperationModeConfigComplete

Description: Notification about the C2 operation mode configuration completion by UAE server.

Known Consumers: UAS application specific server.

Inputs: Refer clause 7.4.3.3.

Outputs: None.

See clause 7.4.2.1 for the details of usage of this API operation.