3GPP TSG-SA WG6 Meeting #45	S6-212033

e-meeting, 25th August – 3rd September 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 0016 | rev | - | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to add the missing UAE layer registration procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP |  |  |  |  |  | Date: |  |  | 2021-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clause 7.2.2.1, there is a pre-condition that “The UAE clients of UAVs have connected to the UAE server” and in clause 7.3.2.1 there is a pre-condition that “Both UAV-C and UAV have successfully connected to the UAE server”. To enable successful connection between UAV/UAV-C and UAE server, a UAE layer registration process is required. Without such a process specified for UAE server, these pre-conditions cannot be established. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing procedure regarding UAE layer registration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Stage 3 implementation will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1a (new), 7.1a.1 (new), 7.1a.2 (new), 7.1a.2.1 (new), 7.1a.2.1.1 (new), 7.1a.2.1.2 (new), 7.1a.2.2 (new), 7.1a.2.2.1 (new), 7.1a.2.2.2 (new), 7.1a.3 (new), 7.1a.3.1 (new), 7.1a.3.2 (new), 7.1a.3.3 (new), 7.1a.3.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 7.1a	UAE layer registration

### 7.1a.1	General

The UAE capabilities provide support for registering UAV/UAV-C at the UAE server. The UAE server uses the registration information to distribute UAE layer messages or UAS application specific layer messages to the appropriate UAS UEs.

### 7.1a.2	Procedures

### 7.1a.2.1	UAS UE registration

#### 7.1a.2.1.1	General

This clause describes the procedures for UAS UE (UAV/UAV-C) to register with the UAE server.

#### 7.1a.2.1.2	Procedure

Pre-conditions:

-	The UAE client has discovered the UAE server and is aware of the address of the UAE server (e.g. FDQN).

-	The UAV/UAV-C has already been assigned with the UAV ID.

Figure 7.1a.2.1.1-1: Procedure for registering the UAE client at the UAE server

1.	The UAE client sends a registration request to the UAE server.

2.	The VAE server sends an acknowledgement to the VAE client.

### 7.1a.2.2	UAS UE deregistration

#### 7.1a.2.2.1	General

This clause describes the procedures for UAS UE (UAV/UAV-C) to deregister at the UAE server.

#### 7.1a.2.2.2	Procedure

Pre-condition:

-	The UAE client has already registered with the UAE server as described in subclause 7.1a.2.1.

Figure 7.1a.2.2.2-1: Procedure for deregistering the UAE client at the UAE server

1.	The UAE client sends a deregistration request to the UAE server.

2.	The UAE server sends a deregistration response to the UAE client.

### 7.1a.3	Information flows

#### 7.1a.3.1	Registration request

Table 7.1a.3.1-1 describes the information flow for a UAE client to register with the UAE server.

Table 7.1a.3.1-1: Registration request

| Information element | Status | Description |
| --- | --- | --- |
| UAV ID | M | The identifier of the UAV/UAV-C (e.g. 3GPP UE ID or CAA level UAV ID) which initiates the registration request |
| UAS UE information | O | UAS UE information like IP address, etc. |

#### 7.1a.3.2	Registration response

Table 7.1a.3.2-1 describes the information flow for UAE server to respond for registration request from the UAE client.

Table 7.1a.3.2-1: Registration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the UAE server in response to registration request indicating success or failure |

#### 7.1a.3.3	Deregistration request

Table 7.1a.3.3-1 describes the information flow for a UAE client to deregister at the UAE server.

Table 7.1a.3.3-1: Deregistration request

| Information element | Status | Description |
| --- | --- | --- |
| UAV ID | M | The identifier of the UAV/UAV-C (e.g. 3GPP UE ID or CAA level UAV ID) which initiates the deregistration request |

#### 7.1a.3.4	Deregistration response

Table 7.1a.3.4-1 describes the information flow for UAE server to respond for deregistration request from the UAE client.

Table 7.1a.3.4-1: Deregistration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the UAE server in response to the deregistration request |