3GPP TSG-SA WG6 Meeting #45	S6-212050

e-meeting, 25th August – 3rd September 2021	(revision of S6-211942)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0029 | rev | 1 | Current version: | <17.0.0> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on ECS configuration information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | <2021-08-19> |
| Category: | F |  |  |  |  |  | Release: |  |  | <Rel-17> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR addresses the following issue on ECS configuration information. -It is not clarfied how EEC and UE can decide which PDU Session can be used or established to perform service provisioning, especially for a case where a combination of DNN and/or S-NSSAI is dedicated to the edge computing service of ECSP that provides the ECS. In that case, that information needs to be configured along with the ECS address. Even if the use of URSP can handle the issue, but it cannot cover all cases (e.g. EPC, UE not supporting URSP). |  |  |  |  |  |  |  |  |
| Summary of change: |  | A combination of DNN and/or S-NSSAI can be included in the ECS configuration information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The followings remain unclear: how EEC and UE can decide which PDU Session can be used or established for service provisioning |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.3.2	ECS Discovery

8.3.2.1	General

ECS configuration information consists of one or more URLs and/or IP Address(es) of ECS(s), and optionally the corresponding ECS Provider Identifier. Table 8.3.2.1-1 describes the information elements of ECS configuration information. ECS configuration information can be

-	pre-configured with the EEC;

-	configured by an edge-aware AC;

-	configured by the user;

-	provisioned by MNO through 5GC procedure if the UE has the capability to deliver the ECS configuration information to the EEC on the UE (see 3GPP TS 23.548 [20], clause 6.5.2); or

-	derived from HPLMN identifier for non-roaming scenario or from VPLMN identifier for roaming scenario.

NOTE:	How the ECS configuration information is configured to the EEC by the AC, user, or pre-configuration is out of scope of the present specification.

It may be possible to provide the ECS configuration information to the EEC from the 5GC if the UE has the capability to deliver the ECS configuration information to the EEC on the UE.

If the ECS configuration information is provided by 5GC and available at the EEC, the EEC shall use the information for the initial provisioning request. Otherwise, the EEC shall use pre-configured ECS address for the initial provisioning if ECS configuration information is preconfigured with the EEC.

If a combination of DNN (APN) and/or S-NSSAI is dedicated to edge computing services of an ECSP that provides ECS, the dedicated DNN (APN) and/or S-NSSAI is included in the ECS configuration information.

Table 8.3.2.1-1: ECS configuration information

| Information element | Status | Description |
| --- | --- | --- |
| ECS address | M | One or more URLs and/or IP Address(es) of ECS(s) |
| ECS Provider Identifier | O | The identifier of the ECSP that provides the ECS. |
| DNN/APN | O | Data Network Name/Access Point Name for ECS |
| S-NSSAI | O | Network Slice information for ECS |