3GPP TSG-SA WG6 Meeting #45-e	S6-211882

e-meeting, 25th August – 3rd September 2021	(revision of S6-212062 was S6-211519)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0293 | rev | 2 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications for Location management PLUS NEW enhancements for MC service UE label |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT |  |  |  |  |  | Date: |  |  | 2021-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. Enhancements for MC service UE label: The MC service UE label was introduced at SA6#42-e with S6-210581. Enhancements, proposed at SA6#42-bis-e with S6-211048 were postponed. This CR adds the MC service UE label to information flows and procedures, handling location information, offering a complete feature within Rel-17. - The location management server shall be enabled to store the MC service UE label. - The MC service server needs NOT to receive the MC service UE label (as it has no use for it). - Aditional Error correction 10.9.2.2 added that the location information report can also be sent from the location management server to the location management client or to the MC service server. 10.9.3.6.2 align changes with 10.9.2.2, so that a location information report is sent from the LMS to the LMC or to the MC service server. Additionally the “MC service UE label” is added. - 10.9.2.5 Split the information flow, into one for MC service server and one for the location management client, as the MC service server does not provide a “MC service ID” - 10.9.2.10 Change information flow to “singular” to provide one location information at a time, when multiple LMCs report their location when sharing the same activated functional alias. - 10.9.2.13 Correct typo from LM“C“ to LM“S“ and remove „identity“ of MC service server (as not needed) - 10.9.3.5 Removal of authentication check for MC service server, as MC service server is implicitly trusted (in one MC system). Adding a NOTE to make clear it is removed on purpose. - 10.9.3.6.2 This procedure mentions “location information report”, but from LMS to LMC or LMS to MC service server it should be “location information notification”. Replacement done also in Figure 10.9.3.6.2-1. Removal of authentication check for MC service server, as implicitly trusted. Adding a NOTE to make clear it is removed on purpose. Changed step 4 to send one single location information at a time (to synchronize it with S6-211498 (SA6#43-e) - 10.9.3.8.1 Align (functional alias) procedure to match information flow changes made in 10.9.2.10 - 10.9.3.8.2 As above, align (functional alias) procedure with information flow 10.9.2.10 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Missing parts for the MC service UE label are provided. Changes are made in the required sub-clauses as described in the “Reason for change”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Possible misinterpretation in the specification. MC service UE label is not usable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.2.2, 10.9.2.5, 10.9.2.7, 10.9.2.10, 10.9.2.13, 10.9.2.14, 10.9.3.1, 10.9.3.2, 10.9.3.5, 10.9.3.6.1, 10.9.3.6.2, 10.9.3.8.1, 10.9.3.8.2, 10.9.3.9.2.1, 10.9.3.9.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 0 agreed at SA6#44-e. |  |  |  |  |  |  |  |  |

* * First Change * * *

#### 10.9.2.2	Location information report

Table 10.9.2.2-1 describes the information flow from the location management client to the location management server for the location information reporting.

Table 10.9.2.2-1: Location information report (LMC – LMS)

| Information element | Status | Description |
| --- | --- | --- |
| Set of MC service IDs | M | Set of identities of the reporting MC service user on the MC service UE (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| MC service ID (see NOTE 4) | O | Identity of the requesting MC service user. |
| Functional alias(es) (see NOTE 1) | O | Functional alias that corresponds to the reporting MC service ID. |
| MC service UE label | O | Generic name of the reporting MC service UE |
| Triggering event (see NOTE 3) | M | Identity of the event that triggered the sending of the report |
| Location Information (see NOTE 2) | M | Location information of the individual MC service user |
| NOTE 1: Each functional alias corresponds to an individual MC service ID. NOTE 2: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. NOTE 3: An on-demand request may be the triggering event. NOTE 4: In case of an on-demand request of an MC service user the MC service ID shall be provided. In case of an MC service server request or an event-triggered report, no MC service ID is provided. |  |  |

Table 10.9.2.2-2 describes the information flow from the location management server to the location management client for location information reporting.

Table 10.9.2.2-2: Location information report (LMS – LMC)

| Information element | Status | Description |
| --- | --- | --- |
| Set of MC service IDs | M | Set of identities of the reporting MC service user on the MC service UE (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| MC service ID | M | Identity of the requesting MC service user. |
| Functional alias(es) (see NOTE 1) | O | Functional alias that corresponds to the reporting MC service ID. |
| MC service UE label | O | Generic name of the reporting MC service UE |
| Triggering event (see NOTE 3) | M | Identity of the event that triggered the sending of the report |
| Location Information (see NOTE 2) | M | Location information of the individual MC service user |
| NOTE 1: Each functional alias corresponds to an individual MC service ID. NOTE 2: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. NOTE 3: An on-demand request may be the triggering event. |  |  |

Table 10.9.2.2-3 describes the information flow from the location management server to the MC service server for location information reporting.

Table 10.9.2.2-3: Location information report (LMS – MC service server)

| Information element | Status | Description |
| --- | --- | --- |
| Set of MC service IDs | M | Set of identities of the reporting MC service user on the MC service UE (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| Functional alias(es) (see NOTE 1) | O | Functional alias that corresponds to the MC service ID. |
| Triggering event (see NOTE 3) | M | Identity of the event that triggered the sending of the report |
| Location Information (see NOTE 2) | M | Location information of the individual MC service user |
| NOTE 1: Each functional alias corresponds to an individual MC service ID. NOTE 2: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. NOTE 3: An on-demand request may be the triggering event. |  |  |

* * Next Change * * *

#### 10.9.2.5	Location information subscription request

Table 10.9.2.5-1 describes the information flow from the MC service server to the location management server for location information subscription request.

Table 10.9.2.5-1: Location information subscription request (MC service server – LMS)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID list | M | List of MC service users whose location information is requested |
| Time between consecutive reports | M (see NOTE) | Indicates the interval time between consecutive reports. The provided time is to be used for all MC service IDs provided in the MC service ID list. |
| NOTE: If the interval time has a value of zero then the location management server will send the Location information notification immediately the location information report is received from the MC service user in the MC service ID list. |  |  |

Table 10.9.2.5-2 describes the information flow from the location management client to the location management server for location information subscription request.

Table 10.9.2.5-2: Location information subscription request (LMC – LMS)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the requesting MC service user |
| MC service ID list | M | List of MC service users whose location information is requested |
| Time between consecutive reports | M (see NOTE) | Indicates the interval time between consecutive reports. The provided time is to be used for all MC service IDs provided in the MC service ID list. |
| NOTE: If the interval time has a value of zero then the location management server will send the Location information notification immediately the location information report is received from the MC service user in the MC service ID list. |  |  |

* * Next Change * * *

#### 10.9.2.7	Location information notification

Table 10.9.2.7-1 and Table 10.9.2.7-2 describe the information flows from the location management server to the MC service server and from the location management server to the location management client.

Table 10.9.2.7-1: Location information notification (LMS to MC service server)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service user whose location information is being notified |
| Triggering event | M | Identity of the event that triggered the sending of the notification |
| Location Information (see NOTE) | M | Location information |
| NOTE: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. |  |  |

Table 10.9.2.7-2: Location information notification (LMS to LMC)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service user whose location information is being notified |
| MC service ID | M | Identity of the MC service user subscribed to location information |
| Triggering event | M | Identity of the event that triggered the sending of the notification |
| Location Information (see NOTE) | M | Location information |
| MC service UE label | O | Generic name of the reporting MC service UE |
| NOTE: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. |  |  |

* * Next Change * * *

#### 10.9.2.10	Location report response

Table 10.9.2.10-1 describes the information flow from the location management server to the requesting location management client for the location information reporting when using functional alias..

Table 10.9.2.10-1: Location report response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the reporting MC service user whose location information is requested |
| MC service ID | M | Identity of the MC service user who requested the location information based on a provided functional alias |
| Functional alias | O | Functional alias that corresponds to the MC service ID of the reporting MC service user |
| Triggering event | M | Identity of the event that triggered the sending of the report |
| Location Information (see NOTE) | M | Location information for the corresponding functional alias and its MC service ID |
| MC service UE label | O | Generic name of the reporting MC service UE |
| NOTE: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. |  |  |

* * Next Change * * *

#### 10.9.2.13	Location information history request

The location management client stored location information, while not reporting location information to the location management server, and subsequently the reporting may start following the reestablishment of a communication link between the location management client and the location management server. Either all or a subset of the stored location information may be requested prior to the location information history reporting.

Table 10.9.2.13-1 describes the information flow from the location management client to the location management server for the location information history request of stored location information.

Table 10.9.2.13-1: Location information history request (LMC – LMS)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service user from whom reports are requested |
| MC service ID | M | Identity of the MC service user who requests location information |
| Number of stored reports (see NOTE) | O | Indicates the number of requested reports |
| Start time (see NOTE) | O | Indicates to send reports having this time of measurement and newer |
| End time (see NOTE) | O | Indicates to send reports having this time of measurement and older |
| Triggered event list (see NOTE) | O | Identifies the criteria when the location management client generated location information, while not reporting location information |
| Minimum time between consecutive reports | O | Defaults to 0 if absent |
| NOTE: If none of these information elements is present, all stored location information shall be reported. Information elements may combined to request a subset of the available location information. |  |  |

Table 10.9.2.13-2 describes the information flow from location management server to the location management client for the location information history request of stored location information.

Table 10.9.2.13-2: Location information history request (LMS – LMC)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service user from whom reports are requested |
| Number of stored reports (see NOTE) | O | Indicates the number of requested reports |
| Start time (see NOTE) | O | Indicates to send reports having this time of measurement and newer |
| End time (see NOTE) | O | Indicates to send reports having this time of measurement and older |
| Triggered event list (see NOTE) | O | Identifies the criteria when the location management client generated location information, while not reporting location information |
| Minimum time between consecutive reports | O | Defaults to 0 if absent |
| NOTE: If none of these information elements is present, all stored location information shall be reported. Information elements may combined to request a subset of the available location information. |  |  |

Table 10.9.2.13-3 describes the information flow from the MC service server to the location management server for the location information history request of stored location information.

Table 10.9.2.13-3: Location information history request (MC service server – LMS)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service user from whom reports are requested |
| Number of stored reports (see NOTE) | O | Indicates the number of requested reports |
| Start time (see NOTE) | O | Indicates to send reports having this time of measurement and newer |
| End time (see NOTE) | O | Indicates to send reports having this time of measurement and older |
| Triggered event list (see NOTE) | O | Identifies the criteria when the location management client generated location information, while not reporting location information |
| Minimum time between consecutive reports | O | Defaults to 0 if absent |
| NOTE: If none of these information elements is present, all stored location information shall be reported. Information elements may combined to request a subset of the available location information. |  |  |

* * Next Change * * *

#### 10.9.2.14	Location information history report

The location management client stored location information, while not reporting location information to the location management server, and subsequently the reporting may start following the reestablishment of a communication link between the location management client and the location management server.

Table 10.9.2.14-1 describes the information flow from the location management client to the location management server for the location information history reporting of stored location information.

Table 10.9.2.14-1: Location information history report (LMC – LMS)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID list | M | List of identities of the reporting MC service user (e.g. MCPTT ID, MCData ID, MCVideo ID) |
| Triggered event | M | Identifies the criterion when the location management client generated location information, while not reporting location information |
| Location Information (see NOTE 1) | M | Location information |
| MC service UE label | O | Generic name of the reporting MC service UE |
| History report (see NOTE 2) | O | Location information history report indicator |
| NOTE 1: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. NOTE 2: Only present, if triggering criteria in emergency cases or triggering criteria in non-emergency cases used. |  |  |

Table 10.9.2.14-2 describes the information flow from the location management server to the location management client for the location information history reporting of stored location information.

Table 10.9.2.14-2: Location information history report (LMS – LMC)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the reporting MC service user |
| MC service ID | M | Identity of the MC service user, who has requested location information |
| Triggered event | M | Identifies the criterion when the location management client generated location information, while not reporting location information |
| Location Information (see NOTE 1) | M | Location information |
| MC service UE label | O | Generic name of the reporting MC service UE |
| History report (see NOTE 2) | O | Location information history report indicator |
| NOTE 1: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. NOTE 2: Only present, if triggering criteria in emergency cases or triggering criteria in non-emergency cases used. |  |  |

Table 10.9.2.14-3 describes the information flow from the location management server to the MC service server for the location information history reporting of stored location information.

Table 10.9.2.14-3: Location information history report (LMS - MC service server)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the reporting MC service user |
| Triggered event | M | Identifies the criterion when the location management client generated location information, while not reporting location information |
| Location Information (see NOTE 1) | M | Location information |
| History report (see NOTE 2) | O | Location information history report indicator |
| NOTE 1: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. NOTE 2: Only present, if triggering criteria in emergency cases or triggering criteria in non-emergency cases used. |  |  |

* * Next Change * * *

### 10.9.3	Procedure

#### 10.9.3.1	Event-triggered location reporting procedure

NOTE 1: This procedure is valid for single MC system operation only.

The location management server provides location reporting configuration to the location management clients, indicating what information the location management server expects and what events will trigger the sending of this information to the location management server. The decision to report location information can be triggered at the location management client by different conditions. The conditions could include, for example, the reception of the location reporting configuration, initial registration, distance travelled, elapsed time, cell change, MBMS SAI change, MBMS session change, leaving a specific MBMS bearer service area, tracking area change, PLMN change, call initiation, or other types of events such as emergency alert, emergency call or imminent peril calls. The location report can include information described as time of measurement, accuracy, longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs.

Figure 10.9.3.1-1: Event-triggered location reporting procedure

1.	The location management server sends location reporting configuration message to the location management client(s) containing the initial configuration (or a subsequent update) for reporting the location of the MC service UE. This message can be sent over a unicast bearer to a specific location management client or as a group message over an MBMS bearer to update the location reporting configuration for multiple location management clients at the same time.

NOTE 2:	The location reporting configuration information can be made part of the user profile, in which case the sending of the message is not necessary.

NOTE 3:	Different location management clients may be given different location reporting criteria.

2.	A location reporting event occurs, triggering step 3.

3.	The location management client sends a location information report to the location management server, containing location information identified by the location management server and available to the location management client.

4.	Upon receiving the report, the location management server updates location of the reporting location management client. If the location management server does not have location information of the reporting location management client before, then just stores the reporting location information for that location management client. If the location information report contains an optional MC service UE label, the location management server stores it together with the location information for the reporting location management client.

* * Next Change * * *

#### 10.9.3.2	On-demand location reporting procedure

NOTE: This procedure is valid for single MC system operation only.

The location management server can request MC service UE location information at any time by sending a location information request to the location management client, which may trigger location management client to immediately send the location report.

Figure 10.9.3.2-1: On-demand location information reporting procedure

1.	Based on configurations such as periodical location information timer, or location information request from other entities (e.g., another location management client, MC service server), location management server initiates the immediate request of location information to be sent from the location management client.

2.	The location management server sends a location information request to the location management client.

3.	MC service user is notified and asked for permission to share location information. MC service user can accept or deny the request.

4.	The location management client immediately responds to the location management server with a report containing location information identified by the location management server and available to the location management client.

5.	Upon receiving the report, the location management server updates location of the reporting location management client. If the location management server does not have location information of the reporting location management client, the location management server just stores the reporting location information for that location management client. If the location information report contains an optional MC service UE label, the location management server stores it together with the location information for the reporting location management client.

* * Next Change * * *

#### 10.9.3.5	Location information subscription procedure

NOTE 1: This procedure is valid for single MC system operation only.

Figure 10.9.3.5-1 illustrates the high level procedure of location information subscription request. The same procedure can be applied for location management client and other entities that would like to subscribe to MC service user location information.

Figure 10.9.3.5-1: Location information subscription request procedure

1.	MC service server or location management client sends a location information subscription request to the location management server to subscribe location information of one or more MC service users.

2.	The location management server checks if the location management client is authorized to initiate the location information subscription request.

NOTE 2: As the MC service server is implicitly trusted, the Location management server needs not to check the authorization for the MC service server.

3.	The location management server replies with a location information subscription response indicating the subscription status.

* * Next Change * * *

#### 10.9.3.6	Usage of location information procedure

##### 10.9.3.6.1	Event-trigger location information notification procedure

NOTE 1: This procedure is valid for single MC system operation only.

Figure 10.9.3.6.1-1 illustrates the high level procedure of event-trigger usage of location information.  The same procedure can be applied for location management client and other entities that would like to subscribe to location information of MC service user.

Pre-condition:

1.	The MC service server or location management client has subscribed to the location information of a target MC service user.

Figure 10.9.3.6.1-1: Event-trigger usage of location information procedure

1.	Based on configuration, e.g., subscription, periodical location information timer, the location management server is triggered to report the latest user location information of the target MC service user to the subscribed MC service server or subscribed location management client.

2.	The location management server sends the location information notification including the latest location information of one MC service user to the MC service server or to the location management client. The latest location information is derived from the location report procedure as described in clause 10.9.3.1, or from PLMN operator (e.g. LCS network). If the optional MC service UE label is present, the location management server sends it, as part of the location information notification, to the MC service user.

NOTE 2: The MC service server does not receive the MC service UE label.

* * Next Change * * *

##### 10.9.3.6.2	On-demand usage of location information procedure

NOTE 1: This procedure is valid for single MC system operation only.

The MC service server or the location management client can request location information at any time by sending a location information request to the location management server, which may trigger location management server to send the location information report immediately.

Figure 10.9.3.6.2-1 illustrates the high level procedure of on-demand usage of location information. The same procedure can be applied for other entities that would like to subscribe to location information at the location management server of an MC service user.

Figure 10.9.3.6.2-1: On-demand usage of location information procedure

1.	MC service server or location management client send a location information request to the location management server.

2.	The location management server checks if the location management client is authorized to request the location information.

NOTE 2: As the MC service server is implicitly trusted, the Location management server needs not to check the authorization for the MC service server.

3.	The location management server acquires the latest location of the MC service clients being requested, by triggering an on-demand location report procedure as described in clause 10.9.3.2, or from PLMN operator (e.g. LCS network).

4.	Then, location management server immediately sends the location information report including the latest location information acquired of one MC service user to the MC service server or to the location management client. If the optional MC service UE label is present, the location management server sends it, as part of the location information notification, to the MC service user.

NOTE 3: The MC service server does not receive the MC service UE label.

* * Next Change * * *

#### 10.9.3.8	Location management using functional alias

##### 10.9.3.8.1	Client-triggered one-time location information report

Figure 10.9.3.8.1-1 illustrates the procedure when a location management client requests one-time location information from other location management clients for location information reporting using functional alias, which can be shared between several MC service users. Under this condition, the actual location of all MC service users sharing the same functional alias are reported.

Pre-conditions:

1.	MC service client 2 and MC service client 3 share the same functional alias.

2.	MC service client 2 and MC service client 3 activated the functional alias.

3.	MC service client 1 may have an activated functional alias.

4.	The location management server has subscribed to the functional alias controlling MC service server within the MC system for functional alias activation/de-activation updates.

Figure 10.9.3.8.1-1: One-time location information report for shared functional alias

1.	Location management client 1 sends a location reporting trigger, limited to one MC service at the time, to the location management server to activate a one-time location report procedure which shall retrieve the location information of the MC service users that may share the contained functional alias. Location management client 1 may include its own activated functional alias.

2.	Location management server checks whether location management client 1 is authorized to send a location reporting trigger.

3.	The location management server uses on-demand location reporting procedures. The location information request contains the functional alias provided by the location management client 1 to address location management client 2 and location management client 3.

4.	Upon receiving the report, the location management server updates location of the reporting location management clients. If the location management server does not have location information of a reporting location management client before, then just stores the reporting location information for that location management client.

5.	The location management server sends location report responses to location management client 1 containing the provided location information for each location management client using the given functional alias. If the optional MC service UE label is present, the location management server sends it, as part of the location report response to location management client 1. If not all location management clients immediately respond to the location management server, i.e. other reports some time later, subsequent location reporting responses may be sent.

* * Next Change * * *

##### 10.9.3.8.2	Client-triggered periodic location information report

Figure 10.9.3.8.2-1 illustrates the procedure when a location management client requests periodic location information from other location management clients for location information reporting using functional aliases which may be shared between several MC service users. Under this condition, the actual location of all MC service users sharing the same functional alias are reported.

Pre-conditions:

1.	MC service client 2 and MC service client 3 share the same functional alias.

2.	MC service client 2 and MC service client 3 activated the functional alias.

3.	MC service client 1 may have an activated functional alias.

4.	The location management server has subscribed to the functional alias controlling MC service server within the MC system for functional alias activation/de-activation updates.

Figure 10.9.3.8.2-1: Periodic location information report for shared functional alias

1.	Location management client 1 sends a location reporting trigger, limited to one MC service at the time, to the location management server to activate a periodic location reporting procedure which shall retrieve the location information of the MC service users sharing the contained functional alias. Location management client 1 may include its own activated functional alias.

2.	Location management server checks whether location management client 1 is authorized to send a location reporting trigger.

3.	Depending on the information given by the location reporting trigger, the location management server uses event-triggered location information procedure and immediately send location information request to the location management clients that contains the functional alias requested by the location management client 1.

4.	Upon receiving the reports, the location management server updates location of the reporting location management clients.

5.	Based on the received location information reports, the location management server will periodically issue  location report responses, one for each location management client at a time, encompassing the MC service ID, the associated functional alias, the individual location information of the addressed MC service ID and the optional MC service UE label, if present.

NOTE 1:	If a functional alias is deactivated for an MC service client, the corresponding location management client stops sending periodic location information reports.

NOTE 2:	If a functional alias has been newly activated for an MC service client, the location management server activates location information reporting for this location management client.

NOTE 3:	If a functional alias is simultaneously shared between several MC service IDs, all location management clients with the associated MC service IDs will send the location report until the functional alias status change, e.g. take-over, for the individual MC service ID.

* * Next Change * * *

##### 10.9.3.9.2	Report location history procedure

###### 10.9.3.9.2.1	On-demand report location history procedure (LMC – LMS)

The location management server can request stored location information at any time from the location management client, following a return to report location information to the location management server.

Figure 10.9.3.9.2.1-1 illustrates the procedure for the on-demand based usage of location information history reporting from the location management client to the location management server.

Pre-conditions:

1.	The location management client is configured to store location information, while not reporting location information to the location management server.

2.	The location management client has stored location information based on triggering criteria in not reporting location information cases.

Figure 10.9.3.9.2.1-1: On-demand based usage of report location history procedure (LMC – LMS)

1.	The location management client returns to report location information while there is a communication link between the location management client and location management server.

2.	Location management server requests the stored location information.

NOTE 1:	The location management client may provide the status of the stored location information prior to the request from location management server.

3.	The location management client responds to the location management server with one or several stored location information history reports.

NOTE 2:	The transmission of requested stored location information does not interrupt the reporting of location information.

NOTE 3:	The most recent stored location information that qualifies for transmission is transmitted first.

NOTE 4:	The transmission may be stopped at any time with the cancel location history reporting procedure, according to the procedure described in clause 10.9.3.9.4 of the present document.

4.	The location management server updates the available location information with one or several stored location information and the optional MC service UE label, if present, from the location management client.

* * Next Change * * *

###### 10.9.3.9.2.2	On-demand report location history procedure

The location management server can request stored location information at any time from the location management client, following a return to report location information to the location management server. The MC service server or the location management client may initiate the request of stored location information.

Figure 10.9.3.9.2.2-1 illustrates the procedure for the on-demand based usage of location history reporting.

Pre-conditions:

1.	The location management client 1 is configured to store location information, while not reporting location information to the location management server.

2.	The location management client 1 has stored location information based on triggering criteria in not reporting location information cases.

3.	The location management server may have none, a subset or all available stored location information reports from location management client 1.

Figure 10.9.3.9.2.2-1: On-demand based usage of report location history procedure

1.	The MC service server or the location management client 2 initiate the location information history request to the location management server.

NOTE:	The MC service server or the location management client 2 may request the status on stored location information from location management client 1, prior to the location information history request.

2.	The location management server checks the authorization of this request and compares the location information history request with the already retrieved stored location information from location management client 1.

3.	The location management server requests stored location information from location management client 1 according to the procedure described in clause 10.9.3.9.2.1 of the present document, if the desired stored location information is not available on the location management server.

4.	The location management server reports the requested stored location information to the MC service server or location management client 2. The location management server reports the optional MC service UE label, if present, to the location management client 2.

* * * End Changes * * *