3GPP TSG-SA WG6 Meeting #45	S6-212081

e-meeting, 25th August – 3rd September 2021	(revision of S6-211930)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0285 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifying the use of deposit file indication IE in MCData FD request using HTTP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData3 |  |  |  |  |  | Date: |  |  | 2021-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | “Deposit file indication” IE element is present in the MCData FD request using HTTP information flow but its usage is not captured in the relevant procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds clarification regarding the usage of “Deposit file indication” IE to the procedure 7.5.2.4.2 as below : MCData client setting the deposit file indication IE to true if the user wants to deposit the file into MCData message store. MCData server setting the “retrieve file indication” IE to true in the depost object request to the MCData message store. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Will lead to confusion while developing stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.4.2, 7.5.2.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

##### 7.5.2.4.2	Procedure for single MCData system

The procedure in figure 7.5.2.4.2-1 describes the case where a MCData user is initiating one-to-one data communication for sending file to the other MCData user, with or without download completed report request.

Pre-conditions:

1.	The MCData users on the MCData client 1 and the MCData client 2 are already registered for receiving MCData service.

2.	The file to be distributed is uploaded to media storage function on MCData content server using the procedures defined in subclause 7.5.2.2.

3.	The MCData client may have activated functional alias to be used.

4.	The MCData server has subscribed to the MCData functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.5.2.4.2-1: One-to-one file distribution using HTTP

1.	The user at the MCData client 1 initiates a file distribution request to the chosen MCData user.

2.	The MCData client 1 sends a MCData FD request towards the MCData server. The MCData FD request contains content payload in the form of file URL and may contain the file metadata information. The MCData FD request contains one MCData user for one-to-one data communication as selected by the user at MCData client 1. The MCData FD request contains conversation identifier for message thread indication. The MCData FD request may include additional implementation specific information in the application metadata container. If MCData user at MCData client 1 has requested to mandatory download at the recipient side, then MCData FD request contains mandatory download indication. If the MCData user at MCData client has requested to deposit the file content into his/her MCData message store account, then MCData FD request contains deposit file indication set. The MCData FD request may contain download completed report indication if selected by the user at MCData client 1. The MCData user at MCData client 1 may include a functional alias within the FD data transfer and may address the target MCData client 2 using a functional alias.

a)	If the MCData user at the MCData client 1 initiates an MCData emergency file distribution using HTTP or MCData emergency state is already set for the MCData client 1 (due to previously triggered MCData emergency alert):

i)	The MCData FD request shall contain emergency indicator; and

ii)	If MCData emergency state is not set already, MCData client 1 sets its MCData emergency state. The MCData emergency state of MCData client 1 is retained until explicitly cancelled by the user of MCData client 1.

NOTE 1:	While MCData client 1 is in the emergency state, all types of MCData one-to-one and group communications initiated by MCData client 1 are initiated as MCData emergency communications.

3.	MCData server checks whether the MCData user at MCData client 1 is authorized to send MCData FD request and that the size of the file is below maximum data size for FD from the service configuration. MCData server verifies whether the provided functional alias of MCData client 1, if present, can be used and has been activated for the user. If functional alias is used to address that target MCData user, the MCData server resolves the functional alias to the corresponding MCData IDs for which the functional alias is active and proceed with step 4 otherwise proceed with step 6.

NOTE 2:	If the MCData server detects that the functional alias used as the target of the MCData FD request is simultaneously active for multiple MCData users, then the MCData server can proceed by selecting an appropriate MCData ID based on some selection criteria. The selection of an appropriate MCData ID is left to implementation. These selection criteria can include rejection of the MCData FD request, if no suitable MCData ID is selected.

4.	The MCData server may verify whether the corresponding file is available in the MCData content server (not shown in the figure) via the MCData-FD-5 reference point using the received file URL in the MCData FD request. For that, the MCData server sends an MCData file availability request to the MCData content server. Upon the receipt of the request, the MCData content server provides an MCData file availability response to the MCData server. If the MCData server identifies that the corresponding file is not available in the MCData content server, the MCData server provides a response to the MCData client 1 indicating that the file distribution request cannot proceed due to the unavailability of the file in the MCData content server.

5.	The MCData server responds back to MCData client 1 with a functional alias resolution response message that contains the resolved MCData ID.

6.	If the MCData server replies with a MCData functional alias resolution response message, the MCData client 1 sends a new MCData FD request towards the resolved MCData ID.

7.	MCData server initiates the MCData FD request towards MCData client 2. The MCData FD request towards the MCData user contains an emergency indicator if it is present in the received MCData FD request from MCData client 1. If the deposit file indication information element is set to true in the received MCData FD request, MCData server shall follow the procedure as defined in the subclause 7.13.3.8 with the retrieve file indication element set to true while depositing this MCData communication to the MCData message store account of the user at MCData client 1.

NOTE 3:	MCData client 2 does not set its emergency state as a result of receiving the MCData FD request containing the emergency indicator.

8.	The receiving MCData client 2 notifies the user about the incoming MCData FD request (including file metadata, if present) which may be either accepted or rejected or ignored.

9.	The MCData user 2 may provide a response (accept or reject) or not (ignore) to the notification, then MCData client 2 sends the MCData FD response to the MCData server. The MCData client 2 automatically sends an accepted MCData FD response when the received request includes a mandatory download indication.

10.	The MCData server forwards the MCData FD response to the MCData client 1.

11.	The Media storage client on the MCData client 2 downloads the file from the MCData content server using the procedures defined in subclause 7.5.2.3, either automatically (for mandatory download) or based upon the MCData user 2 subsequent action. The MCData client 2 records file download completed and notifies the MCData user 2.

12.	The MCData client 2 provides an MCData download completed report for reporting file download completed, if requested by the user at MCData client 1.

13.	The received MCData file download completed report from the MCData client 2 may be stored by the MCData server for download history interrogation from authorized MCData users. The MCData download completed report is sent by the MCData server to the MCData user at MCData client 1, if requested by the MCData client 1.

* * * * * * * SECOND CHANGE * * * * * * *

##### 7.5.2.6.2	Procedure

The procedure in figure 7.5.2.6.2-1 describes the case where a MCData user is initiating group standalone data communication for sending a file to multiple MCData users, with or without download completed report request from the MCData user.

Pre-conditions:

1.	The MCData users on the MCData clients 1 to n belong to the same MCData group and are already registered for receiving MCData service and affiliated to the group.

2.	The file to be distributed is uploaded to the media storage function on the MCData content server using the procedures defined in subclause 7.5.2.2.

3.	The MCData client may have an activated functional alias to be used.

4.	The MCData server has subscribed to the MCData functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.5.2.6.2-1: Group standalone FD using HTTP

1.	The user at the MCData client 1 initiates a file distribution request to multiple MCData users selecting a pre-configured group (identified by MCData group ID) and optionally particular members from that group.

2.	The MCData client 1 sends a MCData group standalone FD request towards the MCData server. The MCData FD request contains content payload in the form of file URL and may contain the file metadata information. The MCData group standalone data request contains either the selected MCData group ID or the target recipients as selected by the user at MCData client 1. The MCData group standalone FD request contains conversation identifier for message thread indication. The MCData group standalone FD request may include additional implementation specific information in the application metadata container. If MCData user at MCData client 1 has requested to mandatory download at the recipient side, then MCData group standalone FD request contains mandatory download indication. The MCData group standalone FD request may contain a download completed report indication if selected by the user at MCData client 1. The MCData user at MCData client 1 may include a functional alias within the FD data transfer. If the MCData user at MCData client has requested to deposit the file content into his/her MCData message store account, then MCData FD request contains deposit file indication set.

If the MCData user at MCData client 1 initiates an MCData emergency FD communication or the MCData emergency state is already set for the MCData client 1 (due to a previously triggered MCData emergency alert):

i)	the MCData group standalone FD request shall contain an emergency indicator;

ii)	the MCData group standalone FD request shall set an alert indicator if configured to send an MCData emergency alert while initiating an MCData group standalone FD request for the emergency FD communication; and

iii)	if the MCData emergency state is not set already, MCData client 1 sets its MCData emergency state. The MCData emergency state of MCData client 1 is retained until explicitly cancelled by the user of MCData client 1.

NOTE 1:	While MCData client 1 is in the emergency state, all types of MCData one-to-one and group communications initiated by MCData client 1 are initiated as MCData emergency communications.

If the MCData user at MCData client 1 initiates an MCData imminent peril FD communication:

i)	the MCData group standalone FD request shall contain an imminent peril indicator.

2a.	If either emergency indicator or imminent peril indicator is present in the received MCData group standalone FD request, the MCData server implicitly affiliates MCData client 1 to the MCData group if the client is not already affiliated.

3.	MCData server checks whether the MCData user at MCData client 1 is authorized to send an MCData group standalone FD request and that the size of the file is below maximum data size for FD from the group configuration. MCData server verifies whether the provided functional alias, if present, can be used and has been activated for the user. If the MCData group ID is used, the MCData server resolves the MCData group ID to determine the members of that group and their affiliation status, based on the information from the group management server.

i)	If an emergency indicator is present in the received MCData group standalone FD request and if the MCData group is not in the in-progress emergency state, the MCData group is considered to be in the in-progress emergency state until cancelled; and

NOTE 2:	While the MCData group is in the in-progress emergency state, all types of MCData communications within the group are processed as emergency group communications by the MCData server. MCData group members that are not in the emergency state do not indicate emergency in group communication requests.

ii)	If an imminent peril indicator is present in the received MCData group standalone FD request and if the MCData group is not in the in-progress imminent peril state, the MCData group is considered to be in the in-progress imminent peril state until cancelled.

4.	The MCData server may verify whether the corresponding file is available in the MCData content server (not shown in the figure) via the MCData-FD-5 reference point using the received file URL in the MCData group standalone FD request. For that, the MCData server sends an MCData file availability request to the MCData content server. Upon the receipt of the request, the MCData content server provides an MCData file availability response to the MCData server. If the MCData server identifies that the file is not available in the MCData content server, the MCData server provides a response to the MCData client 1 indicating that the file distribution request cannot proceed due to the unavailability of the file in the MCData content server and skip rest of the steps. If the deposit file indication information element is set to true in the received MCData FD request, MCData server shall follow the procedure as defined in the subclause 7.13.3.8 with the retrieve file indication element set to true while depositing this MCData communication to the MCData message store account of the user at MCData client 1.

5.	MCData server initiates the MCData group standalone FD request towards each MCData user determined in step 3. The MCData group standalone FD request towards each MCData client contains:

i)	an emergency indicator if it is present in the received MCData group standalone FD request from the MCData client 1;

ii)	an imminent peril indicator if it is present in the received MCData group standalone FD request from the MCData client 1; and

iii)	an alert indicator if requested to initiate an emergency alert in the received MCData group standalone FD request from the MCData client 1.

6.	The receiving MCData clients 2 to n notify the user about the incoming MCData group standalone FD request (including file metadata, if present) which may be either accepted or rejected or ignored.

7.	If the target MCData user on MCData clients 2 to n provides a response (accept or reject) to the notification, then respective MCData client sends the MCData group standalone FD response to the MCData server. MCData client 2 to n automatically sends accepted MCData group standalone FD response when the incoming request included mandatory download indication.

8.	The MCData server forwards the MCData group standalone FD responses to the MCData client 1.

NOTE 3:	Step 8 can occur at any time following step 5, and prior to step 9 depending on the conditions to proceed with the file transmission.

9.	The media storage client on the MCData client(s) accepting the request downloads the file from the MCData content server (not shown in the figure) using the procedures defined in subclause 7.5.2.3, either automatically (for mandatory download) or based upon the MCData user subsequent action. The MCData clients successfully receiving the file through the media storage clients, record file download completed and notify the MCData users.

10.	The MCData clients, successfully receiving the file through the media storage client, provide an MCData download completed report for reporting file download completed, if requested by the user at MCData client 1.

11.	The MCData file download completed reports from MCData clients may be stored by the MCData server for download history interrogation from the authorized MCData users. The MCData file download completed report from each MCData user may be aggregated.

12.	An aggregated or individual MCData download completed report is sent by the MCData server to the MCData user at MCData client 1, if requested by the MCData client 1.

* * * * * * * END	 CHANGE * * * * * * *