3GPP TSG-SA WG6 Meeting #45	S6-211982

e-meeting, 25th August– 3rd September 2021

- Source:	Lenovo, Motorola Mobility

- Title:	New SID on Study on Integrated Analytics Enablement Service

- Document for:	Approval

Agenda Item:	11

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: Study on Application Data Analytics Enablement Service

## Acronym: FS_ADAES

## Unique identifier:	xx

Potential target Release:	Rel-18

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No |  |  | X |  | X |
| Don't know | X |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a Feature.

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 830047 | Architecture enhancements for 5G System (5GS) to support network data analytics services | FS_IAES may consume services specified in eNA |

## 3	Justification

The use of data analytics is a key enabler in 5GS for providing intelligent and pro-active optimized decisions. In TS 22.186, requirements to support predicted QoS to the 3rd party are specified. It is assumed that 3GPP network provides expected QoS to the V2X Application; however, it is required that V2X application will provide the necessary data to allow for QoS analytics. Also, in TS 22.104, some use cases include applications such as condition monitoring and predictive maintenance based on sensor data, but also big data analytics for optimizing future parameter sets of a certain process. Additionally, GSMA [Generic Slice Template (GST) NG.116 V2.0] includes performance prediction as one attribute for the capability of the mobile system to predict the network and service status.

In 3GPP SA2, data analytics services are provided by the NWDAF (TS 23.288) and aim to support network data analytics services in 5G Core network. Such analytics can collect data from other NFs, or AF or OAM and can be exposed to the 3rd party/ AF to provide statistics and predictions related to slice Load level, observed Service experience, NF Load, Network Performance, UE related analytics (mobility, communication), User data congestion, QoS sustainability, DN performance, etc.

Moreover, in 3GPP SA5 (TS 28.533), management data analytics service (MDAS) provides data analytics for the network. MDAS can be deployed at different levels, for example, at domain level (e.g. RAN, CN, network slice subnet) or in a centralized manner (e.g. in a PLMN level). The objective of MDAS is to optimize the management plane (in network / domain level, in slice / slice subnet level) by performing analytics based on network management data. Such service can be exposed to the 3rd party / MDAS service consumer to provide PM analytics, FM Analytics, NSI / NSSI analytics, optionally recommend appropriate management actions e.g. scaling of resources, admission control, load balancing of traffic, etc.

From application service / vertical viewpoint, some gaps can be the following:

- In vertical scenarios, further data analysis on top of the network layer may be needed, to provide a useful output to the application specific layer for the end-to-end application service. For example, where more one PLMN may be used to communicate a service (e.g. multi-operator V2X platoon); or an application session may consist more than one PDU sessions. Such data analytics may relate to predictions on application QoS parameters for the end-to-end service, and recommended application actions (such as recommended automation level, group formation updates, application to DN/EAS re-mapping, application service area/coverage adaptation, mobility / route adaptation, RAT/slice selection update).

- It is not in the scope of 3GPP SA2 / NWDAF to provide prescriptive analytics (recommendations) to the data analytics consumer considering application layer metrics/actions (related to the application session). Network analytics mainly target to guide the applications to use the network transport resource, while the application-level analytics can be useful for the application specific layer to efficiently adapt their behavior to cope with expected network performance changes. Such application-level analytics require application logic, and a translator would be required to transform network related analytics to application-level predictive actions.

- Some types of data may not be available at the 3GPP network side, e.g. app layer information. Such data may be related to collecting HD maps, camera feeds, sensor data, UE routes/trajectories, data related to edge/cloud resources, data related to application server status (e.g. load of EAS/AS), data related to vertical user information. The enabler layer in some implementations can be deployed as part of the vertical or edge/cloud provider premises and could benefit from analyzing this data.

- Edge/Cloud related data analytics may also be provided as edge/cloud platform capabilities independently from 5GS provided analytics. A coordination of edge/cloud analytics with NWDAF and MDAS analytics would allow for application-centric optimized actions; however this may require a middle layer for translating between the predicted network parameters and the edge/cloud related parameters; otherwise this would be pose significant complexity to the ASP/vertical.

For the above reasons, we propose to study how SA6 can provide application layer data analytics as a possible new capability at the enablement layer for supporting the application specific layer to receive useful stats/predictions/prescriptions for the application service, while complementing the analytics provided by the 5GS (MDAS/NWDAF).

This study may also investigate how the existing enablement services (SEAL, EDGEAPP, vertical enablers) can be enhanced to support application data analytics.

## 4	Objective

To study how the enablement layer can be enhanced to provide application data analytics as a new capability to support the application specific layer. The objectives of the study include:

1) Summarize use cases and service requirements for end-to-end integrated and vertical-tailored analytics based on TS 22.261, TS 22.104.

2)  Perform gap analysis and investigate what new analytics are required to be derived by the enabler layer to improve the application enablement support services and what data need to be collected by 5GS (NWDAF, MDAS), by edge/cloud platforms and UE to derive such analytics.

3)  Study possible new enablement value-add services related to potential new application data analytics (Predictive/Prescriptive edge-aware or vertical specific analytics) exposure to the vertical / application specific layer.

4) 	Identify potential enhancements to existing enabler layer services (SEAL, EDGEAPP, vertical specific enablers), to support application data analytics enablement.

5)	Investigate possible impacts of application data analytics enablement, where application data are collected from both edge and cloud platforms.

NOTE: Coordination may be required with SA2, SA4 and SA5 for the data to be collected by the application data analytics enabler entity.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.XYZ | Study on Analytics Enablement Service | SA#95 (Mar 2022) | SA#96 (Jun 2022) | Emmanouil Pateromichelakis, Lenovo, Motorola Mobility, epateromiche@lenovo.com |

## 6	Work item Rapporteur(s)

Pateromichelakis, Emmanouil (Manos), epateromiche@lenovo.com

## 7	Work item leadership

SA6

## 8	Aspects that involve other WGs

SA2 for system aspects and possible coordination for the data collection provided by 5GC, SA3 for security aspects, SA4 for possible coordination for the data collection provided by SA4, and SA5 for management aspects and possible coordination for the data collection provided by OAM.

## 9	Supporting Individual Members

Supporting IM name

Lenovo

Motorola Mobility

AT&T

ChinaUnicom