3GPP TSG-SA WG6 Meeting #45	S6-212065

e-meeting, 25th August – 3rd September 2021	(revision of S6-211985)

Source:	CATT

Title:	Pseudo-CR on fused location service architecture

Spec:	3GPP TR 23.700-96 v0.1.1

Agenda item:	10.7

Document for:	Approval

Contact:	Ling Zhang, zhangling@catt.cn

1. Introduction

This contribution defines functional architecture for the study.

2. Reason for Change

As discussed in S6-211984.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-96 v0.1.1.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[4]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[7]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; Stage 1".

[8]	3GPP TS 22.125: "Unmanned Aerial System (UAS) support in 3GPP; Stage 1".

[9]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

[10]	Open Mobile Alliance, OMA AD SUPL: "Secure User Plane Location Architecture", (http://www.openmobilealliance.org).

[11]	Open Mobile Alliance, OMA AD MLS: "Mobile Location Service Architecture", (http://www.openmobilealliance.org).

* * * Next Change * * * *

# 7	Solutions

## 7.1	Solution #1: Standalone functional architecture for fused location service

7.1.1	Solution description

This solution addresses key issue #1: Architecture enhancement of application enablement for location.

7.1.1.1	Functional architecture

The figure 7.1.1.1-1 identifies the architecture of fused location service enabled by 5GS.

Figure 7.1.1.1-1:	Functional architecture of fused location service

The architecture is composed of logical function modules that are not necessarily physical entities and can reside in or co-locate with existing application layer entities as appropriate.

In the architecture the fused location service modules can be within the MNO domain or third party service provider domain.

Editor’s Note: It is FFS to show the relationship of Fused Location Function with SEAL-LM.

7.1.1.2	Functional components and reference points

The Fused Location Function provides location information of the target UE based on positioning or location data retrived from one or multiple location source. It provides normalized description of location data to the application specific server (e.g. of ecosystem partners) through the northbound API.

The Location Service components provide location centric capabilities exposure with the dimensions of e.g. environment of use (indoor, outdoor, urban, etc.), categorized location service exposure based on accuracy/precision and so on. Different location servic components may have separated function module representation and expose their APIs respevtively. The network operator or service provider can decide to implement certain location service components according to their needs.

NOTE:	The location service components are part of the Fused Location Function.

The 3rd Party Location Service provides the location of a certain location technology (typically the network-based positioning) e.g. Bluetooth.

The Fused Location Client represents the client of target UE providing the UE-based poisitioning.

The NEF (as defined in 3GPP TS 23.501 [5]) exposes location service of 5GC when Fused Location Function is external to MNO domain.

The GMLC (as defined in 3GPP TS 23.273 [4]) provides LCS when Fused Location Function is within the MNO domain.

The SLP (as defined in OMA AD SUPL [10]) provides location of SUPL network.

The interfaces are described as followed:

FLS-1:	Service-based interface providing location reporting by the Fused Location Client of the target UE.

FLS-2:	Service-based interface providing fused location exposure.

FLS-X:	The reference point is used for location retrieval of the target UE from that 3rd party location service.

NOTE:	The definition of FLS-X is out of scope of this specification.

Nnef:	Service-based interface as defined in 3GPP TS 23.501 [5].

Le:	Reference point as defined in OMA AD MLS [11].

7.1.2	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.