3GPP TSG-WG SA6 Meeting #46-e	S6-212562

# e-meeting, 15th – 23th November 2021	(revision of S6-212554)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0012 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS architectural and functionalities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CBN, Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS 23.247 on 5G MBS stage 2 in SA2 was approved in the last SA plenary, and according to cover page of SA2 MBS (S2-2106682), there is no SA2 internal outstanding issues. Therefore, the architecture now is stable and it is sufficient for SA6 to conclude the architectural for 5G MBS for transport only mode. In TR 23.783, the description of Key Issue #13 includes the architectural related descriptions, which can be used as the basis for architectural conclusion. Moreover, Sol#8 (Application architecture over 5G MBS for transport only mode) and Sol#9 (General use of 5G MBS services for MC service group communications) further include the functionality descriptions. In fact Sol#8 and Sol#9 are complementary to each other, and both of the solutions can be adopted as the selected ones. For Service Mode, more coordination with SA4 is needed: The working in SA4 (i.e., 5MBUSA) is still on-going and according to the clause 7.1.1.1 of TS 23.247, "services offered by the MBSF and related interactions based on that service between MBSF and AF or NEF (if MBSF and NEF are split as shown in configuration 2) are specified in TS 26.502, and detailed interactions between the MBSF or NEF and the MBSTF are specified in TS 26.502". The IWK issue is also addressed in this document. Therefore, it is proposed to conclude Application architecture without MBSF/MBSTF for transport only mode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Architectural and functional description of TS 23.783 in TS 23.289. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no description for Architectural and functionalities in current technical specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.x (new), 4.x.1 (new), 4.x.2 (new), 5.4.1, 5.5.1, 5.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[3]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[4]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[5]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[6]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[7]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[8]	3GPP TS 23.002: "Network Architecture".

[9]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[11]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT); Stage 1".

[12]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[13]	3GPP TS 22.281: "Mission Critical (MC) Video".

[14]	3GPP TS 22.282: "Mission Critical (MC) Data".

[x]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

# * * * * Second change * * * *

4.X	MC service over 5G MBS for transport only mode

4.x.1	General description about usage of 5G MBS

Supporting function and corresponding reference points applicable for 5G MBS are defined in 3GPP TS 23.247 [x] clause 5.1 using the Service-based approach and in reference point representation.

Similar to EPS, 5G MBS architecture provide down link transport only services to application layer either in trusted mode (N29mb/N30), or non-trusted mode (N33 via NEF). Compared with the functional features supported by LTE/EPS, the main MBS enhancements in 5G NR are:

- Group scheduling mechanism to allow UEs to receive MBS service including simultaneous operation with unicast reception;

- To deliver the MBS data traffic from MB-UPF to NG-RAN, namely, 5GC Shared Delivery (SD) mode and 5GC Individual Delivery (ID) mode. MBS NG-RAN capabilities determine if Point-to-Point or Point-to-Multipoint gets used for Shared Delivery mode.

- Reliability enhancements by dynamic change between Point-to-Multipoint (PTM) and Point-to-Point (PTP) applicable to multicast/broadcast service delivery;

- Supporting mobility to adapt to available delivery mode and lossless handover;

- Reception of broadcast data irrespective of UE’s Radio Resource Control (RRC) states;

### 4.X.2	MC service functional model for related specific MC services

Broadcast/multicast mode offered by the 5G MBS technology is well suited for group communications, which form a major part of Mission Critical related communications. The MC service on-network functional model for the use of 5G MBS, is based on the MBS functional model in accordance to 3GPP TS 23.247 [x] by the MC service server taking the role of the Application Function (AF).

Figure 4.X.2-1 illustrates the functional model for MC services over 5G MBS for transport only mode.

MC service clients

5G NR

MC service UE

User Plane

Control Plane

5GC+RAN

MC service server

Uu

MCPTT-1;

MCVideo-1;

MCData-1

N6mb

N33 (untrusted)

N30/N29mb (trusted)

Figure 4.X.2-1: MC service functional model applying 5G MBS

Editor's Note: In clause 4.X, the exact interfaces name for interaction between AF and SMF (N29mb or Nmb13) will be further aligned with SA2 final decision.

Editor's Note: In clause 4.X, the exact interfaces name for interaction between AF and PCF (N30 or N5mb) will be further aligned with SA2 final decision.

### 4.X.3	MC service functional model for related specific MC services for interworking with MBMS

When MBSF/MBSTF is deployed in 5GS for supporting 5G MBS-eMBMS interworking at 5GC for transport only mode, Nmb5 are used for trusted operation of MC services, or N33 are used for non-trusted operation of MC services. When working at transport only mode, the MC service server supports the same functions for interacting with MBSF via Nmb5 as interacting with NEF via N33.

Figure 4.X.3-1 illustrates the functional model applying 5G MBS for interworking with MBMS at 5GC.

MC service clients

5G NR

MC service UE

User Plane

Control Plane

5GC+RAN

MC service server

Uu

MCPTT-1;

MCVideo-1;

MCData-1

Nmb5 /MB2-C

Nmb8/MB2-U

Figure 4.X.3-1: MC service functional model applying 5G MBS for interworking with MBMS at 5GC.

Editor's Note: In clause 4.X, the exact interfaces name for interaction between AF and MBSF (Nmb5 or Nmb10) will be further aligned with SA2 final decision.

NOTE:	In 5GC, the functional entities in figure 4.X.3-1 can be collocated of BM-SC and MBSF/MBSTF as joint entities or separated deployed.

When the interworking with MBMS is at MC service server, MC service server provides MC service data to 5GS by invoking N30/N29mb (for trusted operation of MC services), or N33 (for non-trusted operation of MC services), and provides MC service data to EPS by invoking MB2-C and MB2-U.

Figure 4.X.3-2 illustrates the functional model applying 5G MBS for interworking with MBMS at MC service server.

MC service clients

5G NR

MC service UE

User Plane

Control Plane

5GC+RAN

MC service server

Uu

MCPTT-1;

MCVideo-1;

MCData-1

N6mb

N33 (untrusted)

N30/N29mb (trusted)

MC service clients

E-UTRAN

MC service UE

User Plane

Control Plane

EPC+RAN

Uu

MCPTT-1;

MCVideo-1;

MCData-1

MB2-U

MB2-C

Figure 4.X.3-2: MC service functional model applying 5G MBS for interworking with MBMS at MC service server.

# * * * * Third change * * * *

5.4.1	On-network functional model

Figure 5.4.1-1 shows the functional model for the application plane for an MCPTT system using the 5GS.

Figure 5.4.1-1: MCPTT functional model for application plane

In the functional model shown in figure 5.4.1-1, the following is considered:

-	The description of the corresponding functional entities and reference points in 3GPP TS 23.379 [6] applies.

-	The description of the MCPTT-4 and MCPTT-7 reference points in 3GPP TS 23.379 [6] applies considering that it utilizes the N6 reference point defined in 3GPP TS 23.501 [7].

-	The description of the MCPTT-5 reference point in 3GPP TS 23.379 [6] applies considering that it exists between the MCPTT server and the 5GS. It is used for resource management of MCPTT sessions, e.g. QoS control, and utilizes the N5 reference point or the Rx reference point or the N33 reference point as defined in 3GPP TS 23.501 [7], 3GPP TS 23.502 [10] and 3GPP TS 23.503 [9].

-	MCPTT-5, utilizing Rx reference point or N5 reference point, may be used when the MCPTT service provider directly interacts with operator's relevant 5GS network function for QoS control.

-	MCPTT-5, utilizing N33 reference point, may be used when the MCPTT service provider is limited by the operational agreement, i.e., indirect interaction with operator's 5GS network functions for QoS control.

NOTE:	Any coordination between the P-CSCF use of N30/N29mb and the MCPTT server use of N30/N29mb (via MCPTT-5) from the MCPTT service provider domain is not specified in this release of this specification.

-	The MCPTT-6 reference point, which exists between the MCPTT server and the 5GS, is used to establish a MBS session obtaining multicast or broadcast transport resources, request the allocation and activation of MBS transport resources for MCPTT application usage. The MCPTT-6 reference point uses the N29mbN30 or N33 interface as defined in 3GPP TS 23.247 [x].

-	MCPTT-6, utilizing N30 and N29 reference points, may be used when the MCPTT service provider and the PLMN operator have an operational agreement where QoS control is provided directly from the MCPTT service provider domain.

-	MCPTT-6, utilizing N33 reference point, may be used when the MCPTT service provider is limited by the operational agreement, i.e. indirect interaction with operator's 5GS network functions for QoS control.

-	MCPTT-6, utilizing N33 reference point, is also used by the MCPTT server to configure the QoS requirements in correspondence to the MBS session identifier if dynamic PCC is applicable.

-	MCPTT-6, utilizing Nmb5 reference point, maybe used when MBSF is deployed at 5GS for supporting transport only mode service continuity between eMBMS/5G MBS, and MCPTT service provider and the PLMN operator have an operational agreement where QoS control is provided directly from the MCPTT service provider domain.

-	The description of the MCPTT-8 reference point in 3GPP TS 23.379 [6] applies considering that it exists between MCPTT server and the 5GS. The MCPTT-8 reference point uses the N6mb interface defined in 3GPP TS 23.247[x].

-	The description of the MCPTT-9 reference point in 3GPP TS 23.379 [6] applies considering that it exists between MCPTT server and the 5GS. The MCPTT-9 reference point uses the N6mb interface defined in 3GPP TS 23.247 [x].

-	The MCPTT-8 reference point may use the N5mb reference point according to 3GPP TS 23.247 [x] when MBSTF is deployed for eMBMS/5G MBS interworking.

# * * * * Fourth change * * * *

5.5.1	On-network functional model

Figure 5.5.1-1 shows the functional model for the application plane for an MCVideo system using the 5GS.

Figure 5.5.1-1: MCVideo functional model for application plane

In the functional model shown in figure 5.5.1-1, the following is considered:

-	The description of the corresponding functional entities and reference points in 3GPP TS 23.281 [4] applies.

-	The description of the MCVideo-4 and MCVideo-7 reference points in 3GPP TS 23.281 [4] applies considering that it utilizes the N6 reference point defined in 3GPP TS 23.501 [7].

-	The description of the MCVideo-5 reference point in 3GPP TS 23.281 [4] applies considering that it exists between the MCVideo server and the 5GS. It is used for resource management of MCVideo sessions, e.g. QoS control, and utilizes the N5 reference point or the Rx reference point or the N33 reference point as defined in 3GPP TS 23.501 [7], 3GPP TS 23.502 [10] and 3GPP TS 23.503 [9].

-	MCVideo-5, utilizing Rx reference point or N5 reference point, may be used when the MCVideo service provider directly interacts with operator's relevant 5GS network function for QoS control.

-	MCVideo-5, utilizing N33 reference point, may be used when the MCVideo service provider is limited by the operational agreement, i.e., indirect interaction with operator's 5GS network functions for QoS control.

-	TheMCVideo-6 reference point, which exists between the MCVideo server and the 5GS, is used to establish a MBS session obtaining multicast or broadcast transport resources, request the allocation and activation of MBS transport resources for MCPTT application usage. The MCVideo-6 reference point uses the N29mbN30 or N33 interface as defined in 3GPP TS 23.247 [x].

-	The MCVideo-6 reference point provides the similar features as MCPTT-6, with the only difference that when PCC applies, MCPTT-6 provides the QoS control for MCPTT voice media, while MCVideo-6 provides the QoS control for MCVideo media.

-	MCVideo-6, utilizing Nmb5 reference point, maybe used when MBSF is deployed at 5GS for supporting transport only mode service continuity between eMBMS/5G MBS, and MCVideo service provider and the PLMN operator have an operational agreement where QoS control is provided directly from the MCVideo service provider domain.

-	The description of the MCVideo-8 reference point in 3GPP TS 23.281 [4] applies considering that it exists between the MCVideo server and the 5GS. The MCVideo-8 reference point uses the N6mb interface defined in 3GPP TS 23.247 [x].

-	The description of the MCVideo-9 reference point in 3GPP TS 23.281 [4] applies considering that it exists between MCVideo server and the 5GS. The MCVideo-9 reference point uses the N6mb interface defined in 3GPP TS 23.247 [x].

-	The MCVideo-8 reference point may use the N5mb reference point according to 3GPP TS 23.247 [x] when MBSTF is deployed for eMBMS/5G MBS interworking.

# * * * * Fifth change * * * *

5.6.1	On-network functional model

Figure 5.6.1-1 shows the generic functional model for the application plane for an MCData system using the 5GS.

Figure 5.6.1-1: Generic MCData functional model for application plane

In the functional model shown in figure 5.6.1-1, the following is considered:

-	The description of the corresponding functional entities and reference points in 3GPP TS 23.282 [5] applies.

-	The description of the MCData-5 reference point in 3GPP TS 23.282 [6] applies considering that it exists between the MCData server and the 5GS. It is used for resource management of MCData sessions, e.g. QoS control, and utilizes the N5 reference point or the Rx reference point or the N33 reference point as defined in 3GPP TS 23.501 [7], 3GPP TS 23.502 [10] and 3GPP TS 23.503 [9].

-	MCData-5, utilizing Rx reference point or N5 reference point, may be used when the MCData service provider directly interacts with operator's relevant 5GS network function for QoS control.

-	MCData-5, utilizing N33 reference point, may be used when the MCData service provider is limited by the operational agreement, i.e., indirect interaction with operator's 5GS network functions for QoS control.

-	The respective functional models supporting MCData capabilities (e.g., SDS, FD, DS, IPcon) over unicast transmissions along with the corresponding reference points (i.e., MCData-cap-1 to MCData-cap-n) described in 3GPP TS 23.282 [5] also apply when the 5G system is used.

-	The MCData-6 reference point, which exists between the MCData server and the 5GS, is used to establish a MBS session obtaining multicast or broadcast transport resources, request the allocation and activation of MBS transport resources for MCData application usage. The MCData-6 reference point uses the Nmb13/N33 interface as defined in 3GPP TS 23.247 [x].

-	MCData-6 provides the similar features as MCPTT-6, with the only difference that when PCC applies, MCPTT-6 provides the QoS control for MCPTT voice media, while MCData-6 provides the QoS control for MCData media.

-	MCData-6, utilizing Nmb5 reference point, maybe used when MBSF is deployed at 5GS for supporting transport only mode service continuity between eMBMS/5G MBS, and MCData service provider and the PLMN operator have an operational agreement where QoS control is provided directly from the MCData service provider domain.

-	The description of the MCData-8 reference point in 3GPP TS 23.281 [4] applies considering that it exists between the MCData server and the 5GS. The MCData-8 reference point uses the N6mb interface defined in 3GPP TS 23.247 [x].

-	The description of the MCData-9 reference point in 3GPP TS 23.281 [4] applies considering that it exists between MCData server and the 5GS. The MCData reference point uses the N6mb interface defined in 3GPP TS 23.247 [x].

-	The MCData-8 reference point may use the N5mb reference point according to 3GPP TS 23.247 [x] when MBSTF is deployed for eMBMS/5G MBS interworking.

# * * * * End of changes * * * *