3GPP TSG-SA WG6 Meeting #46-e	S6-212584

e-meeting, 15th – 23rd November 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0019 | rev | - | Current version: | 17:0:0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCPTT and MCVideo functional model update to enable MC services over MBS sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2021-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Introduced CR provides an update to the MCPTT and MCVideo functional model enabling MBS services. The updates introduces reference points between the MC service server and the 5GC, based on the configuration under consideration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to the MCPTT and MCVideo functional models for application plane are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No content or information regarding the reference points between MCPTT/ MCVideo and the 5GC from the other. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.4.1 and 5.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 5.4	MCPTT functional model description

### 5.4.1	On-network functional model

Figure 5.4.1-1 shows the functional model for the application plane for an MCPTT system using the 5GS.

Figure 5.4.1-1: MCPTT functional model for application plane

In the functional model shown in figure 5.4.1-1, the following is considered:

-	The description of the corresponding functional entities and reference points in 3GPP TS 23.379 [6] applies.

-	The description of the MCPTT-4 and MCPTT-7 reference points in 3GPP TS 23.379 [6] applies considering that it utilizes the N6 reference point defined in 3GPP TS 23.501 [7].

-	The description of the MCPTT-5 reference point in 3GPP TS 23.379 [6] applies considering that it exists between the MCPTT server and the 5GS. It is used for resource management of MCPTT sessions, e.g. QoS control, and utilizes the N5 reference point or the Rx reference point or the N33 reference point as defined in 3GPP TS 23.501 [7], 3GPP TS 23.502 [10] and 3GPP TS 23.503 [9].

-	MCPTT-5, utilizing Rx reference point or N5 reference point, may be used when the MCPTT service provider directly interacts with operator's relevant 5GS network function for QoS control.

-	MCPTT-5, utilizing N33 reference point, may be used when the MCPTT service provider is limited by the operational agreement, i.e., indirect interaction with operator's 5GS network functions for QoS control.

-	The MCPTT-6 reference point, which exists between the MCPTT server and the 5GS, is used to createan MBS session obtaining multicast or broadcast resources for MCPTT application usage. The MCPTT-6 reference point uses the N33, Nmb13, Nmb10, MB2-C reference points according to 3GPP TS 23.247 [19].

-	MCPTT-6, utilizing Nmb13, may be used when the MCPTT service provider and the PLMN operator have an operational agreement where QoS control is provided directly from the MCPTT service provider domain.

-	MCPTT-6, utilizing N33 reference point, may be used when the MCPTT service provider is limited by the operational agreement, i.e., indirect interaction with operator's 5GS network functions for QoS control.

-	MCPTT-6, is also used by the MCPTT server to configure the QoS requirements in correspondence to the MBS session identifier if dynamic PCC is applicable.

-	MCPTT-6, utilizing Nmb10/MB2-C reference points, may be used when MCPTT service provider interacts with MBSF directly. The interaction with MBSF may either be once it facilitates interworking with LTE, or for full-service mode.

-	MCPTT-8, reference point definition in 3GPP TS 23.379 [6] is applied. When the MC service obtains 5GS full-service capability, the MCPTT-8 reference point uses the Nmb8 reference point according to 3GPP TS 23.247[x].

-	MCPTT-8, reference point definition in 3GPP TS 23.379 [6] is applied towards MBSTF. It is needed once interworking with LTE is facilitated via MBSF/MBSTF, the MCPTT-8 reference point uses the MB2-U reference point according to 3GPP TS 23.247[19].

-	MCPTT-9, reference point definition in 3GPP TS 23.379 [3] is applied. And when MC service is over 5GS, the MCPTT-9 reference point uses the Nmb8 reference point according to 3GPP TS 23.247 [x].

-	MCPTT-9, reference point definition in 3GPP TS 23.379 [6] is applied. And when MC service is over 5GS, the MCPTT-9 reference point uses the MB2-U reference point according to 3GPP TS 23.247 [x].

# * * * * Second change * * * *

## 5.5	MCVideo functional model description

### 5.5.1	On-network functional model

Figure 5.5.1-1 shows the functional model for the application plane for an MCVideo system using the 5GS.

Figure 5.5.1-1: MCVideo functional model for application plane

In the functional model shown in figure 5.5.1-1, the following is considered:

-	The description of the corresponding functional entities and reference points in 3GPP TS 23.281 [4] applies.

-	The description of the MCVideo-4 and MCVideo-7 reference points in 3GPP TS 23.281 [4] applies considering that it utilizes the N6 reference point defined in 3GPP TS 23.501 [7].

-	The description of the MCVideo-5 reference point in 3GPP TS 23.281 [4] applies considering that it exists between the MCVideo server and the 5GS. It is used for resource management of MCVideo sessions, e.g. QoS control, and utilizes the N5 reference point or the Rx reference point or the N33 reference point as defined in 3GPP TS 23.501 [7], 3GPP TS 23.502 [10] and 3GPP TS 23.503 [9].

-	MCVideo-5, utilizing Rx reference point or N5 reference point, may be used when the MCVideo service provider directly interacts with operator's relevant 5GS network function for QoS control.

-	MCVideo-5, utilizing N33 reference point, may be used when the MCVideo service provider is limited by the operational agreement, i.e., indirect interaction with operator's 5GS network functions for QoS control.

-	The MCVideo-6 reference point, which exists between the MCVideo server and the 5GS, is used to createan MBS session obtaining multicast or broadcast resources for MCVideo application usage. The MCVideo -6 reference point uses the N33, Nmb13, Nmb10, MB2-C reference points according to 3GPP TS 23.247 [x].

-	MCVideo -6, utilizing Nmb13, may be used when the MCVideo service provider and the PLMN operator have an operational agreement where QoS control is provided directly from the MCVideo service provider domain.

-	MCVideo -6, utilizing N33 reference point, may be used when the MCVideo service provider is limited by the operational agreement, i.e., indirect interaction with operator's 5GS network functions for QoS control.

-	MCVideo -6, is also used by the MCVideo server to configure the QoS requirements in correspondence to the MBS session identifier if dynamic PCC is applicable.

-	MCVideo -6, utilizing Nmb10/MB2-C reference points, may be used when MCVideo service provider interacts with MBSF directly. It is needed either once interworking with LTE is facilitated via MBSF/MBSTF, or for full-service mode.

-	MCVideo -8, reference point definition in 3GPP TS 23.281 [4] is applied. When the MC service obtains 5GS full-service capability, the MCVideo -8 reference point uses the Nmb8 reference point according to 3GPP TS 23.247[x].

-	MCVideo -8, reference point definition in 3GPP TS 23.281 [4] is applied towards MBSTF. It is needed once interworking with LTE is facilitated via MBSF/MBSTF, the MCVideo -8 reference point uses the MB2-U reference point according to 3GPP TS 23.247[xx].

-	MCVideo -9, reference point definition in 3GPP TS 23.281 [4] is applied. And when MC service is over 5GS, the MCVideo -9 reference point uses the Nmb8 reference point according to 3GPP TS 23.247 [x].

-	MCVideo -9, reference point definition in 3GPP TS 23.281 [4] is applied. And when MC service is over 5GS, the MCVideo -9 reference point uses the MB2-U reference point according to 3GPP TS 23.247 [x].

# * * * * Third change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[3]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[4]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[5]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[6]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[7]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[8]	3GPP TS 23.002: "Network Architecture".

[9]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[11]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT); Stage 1".

[12]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[13]	3GPP TS 22.281: "Mission Critical (MC) Video".

[14]	3GPP TS 22.282: "Mission Critical (MC) Data".

[15]	3GPP TS 23.247: “Architectural enhancements for 5G multicast-broadcast services; Stage 2”