3GPP TSG-SA WG6 Meeting #46-e meeting	S6-212739

15th Nov – 23rd Nov 2021, Online	(Revision of S6-212521_rev3)

Source:	Intel, Nokia, KDDI, SKT

Title:	Edge Dual Deployment Considerations

Spec:	3GPP TR 23.700-98

Agenda item:	10.7

Document for:	Approval

Contact:	samar.shailendra@intel.com

# 1	Introduction

As specified in Annex C of TS 23.558 (Rel-17), the two architectures (EDGEAPP and ETSI MEC) can be present in a single system. This pCR provides deployment considerations possible for the existence of the two architectures in a deployment environment.

# 2	Proposal

It is proposed to modify the text of TR 23.700-98 as follows.

1st CHANGE

### Annex y (Informative): Edge Dual Deployment Consideration – ETSI MEC and EDGEAPP

y.1 Deployment Consideration

Annex C of TS 23.558 provides a relationship between EDGEAPP and ETSI MEC architectures as in figure below.

Figure y.1: Relationship in EDGEAPP and ETSI MEC architecture

This Annex discusses deployment options of both ETSI MEC and EDGEAPP in an operator environment. The following deployment options can be considered for the coexistence of EES and MEC Platform.

y.1.1 Deployment Option-1: Collocated Platforms as different AF

Based on Figure y.1, the two platforms (EES and MEC Platform) are co-located, and made by a single (unique) equipment, which is compliant with both standards.  The two platforms can be implemented as two different AFs (e.g., realized as two different VNFs) deployed at the same equipment (Figure y.1.1).

Figure y.1.1: EES and MEC Platform as two different AFs on a single Physical/NFV Infrastructure

y.1.2 Deployment Option-2: Collocated Platforms as single AF

The two platforms (EES and MEC Platform) are co-located  and even coincide with the same (hybrid) software platform, which can be practically a single AF (r.g. realized as one VNF) compliant with both standards (Figure y.1.2).

Figure y.1.2: EES and MEC Platform as a single AF on a single NFV Infrastructure (NFVI)

y.1.3 Deployment Option-3: non-Collocated Platforms

The two platforms are non-collocated, and reside in two different data networks, where EES is in the Mobile Network Operator (MNO) domain while the ETSI MEC platform is outside of MNO domain (Figure y.1.3).

Figure y.1.3: EES and MEC Platform as two different AFs in two different EDNs

Note: Another flavor of the deployment option #3 might be  the one where the two platforms (i.e., EES and MEC Platform) are non-collocated, and both residing within the same trusted domain.

END OF CHANGES