3GPP TSG-SA6 Meeting #46-e	S6-21xxxx

- Online, , 15th Nov 2021 - 23rd Nov 2021 was S6-212610

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0069 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Functional entity responsibilities related to ACR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone España SA |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2021-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Descriptions of application context relocation in TS 23.558 identify various responsibilities for the entities involved. for example 8.6.3.1 General The EES exposes ACR management event notifications of one or more UEs to an EAS (e.g. in order to trigger the ACR) These responsibilities are missing from clause 6.3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Missing ACR responsibilities added for AC, EEC, EES |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Functional entities implementations will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, 6.3.3, 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.3	Functional entities

### 6.3.1	General

This clause describes the functional entities of the architecture for enabling edge applications.

### 6.3.2	Edge Enabler Server (EES)

EES provides supporting functions needed for EASs and EEC.

Functionalities of EES are:

a)	provisioning of configuration information to EEC, enabling exchange of application data traffic with the EAS;

b)	providing API invoker and API exposing functions as specified in 3GPP TS 23.222 [6];

c)	interacting with 3GPP Core Network for accessing the capabilities of network functions either directly (i.e. via PCF) or indirectly (i.e. via SCEF/NEF/SCEF+NEF);

d)	exposing events related to ACT;

e)	EEC context transfer between EESs;

e)	supporting external exposure of 3GPP network and service capabilities to the EAS(s) over EDGE-3;

f)	registration functions (i.e., registration, update, and de-registration) for the EEC(s) and the EAS(s); and

g) triggering the EAS instantiation on demand.

### 6.3.3	Edge Enabler Client (EEC)

EEC provides supporting functions needed for AC(s).

Functionalities of EEC are:

a)	retrieval and provisioning of configuration information to enable the exchange of Application Data Traffic with the EAS;

b)	discovery of EASs available in the EDN; and

c)	detecting UE mobility events and relating them to its service area.

### 6.3.4	Edge Configuration Server (ECS)

ECS provides supporting functions needed for the EEC to connect with an EES.

Functionalities of ECS are:

a)	provisioning of Edge configuration information to the EEC. The Edge configuration information includes the following:

1)	the information for the EEC to connect to the EES (e.g. EDN service area); and

2)	the information for establishing a connection with EESs (such as URI);

NOTE:	The ECS can be deployed in the MNO domain or can be deployed in 3rd party domain by service provider.

b)	supporting the functionalities of registration (i.e., registration, update, and de-registration) for the EES(s);

c)	supporting the functionalities of API invoker and API exposing function as specified in 3GPP TS 23.222 [6]; and

d)	interacting with 3GPP Core Network for accessing the capabilities of network functions either directly (e.g. via PCF) or indirectly (e.g. via SCEF/NEF/SCEF+NEF).

### 6.3.5	Application Client (AC)

AC is the application resident in the UE performing the client function. Depending on how ACR is implemented, the AC might be required to decide whether to initiate ACR. Other details of the AC are out of scope of this specification.