3GPP TSG-SA WG6 Meeting #46-e meeting	  S6-212530

15th – 23rd November 2021, Online

Source:	ZTE Corporation

Title:	FFAPP Application Architecture

Spec:	3GPP TS 23.545 v0.1.0

Agenda item:	9.1

Document for:	Approval

Contact:			Shao Weixiang, shao.weixiang@zte.com.cn

1. Introduction

This contribution provides a proposal for FFAPP Application Architecture.

2. Reason for Change

Add FFAPP Application Architecture.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.545 v0.1.0.

* * * First Change * * * *

## 5.2	Application Architecture

Figure 5.2-1 illustrates the FF application layer architecture.

Figure 5.2-1: FF application layer architecture

The FF application layer functional entities for the FF UE and the FF application server are grouped into the FF application specific layer and the FAE layer. The FAE layer offers the FAE capabilities to the FF application specific layer. The FF application layer functional model utilizes the SEAL services as specified in 3GPP TS 23.434 [x].

The FAE server is located in the FAE layer. The SEAL services utilized by FAE layer are location management, group management, configuration management, identity management, key management, network resource management and network slice capability management. The FF application specific layer consists of the FF application specific functionalities.

NOTE 1:	The functionalities of the FF application specific layer are out of scope of the present document.

The FF application server consists of the FAE server, the SEAL servers and the FF application specific server. The FAE server provides the FF application layer support functions to the FF application specific server over FAE-S reference point. The SEAL server(s) provide the SEAL services to the FF application specific server over SEAL-S reference point.

The FF UE consist of the FAE client, the SEAL clients and the FF application specific client. The FAE client provides the FF application layer support functions to the FF application specific client over FAE-C reference point. The SEAL client(s) provide the SEAL services to the FF application specific client over SEAL-C reference point.

NOTE 2:	In some deployments, the client and server entities of SEAL can be part of FAE client and FAE server respectively.

The FAE client acts as a VAL client for its interaction with the SEAL clients as specified in 3GPP TS 23.434 [x]. The FAE server acts as a VAL server for its interaction with the SEAL servers as specified in 3GPP TS 23.434 [x].

In the FAE layer, the FAE client communicates with the FAE server over FAE-1 reference point. In the FF application specific layer, the FF application specific client communicates with FF application specific server over FFA-1 reference point.

NOTE 3:	The FFA-1 reference point is out of scope of the present document.

In the FAE layer, the FAE client of FF UE2 communicates with FAE client of FF UE1 over FAE-2 reference point. In the FF application specific layer, the FF application specific client of FF UE2 communicates with FF application specific client of FF UE1 over FFA-2 reference point.

NOTE 4:	The FFA-2 reference point is out of scope of the present document.The following SEAL services for FF applications are supported:

-	Location management as specified in 3GPP TS 23.434 [x];

-	Group management as specified in 3GPP TS 23.434 [x];

-	Configuration management as specified in 3GPP TS 23.434 [x];

-	Identity management as specified in 3GPP TS 23.434 [x];

-	Key management as specified in 3GPP TS 23.434 [x];

-	Network resource management as specified in 3GPP TS 23.434 [x] and;

-	Network slice capability management as specified in 3GPP TS 23.434 [x].

The FAE client interacts with SEAL clients over the SEAL-C reference point specified for each SEAL service. The FAE server interacts with SEAL servers over the SEAL-S reference point specified for each SEAL service. The interaction between the SEAL clients is supported by SEAL-PC5 reference point specified for each SEAL service. The interaction between a SEAL client and the corresponding SEAL server is supported by SEAL-UU reference point specified for each SEAL service.

NOTE 4:	The SEAL-C, SEAL-S, SEAL-PC5, SEAL-UU reference points for each SEAL service is specified in 3GPP TS 23.434 [x].

To support distributed FAE server deployments, the FAE server interacts with another FAE server over FAE-E reference point.

FF UE1 can also act as a UE-to-network relay,

-	to enable FAE client on FF UE2 to access FAE server over FAE-1 reference point; and

-	to enable FF application specific client on FF UE2 to access FF application specific server over FFA-1 reference point.The FAE server interacts with the 5GS over N5 and N33 reference points.

* * * Second Change * * * *

# 2	References

[x]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows;"

* * * End Change * * * *