3GPP TSG-SA WG6 Meeting #47-e	S6-220022

e-meeting, 14th – 22nd Februar 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0302 | rev | – | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of two information flow descriptions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | 2022-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To cancel an “in-progress emergency group state” a client sends a cancel request to the server and the server forwards this request to the relevant client(s). The cancel response is sent from the client(s) to the server and from the server to the initiating client. This is described in in procedure 10.6.2.6.1.3 “MCPTT in-progress emergency group state cancel”. However, information flow 10.6.2.2.2 “MCPTT in-progress emergency group state cancel request” is missing the description that it can also be sent from the MCPTT server to the MCPTT client(s). The same applies for the response, information flow “10.6.2.2.2a MCPTT in-progress emergency group state cancel response“, as it is missing the description that it can also be sent from the MCPTT client(s) to the MCPTT server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Information flows 10.9.2.2.2 and 10.6.2.2.2a of 3GPP TS 23.379 are corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Possible misinterpretation in downstream groups. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.2.2.2, 10.6.2.2.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 10.6.2.2.2	MCPTT in-progress emergency group state cancel request

Table 10.6.2.2.2-1 describes the information flow MCPTT in-progress emergency group state cancel request from the MCPTT client to the MCPTT server, from the MCPTT server to the MCPTT server and from the MCPTT server to the MCPTT client.

NOTE:	In Rel-14 and Rel-13 versions of this specification the name of this information flow is "MCPTT emergency group call cancel request".

Table 10.6.2.2.2-1: MCPTT in-progress emergency group state cancel request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The identity of the cancelling party |
| MCPTT group ID | M | The MCPTT group ID on which the MCPTT in-progress emergency state is to be cancelled. |
| Alert indicator | O | Indicates whether the emergency alert of the cancelling party is to be cancelled |

* * * Next Change * * * *

##### 10.6.2.2.2a	MCPTT in-progress emergency group state cancel response

Table 10.6.2.2.2a-1 describes the information flow MCPTT in-progress emergency group state cancel response from the MCPTT server to the MCPTT client, from the MCPTT server to the MCPTT server and from the MCPTT client to the MCPTT server.

NOTE:	In Rel-14 and Rel-13 versions of this specification the name of this information flow is "MCPTT emergency group call cancel response".

Table 10.6.2.2.2a-1: MCPTT in-progress emergency group state cancel response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The identity of the cancelling party |
| MCPTT group ID | M | The MCPTT group ID on which the MCPTT in-progress emergency in-progress is to be cancelled. |
| Result | M | Result of the MCPTT in-progress emergency group state cancel request (success or failure) |

* * * End of Change * * * *