3GPP TSG-SA WG6 Meeting #47-e	S6-220115

e-meeting, 14th – 22nd February 2022	(revision of S6-22xxxx)

Source:	CATT

Title:	Pseudo-CR on solution#1 update

Spec:	3GPP TR 23.700-96 v0.4.0

Agenda item:	9.8

Document for:	Approval

Contact:	<Chunshan Xiong, xiongchunshan@cictmobile.com>

1. Introduction

This contribution updates architecture for fused location service and provides evaluation for solution#1.

2. Reason for Change

Based on the discussio paper S6-220114, the Functional architecture of fused location service is proposed to be changed.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-96 v0.4.0.

* * * First Change * * * *

7.1	Solution #1: Standalone functional architecture for fused location service

7.1.1	Solution description

This solution addresses key issue #1: Architecture enhancement of application enablement for location.

7.1.1.1	Functional architecture

The figure 7.1.1.1-1 identifies the architecture of fused location service enabled by 5GS.

Figure 7.1.1.1-1:	Functional architecture of fused location service

The architecture is composed of logical function modules that are not necessarily physical entities and can reside in or co-locate with existing application layer entities as appropriate.

In the architecture the fused location server and client can be within the MNO domain or third party service provider domain.

The architecture supports multiple possible sources of location information including:

-	AMF location service exposed by NEF (as defined in 3GPP TS 23.502 [12]);

-	LCS location retrieved from either NEF or GMLC (as defined in 3GPP TS 23.273 [4]);

-	SUPL location retrieved from SLP (as defined in OMA AD SUPL [10]);

NOTE:	Whether and how the SUPL location service can be exposed by 5GC is within the remit of SA2.

-	Certain RAT-independent positioning retrieved from 3rd party location server or Fused Location Client.

7.1.1.2	Functional components and reference points

The Fused Location Server provides location information of the target UE based on positioning or location data retrived from one or multiple location source. It provides normalized description of location data to the application specific server (e.g. of ecosystem partners) through the northbound API. As required by the verticals, the Fused Location Server also provides common service capabilities to support location-based applications, i.e. common services to be reused frequently to support e.g. geofencing, location analytics, heatmap etc. The Fused Location Server does not process service requests of a external user (by a non-target UE) directly, i.e. does not process the external user’s request of location-based service,

The 3rd Party Location Server provides the location of a certain location technology (typically the network-based positioning) e.g. Bluetooth.

The Fused Location Client represents the client of target UE providing the UE-based poisitioning and location related information.

The NEF (as defined in 3GPP TS 23.501 [5]) exposes location service of 5GC when Fused Location Server is external to MNO domain.

The GMLC (as defined in 3GPP TS 23.273 [4]) provides LCS when Fused Location Server is within the MNO domain.

The SLP (as defined in OMA AD SUPL [10]) provides location of SUPL network.

The interfaces are described as followed:

FLS-1:	Service-based interface supporting location reporting, location determination, location management and exchange of location contextual information over application layer transactions between the Fused Location Server and the Fused Location Client of the target UE. The FLS-1 may support HTTP or WebSocket.

FLS-2:	Service-based interface exposing fused location data and common service capabilities exploiting the location data to the applications (including e.g. the vertical applications, the applications of ecosystem partners and etc.). The FLS-2 may support HTTP or WebSocket.

FLS-X:	The reference point is used for location retrieval of the target UE from that 3rd party location server. The FLS-X can be a service-based interface. The FLS-X may support HTTP or WebSocket.

NOTE:	The definition of FLS-X is out of scope of this specification.

Nnef:	Service-based interface as defined in 3GPP TS 23.501 [5].

Le:	Reference point as defined in OMA AD MLS [11].

7.1.2	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.