3GPP TSG-SA WG6 Meeting #47-e	S6-220208

e-meeting, 14th – 22nd February 2022	(was S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0031 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MC services over 5GS supporting EPS interworking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 5G system supports EPS interworking as specified in 3GPP TS 23.501. For the case of a 5GS/EPS capable MC service UE, MC services being provided over 5GS can be handed over to EPS (and vice versa), where the network will handle the associated PDU sessions/PDN connections, respectively. 5GS supporting EPS interworking is introduced for the cases of MC services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Aspects related to the support of MC services over 5GS with EPS interworking are introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It remains unclear how MC services are provided over a 5GS supporting EPS interworking |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 7.x, (new) 7.x.1, (new) 7.x.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * *

## 7.x	MC services over 5GS supporting EPS interworking

### 7.x.1	General

MC services over a 5G system supporting EPS interworking are provided to 5GS/EPS capable MC service UEs as defined in 3GPP TS 23.501 [7], 3GPP TS 23.502 [10], and 3GPP TS 23.503 [9].

This clause describes procedures related to the support of MC services over a 5G system with EPS interworking.

### 7.x.2	Inter-system mobility

As specified in 3GPP TS 23.501 [7], a dedicated user plane anchor point, i.e. UPF + PGW-U function, is defined for interworking between 5GS and EPS. This enables that the network can directly handle PDU sessions (in 5GS) and PDN connections (in EPS) associated to MC service sessions of an MC service UE during inter-system change.

The inter-system change of an MC service UE will be transparent to the MC service server. The MC service server will continue interacting with the same control plane functions, e.g. PCF, and user plane function, e.g. UPF + PGW-U.

NOTE 1:	For the case that seamless session continuity is required for MC services, e.g. for MCPTT services, EPS interworking with N26 (interface between AMF in 5GC and MME in EPC) is required for inter-system change, as described in 3GPP TS 23.501 [7].

The procedure in figure 7.x.2-1 describes in a high level inter-system mobility of 5GS/EPS capable MC service UEs with associated MC service sessions.

Pre-conditions:

-	MC service UE is a 5GS/EPS capable UE.

-	MC service UE is registered to a 5GS supporting EPS interworking for the support of MC services.

Figure 7.x.2-1: MC services over 5GS with EPS interworking – Inter-system mobility

1.	MC service UE is registered to the MC service server over the 5GS. MC service UE has established a PDU session(s) associated to MC service sessions.

2.	The 5GS supporting EPS interworking decides that the MC service UE should be handed over to EPS, e.g. due to radio conditions. 5GS handles associated PDU sessions of the MC service UE to establish corresponding PDN connections in EPS, i.e. required default and dedicated EPS bearers associated to MC service sessions.

NOTE 2:	The interworking procedures with EPS are described in 3GPP TS 23.501 [7], 3GPP TS 23.502 [10], and 3GPP TS 23.503 [9].

3.	MC service UE is registered on EPS. MC services continue being provided to the MC service UE over EPS via the established PDN connections (default and dedicated bearers associated to corresponding MC service sessions) in step 2.