3GPP TSG-SA WG6 Meeting #47-e	S6-220225

e-meeting, 14th – 22nd February 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 0024 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for operations of C2 communication mode switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per stage 3 progress, the APIs and Procedures for C2 Communicaiton mode switching should be corrected to include cancel operation. Currently, only initiate operation is specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the cancel operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Having no cancel operation can lead to denial of service from the UAS due to occupancy of resources and unable to revoke the policies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2.x (new), 7.4.3.a (new), 7.4.3.b (new), 7.4.3.c (new), 7.4.3.d (new), 8.2.1, 8.2.2.a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.4.2.x	Cancellation of C2 mode selection / switching capability

This procedure enables the UASS to cancel the C2 mode selection / switching which was previously initiated as specified in clause 7.4.2.1.

Figure 7.4.2.x-1 illustrates the procedure where the UAE server receives an application request for cancelling the operation mode for C2 communications for a UAS.

Pre-condition:

-	The initiation procedure as specified in clause 7.4.2.1 has been executed.

Figure 7.4.2.x-1: C2 operation mode cancellation request / response

1.	The UAS application specific server sends to the UAE Server a C2 operation mode management cancel request for cancelling the operation modes for the C2 communication for a UAS.

2.	The UAE server checks whether the UASS had previously initiated a C2 operation mode management request with the UAS. If previously initiated, the UAE server further sends a C2 communications mode configuration cancel request to the UAE client for the UAS indicated in step 1.

3.	The UAE client removes the previously stored configurations.

4.	The UAE client sends a C2 communications mode configuration cancel response to the UAE server.

5.	The UAE Server sends a C2 operation mode management cancel response to the UAS application specific server.

* * * Next Change * * * *

#### 7.4.3.a	C2 operation mode management cancel request

Table 7.4.3.a-1 describes the information flow C2 operation mode management cancel request from the UAS application specific server to the UAE server.

Table 7.4.3.a-1: C2 operation mode management cancel request

| Information element | Status | Description |
| --- | --- | --- |
| UASS ID | M | Identity of the UAS application specific server which requests the cancellation of the C2 operation mode management. This ID can be the USS/UTM identifier, when the UAS application specific server is the USS/UTM. |
| UAS ID | M | The identification of the UAS for which the C2 QoS management request applies. This could be in form of identifier for the UAS, e.g group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI |

#### 7.4.3.b	C2 operation mode management cancel response

Table 7.4.3.b-1 describes the information flow C2 operation mode management cancel response from the UAE server to the UAS application specific server.

Table 7.4.3.b-1: C2 operation mode management cancel response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative result of the C2 operation mode management cancellation request. |

#### 7.4.3.c	C2 communication modes configuration cancel request

Table 7.4.3.c-1 describes the information flow C2 communication modes configuration cancel request from the UAE server to the UAE client.

Table 7.4.3.c-1: C2 communication modes configuration cancel request

| Information element | Status | Description |
| --- | --- | --- |
| UAS ID | M | The identification of the UAS for which the cancel request applies. This could be in form of identifier for the UAS, e.g. group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI. |

#### 7.4.3.d	C2 communication modes configuration cancel response

Table 7.4.3.d-1 describes the information flow C2 communication modes configuration cancel response from the UAE client to the UAE server.

Table 7.4.3.d-1: C2 communication modes configuration cancel response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative result of the operation. |

* * * Next Change * * * *

### 8.2.1	General

Table 8.2.1-1 illustrates the UAE server APIs.

Table 8.2.1-1: List of UAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| UAE_C2OperationModeManagement API | Initiate_C2OperationModeManagement | UAS application specific server | Request/ Response |
|  | Cancel_C2OperationModeManagement | UAS application specific server | Request/ Response |
|  | Notify_SelectedC2Mode (NOTE) | UAS application specific server | Subscribe/notify |
|  | Notify_C2ModeSwitching (NOTE) | UAS application specific server | Subscribe/notify |
|  | Notify_C2OperationModeManagementComplete (NOTE) | UAS application specific server | Subscribe/notify |
| UAE_RealtimeUAVStatus API | Subscribe_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
|  | Unsubscribe_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
|  | Notify_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
| NOTE: The subscribe operation for Notify_C2OperationModeManagementComplete, Notify_SelectedC2Mode and Notify_C2ModeSwitching is part of Configure_C2OperationMode |  |  |  |

* * * Next Change * * * *

#### 8.2.2.a	Cancel_C2OperationModeManagement operation

API operation name: Cancel_C2OperationModeManagement

Description: Cancels the configuration of the C2 operation modes for the UAS UEs.

Known Consumers: UAS application specific server.

Inputs: Refer clause 7.4.3.a.

Outputs: Refer clause 7.4.3.b.

See clause 7.4.2.x for the details of usage of this API operation.