3GPP TSG-WG SA6 Meeting #47-e	S6-220238

# e-meeting, February 14 – 22, 2022	(revision of S6-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0045 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Off network private communication for MC service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5GProSe |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TR 23.783, MC service support over Off network private communication was discussed and concluded. Unicast mode 5G ProSe Direct Communication was defined by SA2 in TS 23.304. So this feature is considered as the basic feature for MC service. This CR proposes to add the Off network private communication for MC service, based on TR conclusion on this topic. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the support of Off network private communication for MC service, and mainly describing the difference from the 4G feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No related content about Off network private communication for MC service in current TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.X (new), 7.X.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 7.X	MC service over 5G ProSe

### 7.X.Y	Off network private communication

The procedures and information flows used in TS 23.280, TS 23.379, TS 23.281, and TS 23.282 are reused for Off network private communication for MC service over 5G ProSe with following differences:

-	PPPP(ProSe Per-Packet Priority) is replaced by PQI, as specified in TS 23.304 [X].

-	The MC service group ID and the user info ID of the target MC service user are provisioned to the MC service client in the MC service user profile. The MC service client enables the ProSe layer to determine the IP address for the communication with the target MC service user by providing the MC service group ID and user info ID of the target MC service user.

-	Group Member Discovery procedure in 3GPP TS 23.304 [X] is utilized to enable for the call initiator to determine whether the called MC service user is in the proximity of the calling user prior to the establishment of an off-network private communication.

-	The Unicast mode 5G ProSe Direct Communication procedure in 3GPP TS 23.304 [X] is utilized for the establishment of off-network private communication and the following media plane traffic exchanges.

# * * * * End of changes * * * *