3GPP TSG-SA WG6 Meeting #47-e	S6-220239

# e-meeting, 14th – 22nd February 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0046 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G ProSe UE-to-network relay for MC service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5GProSe |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TR 23.783, MC service support over 5G ProSe UE-to-network relay was discussed and concluded. 5G ProSe UE-to-Network Relay Communication was defined by SA2 in TS 23.304, including both 5G ProSe Layer-2 UE-to-Network Relay and 5G ProSe Layer-3 UE-to-Network Relay. So this feature is considered as the basic feature for MC service. This CR proposes to add the 5G ProSe UE-to-network relay for MC service, based on TR conclusion on this topic. In TS 23.304, service continuity over 5G ProSe Layer-3 UE-to-network relay is not supported in network layer, and service continuity over 5G ProSe Layer-2 UE-to-network relay is only partially supported in network layer. So in this release, the service continuity for MC service defined in Annex B of TS 23.280 can be reused. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the support of UE-to-network relay for MC service, and mainly describing the difference from the 4G feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No related content about 5G ProSe UE-to-network relay for MC service in current TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.X (new), 7.X.Y (new), 7.X.Y.1 (new), 7.X.Y.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 7.X	MC service over 5G ProSe

### 7.X.Y	Use of 5G ProSe UE-to-network relay

#### 7.X.Y.1	5G ProSe UE-to-network relay service authorization

The MC service shall support the capability for 5G ProSe UE-to-network relay to restrict the relayed group communication on a per group basis.

5G ProSe (as specified in TS 23.304 [X]) supports layer 2 UE-to-network relay and layer 3 UE-to-network relay. The procedure defined in clause 10.5 of TS 23.280 [3] applies with the following differences:

- 5G ProSe UE-to-network relay is provisioned that each relay service code is offering layer 2 or layer 3 UE-to-Network Relay service.

- 5G ProSe Remote UE is provisioned that each relay service code is offering layer 2 or layer 3 UE-to-Network Relay service.

#### 7.X.Y.2	UE-to-network relay MC service

The 5G ProSe UE-to-network relay provides a layer 2 or layer 3 routing service for a 5G ProSe remote UE, when the MC service user on the 5G ProSe remote UE requires to access the MC service via a 5G ProSe UE-to-network relay.

The application layer signalling for the MC service user on a 5G ProSe remote UE are identical to the application layer signalling for the MC service user on an on-network UE.

In this release, the service continuity for MC service over 5G ProSe UE-to-network relay defined in Annex B of TS 23.280 can be reused.

# * * * * End of changes * * * *