3GPP TSG-SA WG6 Meeting #47-e	S6-220389

e-meeting, 14th – 22nd February 2022	(revision of S6-220225)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 0024 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for operations of C2 communication mode switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per stage 3 progress, the APIs and Procedures for C2 Communicaiton mode switching should be corrected to include cancel operation. Currently, only initiate operation is specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the procedures, information flows and APIs to handle removal of C2 operation mode management information/configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Having no removal operation can lead to denial of service from the UAS due to occupancy of resources or unable to revoke the previously configured policies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2.1, 7.4.2.2, 7.4.3.1, 7.4.3.8, 7.4.3.9, 8.2.1, 8.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.4.2.1	Management of C2 mode selection / switching capability

This procedure manages the C2 mode selection/switching capability at the UAE server, based on an application request from UAS application specific server (which can be the USS/UTM) to manage the C2 operation modes (direct, network-assisted) of C2 communication for a UAS.

Figure 7.4.2.1-1 illustrates the procedure where the UAE server receives an application request for managing the operation mode for C2 communications for a UAS.

Pre-condition:

-	The UAV has received its UAS ID from the UAS application specific server.

Figure 7.4.2.1-1: C2 operation mode management request / response

1.	The UAS application specific server sends to the UAE Server a C2 operation mode management request for managing the operation modes for the C2 communication for a UAS (consisting a UAV and a UAV-C) and to subscribe for UAE notifications.

2.	The UAE Server sends to the UAS application specific server a C2 operation mode management response with a positive or negative acknowledgement of the request, based on capability of UAE server to undertake this task.

3.	UAE server executes C2 communication modes configuration according to clause 7.4.2.2.

4.	After successful execution of C2 communication modes configuration, the UAE server notifies the UAS application specific server with a C2 operation mode management complete.

#### 7.4.2.2	C2 communication modes configuration

This procedure enables the configuration of the UAE Client, based on an application request from UAS application specific server (which can be the USS/UTM) to manage the C2 operation modes (direct, network-assisted) of C2 communication for a UAS.

Figure 7.4.2.2-1 illustrates the C2 communication modes configuration procedure.

Pre-conditions:

1.	The UAS UEs are connected to 5GS and authenticated and authorized by UAS application specific server as specified in clause 5.2 of 3GPP TS 23.256 [4].

2.	UAE Server has established a UAE session with the respective UAE Clients as the UAE clients are successfully registered to the UAE server.

3.	UAE Server has performed the C2 mode switching/selection capability initiation as in clause 7.4.2.1.

Figure 7.4.2.2-1: C2 communication modes configuration

1.	The UAE Server sends a C2 communication modes configuration request including the UAS identifier, allowed C2 communication modes (e.g., direct, network assisted, USS/UTM navigated), primary and optionally secondary C2 communication mode and policy for switching. In the case of removal of C2 communication mode configuration parameters from the UAV or UAV-C, then the request shall only include the UAS identifier.

2.	The UAE Client stores or removes the C2 communication mode configuration parameters as per the information received in step 1.

3.	The UAE Client sends a C2 communication modes configuration response to the UAE Server.

* * * Next Change * * * *

#### 7.4.3.1	C2 operation mode management request

Table 7.4.3.1-1 describes the information flow C2 operation mode management request from the UAS application specific server to the UAE server.

Table 7.4.3.1-1: C2 operation mode management request

| Information element | Status | Description |
| --- | --- | --- |
| UASS ID | M | Identity of the UAS application specific server which requests the C2 operation mode management. This ID can be the USS/UTM identifier, when the UAS application specific server is the USS/UTM. |
| UAS ID | M | The identification of the UAS for which the C2 QoS management request applies. This could be in form of identifier for the UAS, e.g group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI |
| C2 operation mode management container (see NOTE 2) | O | The C2 operation mode management container consists of the requirements and policy for C2 operation mode management |
| > C2 operation mode management requirement | M | Identification of the type of the C2 mode switching to be supported by the UAE server. This can be either from direct to network-assisted C2, or from network-assisted to direct C2 or to UTM navigated. |
| > Allowed C2 communication modes | M | direct, network assisted, USS/UTM navigated |
| > Primary C2 communication mode | M | Primary C2 communication mode (direct, network assisted) |
| > Secondary C2 communication mode | O | Secondary C2 communication mode (direct, network assisted) |
| > Policy of C2 switching | M | Parameters for C2 switching (NOTE 1) - QoS thresholds on active link - QoS thresholds on target link |
| > C2 service area | O | The area where the C2 operation mode management request applies. This can be geographical area, or topological area in which the capability is active. |
| NOTE 1: A complete list of parameters for the QoS-policy of C2 switching is specified by stage 3. NOTE 2: If C2 operation mode management container IE is not included, it indicates removal of the C2 operation mode management related information. |  |  |

* * * Next Change * * * *

#### 7.4.3.8	C2 communication modes configuration request

Table 7.4.3.8-1 describes the information flow C2 communication modes configuration request from the UAE server to the UAE client.

Table 7.4.3.8-1: C2 communication modes configuration request

| Information element | Status | Description |
| --- | --- | --- |
| UAS ID | M | The identification of the UAS for which the C2 QoS management request applies. This could be in form of identifier for the UAS, e.g. group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI. |
| C2 operation mode management configuration (see NOTE 2) | O | The C2 operation mode management configuration information to be configured at the UAS |
| > C2 operation mode management requirement | M | Identification of the type of the C2 mode switching to be supported by the UAE server. This can be either from direct to network-assisted C2, or from network-assisted to direct C2 or to UTM navigated. |
| > Allowed C2 communication modes | M | direct, network assisted, USS/UTM navigated |
| > Primary C2 communication mode | M | Primary C2 communication mode (direct, network assisted) |
| > Secondary C2 communication mode | O | Secondary C2 communication mode (direct, network assisted) |
| > Policy of C2 switching | M | Parameters for C2 switching (NOTE 1) - QoS thresholds on active link - QoS thresholds on target link |
| NOTE 1: A complete list of parameters for the QoS-policy of C2 switching is specified by stage 3. NOTE 2: If C2 operation mode management configuration IE is not included, it indicates removal of the C2 operation mode management configuration at the UAS ID. |  |  |

#### 7.4.3.9	C2 communication modes configuration response

Table 7.4.3.9-1 describes the information flow C2 communication modes configuration response from the UAE client to the UAE server.

Table 7.4.3.9-1: C2 communication modes configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative result of reception and storing or removal of the communication mode configuration parameters |

* * * Next Change * * * *

### 8.2.1	General

Table 8.2.1-1 illustrates the UAE server APIs.

Table 8.2.1-1: List of UAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| UAE_C2OperationModeManagement API | Manage_C2OperationMode | UAS application specific server | Request/ Response |
|  | Notify_SelectedC2Mode (NOTE) | UAS application specific server | Subscribe/notify |
|  | Notify_C2ModeSwitching (NOTE) | UAS application specific server | Subscribe/notify |
|  | Notify_C2OperationModeManagementComplete (NOTE) | UAS application specific server | Subscribe/notify |
| UAE_RealtimeUAVStatus API | Subscribe_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
|  | Unsubscribe_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
|  | Notify_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
| NOTE: The subscribe operation for Notify_C2OperationModeManagementComplete, Notify_SelectedC2Mode and Notify_C2ModeSwitching is part of Configure_C2OperationMode |  |  |  |

* * * Next Change * * * *

#### 8.2.2.2	Manage_C2OperationMode operation

API operation name: Manage_C2OperationMode

Description: Manage (initiate, change or delete) the configuration of the C2 operation modes for the UAS UEs.

Known Consumers: UAS application specific server.

Inputs: Refer clause 7.4.3.1.

Outputs: Refer clause 7.4.3.2.

See clause 7.4.2.1 for the details of usage of this API operation.