3GPP TSG-SA WG6 Meeting #47-e	S6-220394

e-meeting, 14th – 22nd February 2022	(revision of S6-220231)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS23.289 | CR | 0038 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ### Description of 5G MBS usage for MCData |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is no descirption of how to use 5G MBS for MCData in procedure part in current TS 23.289. The related description should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding description of the usage of 5G MBS for MCDtata |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 7.3.3.X (new), 7.3.3.X.1 (new), 7.3.3.X.2 (new), 7.3.3.X.3 (new) |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*********************************Start of the change************************

#### 7.3.3.X	Usage of 5G MBS for MCData

##### 7.3.3.X.1	General

The procedures and information flows for Short data service and file distribution of MC Data group are defined in 3GPP TS 23.282 [5].

When using 5G MBS, MCData server can determine to use either pre-created MBS session or dynamic MBS session as defined in clause 7.3.3.1 for the transmission of DL link media for different types of MCData service capabilities.

MCData may use the 5G MBS session for the MCData features short data service and file distribution for MC Data group. The 5G MBS session can be used by any group.

Both the media packets as well as application level control signalling (e.g. transmission control) to the receiving users may be sent on the MBS session. Optionally, a separate MBS Session could be used for the application level control signalling (e.g. transmission control).

When MBSF/MBSTF are deployed in 5GS, MCData server could also determine to use File distribution capabilities provided by MBSF/MBSTF for file distribution.

Editor's Note: The related procedure and the reference to the SA4 specification is FFS.

##### 7.3.3.X.2	Usage of MBS session for MCData

The MCData service shall support the procedure for using pre-created MBS session or dynamic MBS session as specified in Clause 7.3.3.1 with the following clarifications:

-	The MC service client is the MCData client;

-	The MC service server is the MCData server; and

-	The MC service ID is the MCData ID.

### 7.3.3.X.3 	Switching from MBS session to unicast bearer or eMBMS bearer for MCData

The MCData service shall support the procedure for switching from MBS session to unicast bearer or eMBMS bearer as specified in clause 7.3.3.8 and clause 7.3.3.9 with the following clarifications:

-	The MC service client is the MCData client;

-	The MC service server is the MCData server; and

-	The MC service ID is the MCData ID.

The MCData service shall use the MCData-SDS-1, MCData-SDS-2, MCData-FD-1, MCData-FD-3, MCData-DS-1 and MCData-DS-3 reference points for this procedure.

*********************************The second change*********************************************