3GPP TSG-SA WG6 Meeting #47-e meeting	S6-220454

14th Feb – 22nd Feb 2022, Online	(Revision of S6-220301)

Source:	Intel

Title:	Mapping Considerations of EAS IE and MEC App Instance IE

Spec:	3GPP TR 23.700-98

Agenda item:	9.9

Document for:	Approval

Contact:	samar.shailendra@intel.com

# 1	Introduction

As specified in Annex C of TS 23.558 (Rel-17), the two architectures (EDGEAPP and ETSI MEC) can be present in a single system. This pCR provides considerations for mapping EAS IE to MEC Application Instance IE (AppInfo).

# 2	Proposal

It is proposed to modify the text of TR 23.700-98 as follows.

1st CHANGE

## A.2	Service consumer and service provider

The functionalities enabled via the Mp1 reference point between MEC applications and MEC platform is mainly described in ETSI GS MEC011 [14]. The related functionality includes MEC service registration/deregistration, MEC service discovery and event notifications. Other functionality includes MEC service availability, traffic rules, DNS and time of day.

From ETSI MEC's perspective, there are two types of MEC Applications, i.e., MEC Application that consumes MEC Services and MEC Application that provides MEC service(s). For the MEC Application that provides MEC service(s), the MEC Application sends a service registration request to the MEC platform to register the MEC service during the MEC Application start-up procedure. As for the MEC Application that consumes MEC Services, the MEC Application can send a service query request to the MEC platform to discover a MEC service. It should be noted that the API of registration and discovery is defined for MEC service.

In R17 of SA6, the EES can take the role of the CAPIF core function, and the vertical application enabler server acting the AEF and publish the vertical application enabler server APIs to the EES. Further, the vertical application enabler server APIs is discovered by the EASs acting as the API invoker during the service API discover procedure as specified in 3GPP TS 23.222 [15].

In clause 4.2, the Key issue #2 plans to study Enablement of Service APIs exposed by EAS. The R17 of EDGEAPP only defines the functionality of EAS acting as an invoker, which is similar to MEC Application that consumes MEC Services defined in ETSI MEC:

[Observation A.2-1] The R17 of EDGEAPP only defines the functionality of EAS acting as an invoker, which is similar to MEC Application that consumes MEC Services defined in ETSI MEC. According to the Key issue #2 in clause 4.2, The EAS acting as a service provider is expected to be defined in R18 and expose service APIs.

## A.3	EAS/MEC application profile provisioning

ETSI MEC and EDGEAPP defined different styles of EAS/MEC application profile provisioning. The information flows for lifecycle management of MEC applications is described in ETSI GS MEC010-2 [13]. The informational flows for the optional MEC Application registration are described in ETSI GS MEC 011 [14]. The MEC application can start producing or consuming MEC Services after the MEC Application is instantiated and running. The application information (AppInfo), which can be regarded as the MEC application profile, represents the information provided by the MEC application instance as part of the “application registration request” message. The attributes of the AppInfo are available from the clause 7.1.2.6 of ETSI GS MEC011 [14].

Some fields in AppInfo are intentionally not duplicating the EAS profile (if present) with conflicting parameters but should be consistent with them. This is highlighted in NOTE 1 and NOTE 2, for example. It can be seen that unlike AppD [13], which is mainly used in the management plane for instantiating an application, and is static in nature, AppInfo carries the runtime information about the MEC application instance.

In EDGEAPP, the EAS profile is provided in the EAS registration request. According to clause 8.2.4 of 3GPP TS 23.558 [2] the information element of the EAS profile is listed as below:

Table A.3-2: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| ACID(s) | O | Identifies the AC(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the ASP that provides the EAS. |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Geographical Service Area | O | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Topological Service Area | O | The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS |
| EAS Service continuity support | O | Indicates if the EAS supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EAS. |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [14]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |
| List of N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Required Service APIs | O | A list of the Service APIs that are required by the EAS |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |

Comparison of AppInfo in 7.1.2.6 of ETSI GS MEC 011 [14]  and EAS Profile in Table A.3.2 shows that:

[Observation A.3-1] Both ETSI MEC and EDGEAPP provides similar types of EAS/MEC application profile provisioning. Both the EAS Profile and MEC Application Instance (AppInfo) is provided during the application registration request.

[Observation A.3-2] Some IEs of the EAS profile may overlap with the ones defined in AppInfo (e.g., EAS ID vs. appName, EAS Provider Identifier vs. appProvider, EAS Endpoint vs. emdpoint). Both AppInfo and EAS Profile has many optional IEs, whether and how to address their differences in SA6 is FFS.

End of 1st CHANGE

2nd CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.558: "Architecture for enabling Edge Applications".

[3]	ETSI ISG MEC ETSI GS MEC 003, "Multi-access Edge Computing (MEC); Framework and Reference Architecture".

[4]	GSMA OPG.02: "Operator Platform Telco Edge Requirements", https://www.gsma.com/futurenetworks/wp-content/uploads/2021/07/GSMA-OPG-Telco-Edge-Requirements-2021.pdf

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TR 23.758: "Study on application architecture for enabling Edge Applications".

[7]	3GPP TR 23.721: "Study on Sponsored Data Connectivity Improvements".

[8]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[9]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[10]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[11]	3GPP TS 23.503: "Policies and Charging control architecture; Stage 2".

[12]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[13]	ETSI GS MEC 010-2: "Multi-access Edge Computing (MEC); MEC Management; Part 2: Application lifecycle, rules and requirements management".

[14]	ETSI GS MEC 011 v3.0.5: " Multi-access Edge Computing (MEC); Edge Platform Application Enablement".

[15]	ETSI GS MEC 001: " Multi-access Edge Computing (MEC); Terminology ".

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

End of 2nd CHANGE

END OF CHANGES