3GPP TSG-SA WG6 Meeting #47-e	S6-220296

e-meeting, 14th – 22nd February 2022	(revision of S6-220164)

Source:	Ericsson

Title:	Initial EAS selection declaration

Spec:	3GPP TS 23.700-98 v0.4.0

Agenda item:	9.9

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution proposes a new solution to support traffic influence in initial EAS selection.

2. Reason for Change

This pCR proposes a new solution allowing the EEC to indicate the selected EAS and desire to influence EAS traffic to the EES, during the initial EAS discovery and selection.

Rev:

In previous meeting, it was commented that

“Selected EAS declaration is waste of the signaling and it will increase the delay for establishing the connection with the EAS which is not aligned with the principle of the traffic influence to reduce the delay.”

Actually, the existing Traffic influence in R17 has the possibility per ACR process (e.g. EEC executed ACR via S-EES scenario), it is also possible to have such option for the initial EAS discovery. After all, the decision to execute traffic influence is still in EES (if influence was not done it can be done upon receipt of the message from EEC).

Hence, the EES handling description is improved with such consideration.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes in TS 23.700-98.

* * * First Change * * * *

## 7.x	Solution #XX: Initial EAS selection declaration

### 7.x.1	Architecture enhancements

None.

### 7.x.2	Solution description

This solution addresses KI#8 and KI#14. In this solution, the EES can know the selected EAS and the EES is enabled to trigger the EAS traffic influence after initial EAS is being determined.

Figure 7.x.2-1: Initial EAS selection declaration

In Figure 7.x.2-1 step 1 and 2, the EEC performs the start-up procedures for initial service provisioning and EAS discovery. EEC may send EAS discovery to multiple EESs. If registration is required by an EES, EEC registers into the EES before EAS discovery.

In step 3, the EEC (or AC and EEC) selects the initial EAS from the discovered EAS candidates.

In step 4, the EEC sends Selected EAS declaration request with AC ID, EAS ID, EAS endpoint and UE ID to the selected EES (which is determined based on the selected EAS).

The EES, in step 5:

- may apply the EAS traffic influence with the N6 routing information of the EAS in the 3GPP Core Network, if applicable and if the EAS traffic influence was not done before.

NOTE 1:	EES can also influence the EAS traffic in advance.

The EEC is then responded by the selected EES with success/failure of the request in step 6.

Editor’s Note: It is FFS whether EES can perform more actions (e.g. store SSC) and the timing for AC to connect to selected EAS.

NOTE 3:	The AC is not depicted in above figure for simplicity and solution to address interaction between AC and EEC is related to KI#4 for step 1 to 3.

Table 7.x.2.1-1 describes information elements for the selected EAS announcement request sent from the EEC to the serving EES.

Table 7.x.2.1-1: Selected EAS announcement request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | M | The identifier of the UE. |
| AC ID | O | The Application Client ID. |
| Security credentials | M | Security credentials. |
| Selected EAS ID | M | Selected EAS identifier. |
| Selected EAS Endpoint | M | Endpoint of the selected EAS. |

### 7.x.3	Solution evaluation

This solution addresses KI#14. It allows the EES to be aware of the selected EAS in initial service start so that the EES can control application traffic influence for the initial application traffic. It is a viable solution.

Editor’s Note: It is FFS whether and how management of SSC required by this solution can be provided if the EEC does not register to EES before EAS discovery.

Editor's note:	Whether a new selected EAS declaration announcement API is needed or existing API (e.g. EEC registration) can be re-used is FFS.

* * * End of Changes * * * *