3GPP TSG-SA WG6 Meeting #47-e	S6-220412

e-meeting, 14th – 22nd February 2022	(revision of S6-220251)

Source:	Huawei, Hisilicon, InterDigital

Title:	Key issue on ACR scenarios combination

Spec:	3GPP TR 23.700-98 v0.4.0

Agenda item:	9.9

Document for:	Approval

Contact:	Yajie Hu <Huyajie1@huawei.com>

1. Introduction

This pCR provides a Key issue on ACR scenarios combination.

2. Reason for Change

The issue of ACR scenario overlapping is still not resloved in release 17, and related EN is remained in 23.558 specification.

Editor's note:	whether the scenarios are overlapping and how to solve any co-existence issues are FFS.

In current 23.558 specification, multiple entities can detect ACR event and perform ACR procedure, so it may happen AC(s)/EEC has different capabilities with EAS(s)/EES, and select different approaches. From another point of view, if the AC and EAS both support mutiple ACR scenarios, the ACR event will be detected multiple times, furthermore the ACR procdure will be initiated repeatedly for one AC causing large amout of unnecessary signalling waste.

Thus the overlapping of ACR scenario should be studied in release 18.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-98.

* * * First Change * * * *

## 4.X	Key issue #x: ACR scenario combination

Multiple ACR scenarios are specified in 3GPP TS 23.558[x] clause 8.8. Applications can utilize one or more ACR scenarios. Different combinations of utilizing ACR scenarios by Applications should be enabled by the Edge Enabler Layer (e.g. only one ACR scenario allowed or several ACR scenarios allowed).

Open issues:

-	Whether and how the EEL can support the determination of the ACR scenario for one AC?

* * * End of Change * * * *