3GPP TSG-SA WG6 Meeting #47-e	S6-220149

e-meeting, 14th – 22nd February 2022	(revision of S6-22xxxx)

Source:	CATT

Title:	Pseudo-CR on update on solution #4

Spec:	3GPP TR 23.700-96 v0.4.0

Agenda item:	9.8

Document for:	Approval

Contact:	Chunshan Xiong, xiongchunshan@cictmobile.com

1. Introduction

This contribution provides solution update and evaluation for solution #4: Location service registration.

2. Reason for Change

The contribution provide additional service and capability information in the location service registration result, which can be based on authorization and privacy check towards the UE.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-96 v0.4.0.

* * * First Change * * * *

## 7.4	Solution #4: Location service registration

### 7.4.1	Solution description

This solution addresses key issue #5: Initialization and configuration for fused location service.

The procedure for location service registration is illustrated in figure below. This procedure is based on fused location architecture of solution#1. The purpose of this procedure is for the fused location client to register to the location services available at the fused location server while ensuring the privacy of the user.

Figure 7.4.1-1: Location service registration procedure

1.	The fused location client of a target UE sends location service registration request to the fused location server, carrying the identifier of the UE (e.g. GPSI, MSISDN, UUID, etc.).

2.	The fused location server checks authorization for the UE's request. If the fused location server supports privacy checking it also performs or assists with e.g. 5GC on privacy check.

3.	The fused location server, upon successful authorization and privacy check (if any), responds to the fused location client with registration result. The registration result may contain location service or capability information that are authorized for the fused location client.

Editor's Note: Information flow for the registration procedure is FFS.

### 7.4.2	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.