3GPP TSG-SA WG6 Meeting #48-e	S6-220499

e-meeting, 5th – 14th April 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0311 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MC gateway UE – MC client disassociation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC, Nokia, Nokia Shanghai Bell, BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 30.03.2022 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The association for using an MC gateway UE can be requested by an MC client. Since this association between MC client and MC gateway UE does not have to exist permanently, the possibility of disassociation between an MC client and the MC gateway UE should be available. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR proposes an MC client - MC gateway UE disassociation procedure for both, MC clients hosted on a non-3GPP device and for MC clients that cannot be hosted on a non-3GPP device. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The association between MC client and MC gateway UE lasts forever. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.379, TS 23.282, TS 23.282 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************************************1st Change*******************************************

### 11.5.4	Connection disassociation mechanism

#### 11.5.4.1	General

A connection using an MC gateway UE by an MC client can be cancelled over time or re-established using same or another MC gateway UE. The connection disassociation mechanism allows the MC client to disassociate the use of the corresponding MC gateway UE considering the various MC client hosting scenarios.

Under certain circumstances, the association with the corresponding MC gateway UE can change or has to be adjusted. The various reasons are detailed in the informative Annex D. For this purpose, the MC gateway UE can send a notification to the corresponding MC client hosted on a non-3GPP device.

#### 11.5.4.2	Connection disassociation for non-3GPP devices that host an MC client

##### 11.5.4.2.1	General

The solution is applied to non-3GPP devices which can host an MC client. The MC gateway UE forwards the connection disassociation request to the corresponding MC server to unbind the MC gateway UE to MC client association.

##### 11.5.4.2.2	Information flows

###### 11.5.4.2.2.1	Connection disassociation request

Table 11.5.4.2.2.1-1 describes the information flow connection disassociation request sent from the MC client, which resides on a non-3GPP device, to the MC gateway UE, and from the MC gateway UE to the corresponding MC server.

Table 11.5.4.2.2.1-1: Connection disassociation request

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user. |
| NOTE: The GW MC service ID indicates for which MC service the connection is to be disassociated. |  |  |

###### 11.5.4.2.2.2	Connection disassociation response

Table 11.5.4.2.2.2-1 describes the information flow connection disassociation response sent from the MC server to the MC gateway UE, and from the MC gateway UE to the MC client residing on a non-3GPP device.

Table 11.5.1.2.2.2-1: Connection disassociation response

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user. |
| Response | M | Result of the connection disassociation request. |

###### 11.5.4.2.2.3	Connection disassociation notification

Table 11.5.4.2.2.3-1 describes the information flow connection disassociation notification sent from the MC gateway UE to the MC client, which resides on a non-3GPP device.

Table 11.5.4.2.2.3-1: Connection disassociation notification

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the associated MC client. (NOTE 1) |
| Status information | M | This information element provide the reason for disassociation. (NOTE 2). |
| NOTE 1: The GW MC service ID indicates for which MC service the connection is to be disassociated. NOTE 2: Information about disassociation reasons are further detailed in Annex D. |  |  |

##### 11.5.4.2.3	Connection disassociation procedure

The procedure for connection disassociation via an MC gateway UE towards an MC server is shown in figure 11.5.4.2.3-1.

Pre-conditions

-	The MC service user has an authorized connection via an MC gateway UE.

-	The MC service user on a non-3GPP device wishes to unbind from a MC gateway UE.

-	The MC client has been provided with an appropriate GW MC service ID.

Figure 11.5.4.2.3-1: Connection disassociation with an MC server via an MC gateway UE

1.	The MC client requests connection disassociation via the MC gateway UE with an MC server. The client of the MC service user provides the GW MC service ID.

2.	The MC gateway UE checks the service binding of the provided GW MC service ID. If the binding between MC gateway UE and MC server is active, the procedure continues with step 3, otherwise the procedure proceeds with step 7.

3.	The MC gateway UE sends the connection disassociation request to the MC server.

4.	The MC server verifies if the association of the provided GW MC service ID is active and can be disassociated.

5.	The MC server sends the connection disassociation response to the MC gateway UE.

6.	The MC gateway UE updates MC client association status.

7.	The MC gateway UE sends the connection disassociation response to the MC client.

##### 11.5.4.2.4	Connection notification

The procedure for connection notification initiated by an MC gateway UE towards an MC client is shown in figure 11.5.4.2.4-1 informs about the status of connection association that may result into an connection disassociation.

Pre-conditions

-	The MC client has an authorized connection via an MC gateway UE.

-	The MC gateway UE is no longer able to provide the requested service depending on reasons further detailed in Annex D.

-	The MC client has been provided with an appropriate GW MC service ID.

-	The MC gateway verifies the service binding of the associated GW MC service IDs.

Figure 11.5.4.2.1-1: Connection notification to an associated MC client

1.	The MC gateway UE needs to terminate the association with an MC server for the corresponding MC client. The MC gateway UE sends connection disassociation notification to the MC client using the corresponding GW MC gateway ID.

2.	The disassociation reason code may determine if the MC client want to disassociate from the MC server. If it can be disassociated, the MC client can optionally start connection disassociation procedure in accordance to 11.5.4.2.3.

#### 11.5.4.3	Connection disassociation for non-3GPP devices that do not host an MC client

##### 11.5.4.3.1	General

The solution is applied to non-3GPP devices which cannot host an MC client. The MC server is requested to unbind the MC gateway UE to MC client association on demand.

##### 11.5.4.3.2	Information flows

###### 11.5.4.3.2.1	Connection disassociation request

Table 11.5.4.3.2.1-1 describes the information flow connection disassociation request sent from the MC client, which resides on a MC gateway UE, to the MC server.

Table 11.5.4.3.2.1-1: Connection disassociation request

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user. |
| NOTE: The GW MC service ID indicates for which MC service the connection is to be disassociated. |  |  |

###### 11.5.4.3.2.2	Connection disassociation response

Table 11.5.4.3.2.2-1 describes the information flow connection disassociation response sent from the MC server to the MC client residing on the MC gateway UE.

Table 11.5.4.3.2.2-1: Connection disassociation response

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user. |
| Result | M | Success or failure of the connection disassociation request (disassociation successful/failed; not permitted). |

##### 11.5.4.3.3	Connection disassociation procedure

The procedure for connection disassociation of an MC client hosted by the MC gateway UE towards an MC server is shown in figure 11.5.4.3.3-1.

Pre-conditions

-	The MC service user has an authorized connection using an MC gateway UE.

-	The MC service user hosted on a MC gateway UE wishes to unbind from MC services.

-	The MC client has been configured with the necessary parameters applicable for connectivity with the MC gateway UE.

Figure 11.5.4.3.3-1: Connection disassociation of an MC client hosted by an MC gateway UE

1.	The MC client, hosted by the MC gateway UE, sends a connection disassociation request to the corresponding an MC server encompassing the GW MC service ID.

2.	The MC server verifies if the association of the provided GW MC service ID is active and can be disassociated.

3.	The MC server sends the connection disassociation response to the MC client residing on the MC gateway UE.

******************************************End of Change******************************************