3GPP TSG-SA WG6 Meeting #48-e meeting	S6-220568

05th – 14th April 2022, Online

Source:	ETRI, Uangel

Title:	Updates to architectural requirements for EAS Service APIs

Spec:	3GPP TR 23.700-98

Agenda item:	9.8

Document for:	Approval

Contact:	Seung-Ik Lee <seungiklee@etri.re.kr>

1	Introduction

This paper proposes to revise the architectural requirements for EAS Service APIs as specified in the clause 5.2.

2	Discussion

- Some editorial changes are applied.

- A new requirement for EAS Service API KPIs is added as per solution #8.

3		Proposal

It is proposed to modify the text of TR 23.700-98 as follows.

1st CHANGE

5.2	Enablement of Service APIs exposed by EAS

5.2.1	General

This clause specifies the requirements for EAS Service API enablement in the EDGEAPP architecture to address the KI#2 in the clause 4.2.

5.2.2	Requirements

The followings are the architectural requirements to support EAS Service APIs in the EDGEAPP Rel-18 architecture.

1)	EAS capability exposure

-	The application layer architecture shall support exposure of EAS's capabilities to the other EASs.

2)	EAS Service API publication

-	The application layer architecture shall support EAS to publish its exposing Service API information to EES

-	The application layer architecture shall support EAS to update the published EAS Service API information on the EES.

3)	EAS Service API discovery

-	The application layer architecture shall provide mechanisms for an EAS to discover available EAS Service APIs.

4)	Subscription service

-	The application layer architecture shall provide subscription and notification mechanisms enabling an EAS to receive changes in dynamic information of EAS Service APIs from an EES.

-	The application layer architecture shall provide subscription and notification mechanisms enabling an EAS to receive changes in availability of EAS Service APIs from an EES.

5)	EAS Service API KPIs

-	The application layer architecture shall provide mechanisms for the EAS to publish and update KPIs of its Service APIs when available.

END OF CHANGES