3GPP TSG-SA WG6 Meeting #48-e	S6-220690

e-meeting, 5th – 14th April 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0054 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections in MBS UE session join notification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2022-03-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The UE will send the MBS UE session joining notification to the MC service server after successfully joining a certain multicast MBS session. Currently in the MBS UE session join notification information flow, the ‘MBS session ID’ description indicates that the MBS session ID is identity of the broadcast MBS session. Therefore, existing description need to be corrected to indicate that the MBS session ID is identity of the multicast MBS session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The ‘MBS session ID’ description is updated to include the MBS session ID is identity of the multicast MBS session instead of broadcast MBS session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Creates the confusion whether MBS UE session join notification is for successfully joining a certain multicast MBS session or broadcast MBS session |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.3.2.2	MBS UE session join notification

Table 7.3.2.2-1 describes the information flow of MBS UE session joining notification from the MC service client to the MC service server after successfully joining a certain multicast MBS session procedure as defined in 3GPP TS 23.247 [15].

Table 7.3.2.2-1: MBS UE session join notification

| Information element | Status | Description |
| --- | --- | --- |
| MBS session ID(s) | M | The identity of the multicast MBS session(s) being monitored. It is the TMGI type. |
| MC service ID | M | Identity of the MC service user who is reporting the session status |
| MBS multicast joining status | M | The multicast listening status can be joined if successfully joined MBS session. |

* * * End of Changes * * * *